3GPP TSG-RAN WG2 Meeting #89bis	 R2-151227

20.04. to 24.04.2015, Bratislava, Slovakia

Agenda Item:	7.8

Source: 	CMCC

Title:  		Discussion on use cases and related measurements of feMDT

Document for:	Discussion and decision

Introduction

A new SI on further enhancement of Minimization of Drive Tests (feMDT) for E-UTRAN [1] was approved in RAN#67 meeting. One target of this study item is to investigate possible measurement quantities and corresponding procedures of MDT functionality in order to support better understanding of the QoS and its limiting factors for MMTEL voice and video traffic. This paper will discuss the use cases of feMDT related to MMTEL voice and video traffic. It is noted that in the discussion of the paper, we take VoLTE as an example of MMTEL voice and video traffic. All the use cases and corresponding measurement quantities discussed in the paper are applied not only for VoLTE, but also for other MMTEL voice and video traffic.

Use cases discussion

Many operators are planning to launch commercial VoLTE business at the end of this year or next year. As the key method of proving high-quality voice service to customers, VoLTE is being and will be tested thoroughly by operators and their vendors. So far the VoLTE test is mainly performed in a drive test manner, which means a highly complex and low cost-efficient operation. MDT is considered as an efficient tool to reduce the test complexity and cost; however, the currently standardized mechanism cannot fulfill the need of VoLTE test. For instance, the current MDT function is not able to provide fine measurement for QoS verification of VoLTE traffic. Therefore it is natural to enhance the existing MDT functionality to bring operator a better understanding of this traffic through the benefits of MDT compared with traditional drive tests. Taking China Mobile as an example, commercial VoLTE will be fully activated at the end of 2015. And at the same point of time, there will be 1 million eNBs equipped with MDT functionality providing services in our network. Thus if MDT can support the QoS verification of VoLTE, a large number of traditional drive tests will be saved, and accordingly the cost and emissions of carbon dioxide will be significantly decreased.

Specifically speaking, there are two aspects of QoS verification of VoLTE. One is to assess the VoLTE performance. The other one is to identify the traffic distribution among geographical areas during different time.

Proposal 1: One use case of feMDT should be QoS verification of VoLTE, including performance assessment and traffic distribution identification.

Performance assessment of VoLTE

Regarding the performance assessment of VoLTE, there are three typical metrics used according to drive tests, which are packet loss rate, packet delay and voice dropping rate. Obviously voice dropping or large delay can easily be perceived by a customer when VoLTE is ongoing. The packet loss also can directly impact the customer experience as shown in figure 1, where the MOS rank goes down sharply due to the increasing of packet loss rate (poor SINR).

Figure 1. Field test results of VoLTE Quality vs. RS-SINR

Therefore for the purpose of performance assessment of VoLTE, voice dropping, packet delay rate and packet loss rate should be taken into consideration as the measurement quantities of feMDT.

Proposal 2: For the purpose of performance assessment of VoLTE, voice dropping, packet delay and packet loss rate should be taken into consideration as metrics.

As said in the SID [1] that priority should be given to solutions based on the existing UE and network measurements and procedures. According to TS 36.314 [2], Packet discard rate in the DL per QCI and packet Uu loss rate in the DL per QCI could be reused together to reflect the VoLTE (QCI = 1) packet loss in downlink. One small modification could be to add a new dimension to these measurements, i.e., per UE per QCI. Because for the purpose of drive tests, not only cell level performance needs to be acquired but also geographical location specific performance (coming from coupling location information reported by UE and measurement results from eNB or UE) need to be acquired. For uplink, Packet loss rate in the UL per UE per QCI could also be introduced based on the existing UL packet loss rate in [2]. However, the discarded packets in UL are not taken into account when assessing uplink packet loss. Therefore we should study the necessity of introducing UL packet discard rate of VoLTE per UE.

The similar situation can be found when considering the delay. In [2], the packet delay in the DL per QCI was specified, which can also be reused and modified in feMDT. But for uplink, no delay related measurement exists. So we should study the necessity of introducing UL packet delay of VoLTE per UE.

Regarding voice dropping, so far there is no straightforward measurement to reflect voice dropping. One possible metric is the RLF, which will lead potential voice dropping. In addition, the RLF report could be reused just with some small modification, e.g. adding some kind of tag indicating RLF happens when VoLTE ongoing.

Proposal 3: For the metric of packet loss rate, the following measurements should be introduced for feMDT:

- Packet discard rate in the DL per QCI per UE based on the existing measurement of packet discard rate in the DL per QCI specified in TS 36.314

- Packet Uu loss rate in the DL per QCI per UE based on the existing measurement of packet Uu loss rate in the DL per QCI specified in TS 36.314

- Packet loss rate in the UL per QCI per UE based on the existing measurement of packet loss rate in the UL per QCI specified in TS 36.314

Proposal 4: For the metric of packet loss rate, the necessity of introducing VoLTE packet discard rate in the UL per UE should be studied.

Proposal 5: For the metric of packet delay, the packet delay in the DL per QCI per UE should be introduced for feMDT based on the existing measurement of the packet delay in the DL per QCI specified in TS 36.314.

Proposal 6: For the metric of packet delay, the necessity of introducing VoLTE packet delay in the UL per UE should be studied.

Proposal 7: For the metric of VoLTE dropping, a new indicator should be introduced in RLF report. The new indicator should indicate whether or not the failure leads to VoLTE dropping.

Identification of the VoLTE traffic distribution

Regarding the use case of identifying the VoLTE traffic distribution among different geographical areas during different time, it will can help operator to figure out where and when the VoLTE traffic hotspot is. Such information can help operator for network planning, network optimization and Radio Resource Management (RRM) parameter optimization aiming at achieving high-quality voice service and efficient resource utilization. According to current specification, eNB can straightly identify the VoLTE traffic through QCI mapping when the radio bearer set up and the corresponding UE location can also be derived through Rel-11 MDT. What seems to be lacking is the procedure of configuration, activation/deactivation and collection between OAM and eNB.

Proposal 8: The VoLTE traffic distribution among different geographical areas during different time can be derived through eNB implementation and Rel-11 MDT functionality.

Conclusion

In this paper, the use case of feMDT for VoLTE traffic is discussed. And the related measurement quantities are also discussed. The following proposals are given:

Proposal 1: One use case of feMDT should be QoS verification of VoLTE, including performance assessment and traffic distribution identification.

Proposal 2: For the purpose of performance assessment of VoLTE, voice dropping, packet delay and packet loss rate should be taken into consideration as metrics.

Proposal 3: For the metric of packet loss rate, the following measurements should be introduced for feMDT:

- Packet discard rate in the DL per QCI per UE based on the existing measurement of packet discard rate in the DL per QCI specified in TS 36.314

- Packet Uu loss rate in the DL per QCI per UE based on the existing measurement of packet Uu loss rate in the DL per QCI specified in TS 36.314

- Packet loss rate in the UL per QCI per UE based on the existing measurement of packet loss rate in the UL per QCI specified in TS 36.314

Proposal 4: For the metric of packet loss rate, the necessity of introducing VoLTE packet discard rate in the UL per UE should be studied.

Proposal 5: For the metric of packet delay, the packet delay in the DL per QCI per UE should be introduced for feMDT based on the existing measurement of the packet delay in the DL per QCI specified in TS 36.314.

Proposal 6: For the metric of packet delay, the necessity of introducing VoLTE packet delay in the UL per UE should be studied.

Proposal 7: For the metric of VoLTE dropping, a new indicator should be introduced in RLF report. The new indicator should indicate whether or not the failure leads to VoLTE dropping.

Proposal 8: The VoLTE traffic distribution among different geographical areas during different time can be derived through eNB implementation and Rel-11 MDT functionality.

References

[1] RP-150472, New Study Item Proposal: Further Enhancements of Minimization of Drive Tests for E-UTRAN; CMCC

[2] 3GPP TS 36.314 (v12.0.0): “Layer 2 - Measurements”