3GPP TSG-RAN WG2 Meeting #89bis 	R2-151316

# Bratislava, Slovakia, April 20 – April 24, 2015

| CR-Form-v11.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.323 | CR | CRNum | rev | - | Current version: | 12.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Data available for transmission in split bearer |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | LG Electronics Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_SC_enh_dualC-Core |  |  |  |  |  |  | Date: |  | 2015-04-20 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-12 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) |  |
| Reason for change: |  | For split bearers, the PDCP data available for transmission shall trigger BSR only in the MAC entity configured by RRC. However, the current spec. is not clear whether the PDCP data available for transmission triggers BSR in both MAC entities or only in the configured MAC entity. “Indicate the data available for transmission to the MAC entity” is not aligned with MAC spec. because the MAC spec. specifies “what data shall be considered as available for transmission”. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Make it clear that the “data available for transmission in the PDCP entity” is considered only in the MAC entity configured by RRC. Impact analysis Impacted functionality: Data available for transmission in PDCP Inter-operability: There is no inter-operability issues |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | For split bearers, both MCG MAC and SCG MAC may trigger BSRs uselessly when a PDCP data becomes available for transmission. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

4.5	Data available for transmission

For the purpose of MAC buffer status reporting, the UE shall consider PDCP Control PDUs, as well as the following as data available for transmission in the PDCP layer:

For SDUs for which no PDU has been submitted to lower layers:

- the SDU itself, if the SDU has not yet been processed by PDCP, or

- the PDU if the SDU has been processed by PDCP.

In addition, for radio bearers that are mapped on RLC AM, if the PDCP entity has previously performed the re-establishment procedure, the UE shall also consider the following as data available for transmission in the PDCP layer:

For SDUs for which a corresponding PDU has only been submitted to lower layers prior to the PDCP re-establishment, starting from the first SDU for which the delivery of the corresponding PDUs has not been confirmed by the lower layer, except the SDUs which are indicated as successfully delivered by the PDCP status report, if received:

- the SDU, if it has not yet been processed by PDCP, or

- the PDU once it has been processed by PDCP.

For split bearers, the UE shall:

-	if ul-DataPath is set to scg by upper layer [3]:

-	consider the data available for transmission  in the MAC entity configured for SCG;

-	else:

-	consider the data available for transmission in the MAC entity configured for MCG.