3GPP TSG-RAN WG2 #89bis	Tdoc R2-151545

Bratislava, Slovakia, 20–24 Apr 2015

Agenda Item:	6.2.9.2

Source:	Ericsson

Title:	UE behaviour receiving reserved values in MAC PDU on MCH

Document for:	Discussion, Decision

# Introduction

In RAN2 #89 the UE behaviour when receiving a MAC PDU containing reserved values was discussed. In this discussion several different UE behaviour were noted as a result of interpretation of the current 36.321 specification text. In this contribution we list the current situation and list a few options for a changed UE behaviour from Rel-12.

# Discussion

In Section 5.11 in 36.321 it is currently specified:

When a MAC entity receives a MAC PDU for the MAC entity’s C-RNTI or Semi-Persistent Scheduling C-RNTI, or by the configured downlink assignment, containing reserved or invalid values, the MAC entity shall: - discard the received PDU. When a MAC entity receives a MAC PDU on MCH containing reserved values, the MAC entity shall: - ignore the fields in the PDU header and the control elements containing reserved values and the corresponding parts indicated by the fields in the received PDU.

From the discussion on the handling by legacy UEs (pre-Release 12 UEs) receiving a MAC PDU containing MCH scheduling information MAC CE containing a reserved value, it seems clear that different companies interpret the current text differently. In one instance the entire MAC PDU or MSI MAC CE will be ignored by the UE while other only ignore the field containing the reserved value.

For future extensibility it can be discussed if a clarification is necessary, i.e for the use of field which have been reserved; for which UE that do not implement the functionality can be assumed to be ignored, but where the remaining content of the MAC PDU are received accordingly.

In current MAC specification [1], we can in general find different options w.r.t reserved values/bits, e.g.:

“discard the received PDU”, 5.11 (extract above)

“The MAC entity shall ignore the value of Reserved bits in downlink MAC PDUs.” sect 6.1.1

Including the notion of “Reserved for future use”. E.g. sect. 7.1

In general it is beneficial if the handling of a reserved field/value is consistent throughout specifications, so that special consideration in UE implementations is not needed unless required for specific functionality.

This suggests a generic statement that applies if no function specific handling is specified. Looking at the current text in 36.321 considering the discussion up to now, it seems however that the remaining uncertainty is limited to the reception of reserved values receiving a MAC PDU on MCH.

To address this specific issue, a sentence on top level in Sect. 5 in [1] could state:

------

When a MAC entity receives a MAC PDU containing reserved values, the MAC entity shall:

-	ignore the corresponding fields in the PDU header and in the control elements containing reserved values and the corresponding parts indicated by the fields with reserved values in the received PDU

------

This would for example result in that a UE (MAC) would ignore a LCID n+STOP MTCH n, where n is between 2043-2046 in the MSI MAC CE, but still processes other LCID, STOP MTCH values. UEs are thus not expected to discard the entire MSI MAC CE or MAC PDU.

## Legacy Impact

If a sentence such as in the above example is introduced in Rel-12, the handling and UE behaviour from Rel-12 can be expected to be consistent in that only fields using reserved values are ignored when the MAC entity receives a MAC PDU on MCH. This also helps for future extension where reserved values can be used to extend and enhance functionality. However, the specification must be clearly understood such that any subsequent changes to reserved fields in functionality that exist in an earlier release still allow different handling of the MAC PDU.

The newly agreed suspension indication functionality avoids legacy impact as it restricts the change to Rel-12, by using a reserved value (as defined up to Rel-12) to the MSI using pmch-InfoListExt [5.12 in [1]]:

“…If pmch-InfoListExt is configured for an MCH, an Extended MCH Scheduling Information MAC control element is included in the first subframe allocated to the corresponding MCH within the MCH scheduling period to indicate the position of each MTCH and unused subframes on the MCH, and to indicate whether MTCH transmission is to be suspended.”

Note however that the uncertainty in Rel-12 and legacy UE behaviour upon receiving a reserved value remains, even though the reserved value range is changed. In order to remedy this, it is suggested to not to change the legacy text as suggested, but rather add an additional text:

---

When a MAC entity receives a MAC PDU on MCH containing reserved values, the MAC entity shall:

-	if pmch-InfoListExt is configured and the MAC PDU contains fields with reserved values

-	ignore the corresponding fields in the PDU header and in the control elements containing reserved values and;

-	 the corresponding parts indicated by the fields with reserved values in the received PDU

-	else

-	ignore the fields in the PDU header and the control elements containing reserved values and the corresponding parts indicated by the fields in the received PDU.

---

By this the handling is restricted to apply from Rel-12 (and pmch-InfoListExt).

It is thus proposed that:

Proposal 1: Agree to tie the configuration of pmch-InfoListExt to a clarified UE behaviour upon receiving a MAC PDU containing reserved values.

Proposal 2: In Rel-12, the UE ignores only headers and corresponding fields when receiving a MAC PDU on MCH with reserved values.

# Conclusion

In this contribution the UE behaviour for the use of fields on MTCH that have reserved values has been discussed. It is suggested that for Rel-12 and on, a UE that is configured with pmch-InfoListExt receiving a MAC PDU containing reserved values shall only ignore those fields, and process remaining content of the MAC PDU accordingly. Legacy UE behaviour is thus not impacted while the extensibility in later release is possible.

.

Based on the discussion in section 2 we propose the following:

Proposal 1	Agree to tie the configuration of pmch-InfoListExt to a clarified UE behaviour upon receiving a MAC PDU containing reserved values.

Proposal 2: In Rel-12, the UE ignores only headers and corresponding fields when receiving a MAC PDU on MCH with reserved values.

# References

3GPP TS 36.321 V12.5.0