3GPP TSG-RAN WG2 Meeting #89bis 	R2-151549

# Bratislava, Slovakia, 20-24 Apr 2015

| CR-Form-v11.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.321 | CR | CRNum | rev | - | Current version: | 12.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on handling of reserved values in MAC PDU on MCH |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI12, LTE-L23 |  |  |  |  |  |  | Date: |  | 2015-04-11 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-12 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) |  |
| Reason for change: |  | The UE handling upon receiving a MAC PDU on MCH containing a reserved value is unclear. In some instance the current UE behaviour entail that the entire MAC PDU or a MSI MAC CE will be discarded by the UE, while in other only the field containing the reserved value is ignored. The UE behaviour should be consistent in that only fields using reserved values are ignored when the MAC entity receives a MAC PDU on MCH. This also allows for future extension where reserved values can be used. |  |  |  |  |  |  |  |  |
| Summary of change: |  | It is clarified that a UE that is configured with pmch-InfoListExt and is receiving a MAC PDU on MCH containing reserved value shall ignore fields and corresponding headers containing reserved values, while processing the remaining content of the MAC PDU. Impact Analysis Impacted functionality: The change is limited to the MAC entity receiving a MAC PDU on MCH Inter-operability: If the CR is implemented by the UE but not by eNB, no interoperability problems are foreseen. If the CR is implemented by eNB, but not by the UE, there is a risk the UE does not receive any content in the MAC PDU or the MAC Control Element if reserved values are present in the MAC PDU on MCH. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A UE receiving a MAC PDU containing reserved values may not process the MAC PDU or MAC CE |  |  |  |  |  |  |  |  |
| NS |  |  |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.11 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

--- Parts omitted ---

## 5.11	Handling of unknown, unforeseen and erroneous protocol data

When a MAC entity receives a MAC PDU for the MAC entity’s C-RNTI or Semi-Persistent Scheduling C-RNTI, or by the configured downlink assignment, containing reserved or invalid values, the MAC entity shall:

-	discard the received PDU.

When a MAC entity receives a MAC PDU on MCH containing reserved values, the MAC entity shall:

-		if pmch-InfoListExt is configured and the MAC PDU contains fields with reserved values

-	ignore the corresponding fields in the PDU header and in the control elements containing reserved values and;

-	 the corresponding parts indicated by the fields with reserved values in the received PDU

-	else

-	ignore the fields in the PDU header and the control elements containing reserved values and the corresponding parts indicated by the fields in the received PDU.

--- Pats Omitted ---