3GPP TSG-RAN WG2 #89bis	R2-151684

Bratislava, Slovakia, 20–24 Apr 2015 	revision of R2-151526

Agenda Item:	7.3.2

Source:	Ericsson

Title:	Initial Radio resource efficiency evaluation of Single-Cell PTM

Document for:	Discussion

# Introduction

At RAN#66, a new RAN2 study item on “Support of single-cell point-to-multipoint transmission in LTE” was agreed [1]. The focus of the study is to investigate technical solutions for single-cell point-to-multipoint (SC-PTM) transmission in E-UTRAN and evaluate the gains and feasibility of the solutions. One major objective of the SI is improved radio resource efficiency.

# Radio resource efficiency

## General discussion

In the study item description, the current mechanisms for MBMS are described as non-optimised for dynamically changing number of groups, which are localised and have dynamically changing traffic load. There are situations when pre-assigned resources for critical services may remain unused. The SI should thus study and evaluate the gain in resource efficiency for solutions in relevant scenarios considering defined performance requirements and by comparing to existing functionality such as unicast and eMBMS in MBSFN mode.

SI lists some potential drawbacks with current MBSFN mode:

- Subframe granularity: if eMBMS data does not fill the whole subframe, no frequency multiplexing with unicast is possible due to the different subframe structure in the data region. MBSFN subframes have a longer OFDM symbol duration and CP duration.

- If the MBSFN area is small or only consists of one cell, the use of the extended CP is not needed. The longer CP of MBSFN mode implies a lower code rate for a given MCS, or, conversely, in order to achieve the same desired BLEP for a given SINR a lower MCS has to be used, so that less data can be transmitted per subframe.

- If there are no data transmissions the overhead for pre-configured eMBMS bearers with no data transmission can be up to 2.5 % of the total subframes if the delay-acceptable MSP of 40 ms that has been introduced for GCSE Rel-12, is used

The following aspects should be taken into account in the discussion of SC-PTM:

- Without link adaptation the MCS configuration must be chosen for cell edge UEs

  - The switching point is expected to be around 5 users per cell according to [2] for 95 % coverage and 1 % BLER

  - The use of MIMO is questionable if the transmission mode has to be selected for cell-edge UEs

  - If there are multiple neighboring cells providing the MBMS service via SC-PTM, there will be no MBSFN gain.

- For better adaptation to the dynamic user distribution and group size, it might be beneficial to discuss feedback mechanisms to allow for adaptive MCS selection and retransmissions specifically for cell-edge UEs. This will on the other hand increase the complexity.

The resource waste is only relevant when unicast resources are scarce and multiplexing of MBMS and unicast traffic is required. If unicast resources are scarce in a specific cell, the likelihood that neighboring cells are also crowded is rather high. With more interference from neighboring cells it seems the use of MBSFN transmission would be more efficient.

Consequently, for the comparison of PTP, SC-PTM and MBSFN should be for high load scenarios .

In the following we present initial results for the SC-PTM performance.

## Simulation assumptions

For the initial simulations, we chose the 3GPP urban scenario because emergency incidents are more likely to happen in urban areas. We use the following assumptions:

- 3GPP case 1, including vertical eNB antenna pattern according to TR36.814 with 15 el. tilt.

- ISD = 500 m

- no macro mobility

- statistically representative # of group-drops

- 10MHz bandwidth

- 2GHz carrier frequency

- 95% UEs with BLER<1% (95 % is typically used for latency requirements for MCPTT, see TS 22.179 [3])

  - For public safety, it should be discussed to ensure BLER < 1% for 99 % of the UEs, as this is not about e.g. late packet arrival, but packet loss.

- 100 % cell load  maximum interference

- 19 sites, 57 cells

- 2x2 MIMO with Alamouti and IRC

- CFI=2

For public safety, it should be discussed whether to ensure BLER < 1% for 99 % of the UEs, since packet loss is more severe than late packet arrival or delayed call setup.

## Simulation results for SC-PTM

Figure 1 shows the CDF of the BLER without HARQ for all MCS (MCS <=28 are as defined by 3GPP, MCS>28 are the MCSes defined by 3GPP for 256QAM, just reindexed). It becomes clear that the requirement of 95% UEs with BLER<1% is just met for MCS 4, i.e. without HARQ and without adapting the MCS to the (worst) positions of the UEs in a group, MCS 4 can be supported, i.e. a spectral efficiency of 0.36b/s/Hz. In all our spectral efficiency results reported in this document, each received PTM bit is counted only once regardless by how many UEs it is received.

Figure 1 - CDF of BLER

In the following the 5% UEs with largest MCS-0 BLEP are disregarded. . We split the UE population into groups. All groups in one simulation have the same number of UEs.

Figure 2 shows spectral efficiency vs group size where for each group the largest MCS is chosen to achieve BLEP>1% for all UEs (ideal MCS adaptation, i.e. no CQI measurement errors or reporting delays). The spectral efficiency for each group size is the average over all groups. The blue curve is for the case without HARQ. As expected for small groups a tremendously higher spectral efficiency can be achieved, and even for group size of 10 UEs the gain is still significantly higher than the 0.36b/s/Hz without group specific MCS adaptation.

The red curve is for the case of at most 2 retransmission on average. The retransmissions are separated by a time such that the UE travels by a quarter-wavelength, so that the channel gets de-correlated. Retransmissions are performed for a transport block and a group until all of the 95% best UEs in the group have BLEP<1% (estimated from the SINR).

The green curve shows the result for unlimited number of retransmissions. The spectral efficiency gain is achieved mainly by the further increased time diversity achieved by the retransmissions. However, for low UEs speed of 3km/h and at 2GHz, travelling a quarter wavelength takes 45ms, so the transport block delay limit does not allow for much time diversity. The green curve is therefore considered to be an impractical upper bound for retransmissions gains. Summarizing, HARQ retransmissions are very beneficial to achieve an increase in the spectral efficiency .

.

Figure 2 - Spectral efficiency vs # UEs per group, for different average number of retransmissions.

## Comparison with MBSFN

We have simulated a scenario where the user group has the center cell number 1 as the serving cell, see Figure 3.  The group service is provided by  MBSFNs of 3 different sizes, 1, 7 or 19 cells. All MBSFNs are centered on cell number 1. The example of Figure 3 shows the case of MBSFN area of 19 cells. The remaining of the in total 57 cells generate interference at full resource utilization. The MBSFN cells that are not selected by group users as serving cells are only part of the MBSFN to provide useful signal aggregation and to increase the distance to the interfering cells. They are called here supporting cells. An alternative to the simulated scenario would be to mute some border cells of the MBSFN. For legacy eMBMS, the number of UEs in the group is irrelevant  because there is no MCS adaptation to UE CQIs and no HARQ.

.  For the other MBSFN area sizes a higher MCS can be selected and the resulting spectral efficiencies are listed in Table 1.

The scenario for # MBSFN cells=1, in other words a single-cell MBSFN, is except for the lower number of resource elements (due to extended CP) and lack of Alamouti TX-diversity equivalent to the SC-PTM scenario without HARQ and group specific MCS adaptation, where section 2.3 reported a spectral efficiency of 0.36b/s/Hz. Accordingly, the result of the SC-MBSFN is with 0.22b/s/Hz lower.

Figure 3 - MBSFN simulation deployment: red cells: MBSFN; yellow cells: interfering

| # MBSFN cells | 1 | 7 | 19 |
| --- | --- | --- | --- |
| spectral efficiency [b/s/Hz] | 0.22 | 1.29 | 1.64 |
| # supporting cell rings | 0 | 1 | 2 |

Table 1 - MBSFN simulation results without weighting, but users only located in center cell

Based on these results, we can calculate mean spectral efficiency over the entire MBSFN area,,as the spectral efficiency for the respective number of supporting rings from Table 1 times the number of group cells divided by the total number of MBSFN cells.

In Table 2, we have extended this methodology to the case where the group users are distributed in multiple cells in the center of the MBSFN, assuming that for the spectral efficiency result of Table 1 the most characteristic property is not the # cells in the MBSFN but the # rings of supporting cells. We do this only for the cases of 1 and 2 supporting rings, not for zero supporting rings, because this achieved very low spectral efficiency in Table 1.

Group cell: 	cell that contains at least one group member; 
in such a cell, SC-PTM transmission would be active

Supporting cell: 	cell that participates in the MBSFN transmission, but does not contain any group members; in such a cell SC-PTM transmission would be switched off

Supporting cell ring: tier around the group cell(s) 
 homogenous interference protection at the MBSFN area border

.

| # group cells | 1 | 1 | 1 | 3 | 3 | 7 | 7 | 19 | 19 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| # supporting cell rings | 0 | 1 | 2 | 1 | 2 | 1 | 2 | 1 | 2 |
| # supporting cells | 0 | 6 | 18 | 9 | 24 | 12 | 30 | 18 | 42 |
| # MBSFN cells | 1 | 7 | 19 | 12 | 27 | 19 | 37 | 37 | 61 |
| spectral efficiency [b/s/Hz] | 0.22 | 0.18 | 0.09 | 0.32 | 0.18 | 0.48 | 0.31 | 0.66 | 0.51 |

Table 2 - MBSFN extrapolated mean spectral efficiency

It can be seen that the case of 1 supporting cell ring is always better than the case of 2.  For 19 group cells the mean spectral efficiency is with 0.66b/s/Hz  about equal to the SC-PTM spectral efficiency with 2 retransmissions for groups  of10 UEs. The latter, however, will be further reduced with non-ideal group specific MCS adaptation.

# Conclusion

In this contribution we presented initial results for resource efficiency of single-cell PTM with and without feedback. Regarding the general assumptions and the packet loss requirements specifically, we propose:

- For public safety, it should be discussed whether to ensure BLER < 1% for 99 % of the UEs, since packet loss is more severe than late packet arrival or delayed call setup.

Then, with respect to actual SC-PTM simulation results, as expected the gain of ideal group-specific MCS adaptation is very high for small group size and decreases quickly with increasing group size, but remains decent up to 10 UEs, though more detailed MCS adaptation simulations will be required to confirm this. HARQ with up to 2 retransmissions spread in time to exploit time diversity achieve a further gain of about 0.1-0.2 b/s/Hz regardless of group size in the range of 1-10.

A small MBSFN with 61 cells providing a group service to the central 19 cells achieves the same average spectral efficiency calculated over all 61 cells as this SC-PTM configuration for group size of 10 UEs per cell.

SC-PTM without group specific MCS adaptation and HARQ is worse than MBSFN already for a group service area of 3…7 cells.

Capture the above results in the draft TR 36.890

# References

RP-142205, “New Study Item Proposal for Support of single-cell point-to-multipoint transmission in LTE”, Huawei, HiSilicon, RP#66, 2014/12/08-11

RP-141920, “Motivation of Rel-13 New Study Item proposal for Support of single-cell point-to-multipoint transmission in LTE”, Huawei, HiSilicon, RP#66, 2014/12/08-11

TS 22.179 , “Mission Critical Push To Talk (MCPTT) over LTE”, 3GPP, V13.0.1, 2015/01/05