3GPP TSG-RAN WG2 #90	Tdoc R2-152415

Fukuoka, Japan, 25–29 May 2015

Agenda Item:	7.11

Source:	Ericsson

Title:	Areas for reducing latency

Document for:	Discussion, Decision

# Introduction

Latency is an important aspect of many applications that affects the end user experience. The current latency performance shows potential for large improvements, and a reduced latency can have a major impact on e.g. FTP download speeds. In this paper we outline two relevant classes of solutions which both aim at achieving a significant latency reduction.

# Discussion

## Areas for reducing latency

### Fast initial uplink grant

As discussed in [4], the scheduled uplink transmission for the initial packet includes Scheduling Request transmission and subsequent transmission at the time of the initial grant. Thus the delay is 10 ms – 20 ms depending on SR periodicity.

Table 1. Typical radio access latency components (rel. 8) for an UL transmission from a UE without a valid uplink grant.

Fast uplink grant refers to a solution area where there is no need to go through the same lengthy procedure from SR until data transmission. For example, one solution is to schedule the UE so that the grant is always available. Fast uplink scheduling can be achieved with protocol enhancements in L2 like in MAC protocol without changing physical layer principles like TTI length.

### TTI shortening

The packed latency can be reduced with a reduction of transport time of data and control by addressing the length of a TTI. This length of the TTI will have an impact both of the time for transmitting over air and on processing time in transmitter and receivers.

The length of the TTI can be reduced, as compared to the 1 ms sub-frame in LTE release 8, by defining the TTI to be constructed of less than 14 OFDM or SC-FDMA symbols. This reduction in number of symbols should be done for both downlink and uplink.

The time required to prepare the transmitter will be impacted of the number of OFDM or SC-FDMA symbols used in the TTI. This transmitter processing time will have an impact of the time budget for HARQ round trip time.

The layer 1 format of the physical format will also have impact on the receiver processing time, with a processing need for demodulation and channel decoding. Other layer 1 aspects such as placement of reference signals in relation to user data might also impact processing time. With reference signals placed early in the transmitted TTI, the channel estimate and thus also the demodulation of user data can start earlier as compared to placing the reference signals later in a TTI. Also different error correction aspects such as interleaver design and channel coding selection (e.g. block codes, convolution codes or turbo codes) can have a large impact of the receiver processing time. This receiver processing time will have an impact of the time budget for HARQ round trip time.

In current LTE, the UE should transmit a HARQ-ACK for FDD in sub-frame n for a detected PDSCH in sub-frame n-4 [2]. Similarly, an HARQ-ACK received by PHICH in subframe i, is associated to PUSCH transmission in subframe i-4 [1] for FDD.  Similar requirements for UL and DL are given for TDD. Length of TTI and processing requirements needs to be analysed in order to design this uplink/downlink HARQ-ACK timing with reduced TTI.

The uplink transmission in sub-frame n+4 is configured by PDCCH/EPDCCH with DCI format 0 or 4 or PHICH transmission in sub-frame n [3]. This delay of 4 subframes should be significantly reduced in in order to reduce latency. With a reduction of the TTI from one sub-frame to a few OFDM or SC-FDMA symbols, this delay from uplink grant to uplink transmission might be reduced with a similar amount. Here, the required delay depends on the receiver processing time of the grant, and transmitter processing time for PUSCH. Length of TTI and processing requires needs to be analysed in order to design this uplink grant delay.

## Potential gain from latency reductions

In this section we provide simulation results showing the gain from applying two latency reductions techniques, a faster first UL grant, and Shorter RTT. We analyse the performance of a single FTP download over TCP for file sizes between 100 kB and 5 MB. In addition to the radio network delay we have assumed a fixed delay of 10 ms from core, transport and Internet.

For the LTE radio part we consider a system with a 10 TTI PUCCH cycle, an 8 TTI SR to grant period, and an 8 TTI HARQ round trip time, allowing for 8 HARQ processes. For the LTE baseline we consider a TTI length of 1ms (14 OFDM symbols).

With the technique of TTI shortening we scale the TTI length to 7 and 2 OFDM symbols as examples. Scaling down to 1 symbol is also a realistic possibility. The TTI scaling also directly scales the HARQ RTT, and we assume a proportional scaling also of the transport block size. For simplicity, we further assume that a reduction of the TTI length scales also the SR to grant period, as well as the PUCCH cycle.

With the technique of a faster first UL grant, the UE does not need to undergo this PUCCH cycle nor SR to grant period, i.e. the grant acquisition delay is assumed to be 0ms with this technique.

In Table 1 below we summarize some of our simulation parameter setups.

| Simulation scenario | Static UE in a cell |
| --- | --- |
| DL and UL throughput | DL 150 Mb/s, UL 50 Mb/s |
| Number of OFDM symbols per TTI, same in both DL and UL | 2, 7, 14 corresponding to - HARQ RTT: of 1.14 ms, 4 ms, 8 ms - SR to grant period: 1.14, 4 ms, 8 ms - PUCCH cycle: 1.4 ms, 5 ms, 10 ms |
| HARQ error probability | 0.3 for 1st transmission, 0.1 for 2nd, 0.05 for 3rd, 0.01 for 4th, … |
| Traffic model | FTP download |
| Core, transport and internet network delay | 10ms |
| FTP file size | [100 kB, 500 kB, 1 MB, 5 MB] |
| SR period | 10 TTI |

Table 1. TCP download simulation setup

Figure 1: Download speed and delay, as well as relative gains in speed and delay compared to baseline for different file sizes and latency reduction techniques in LTE. The baseline is 1 ms TTI and SR.

From the results in Figure 1 it is obvious that significant gains in download delay and FTP object bit rate can be achieved at smaller FTP file sizes with the two studied latency reduction techniques. Generally, the smaller the FTP file size is (for a given link level throughput), the larger the fraction of the TCP slow start phase on the overall download time. During the TCP slow start phase the maximum achievable throughput is not fully utilized, instead the performance depends on the RTT, i.e. directly on the latency.

It becomes evident that a high gain in terms of overall download time reduction can be achieved with a faster first UL grant (green line); compared to the baseline the download time is reduced by 20% for smaller file sizes. A similar and somewhat higher reduction can be achieved with a 7 symbol TTI (0.5ms), i.e. 25% reduction (red line). It is noteworthy that these techniques can complement each other. With a combined use, reductions of about 38% can be achieved for the download time (cyan line).

Further reductions are achievable with further reduction of the TTI length. In the case of 2 OFDM symbol TTI (1/7ms) and use of a faster first UL grant as well (orange line), the total download time reduces by 50% at the smaller file sizes. For a 0.5 MB FTP download this corresponds to a reduction from a 0.35 s to 0.18 s download time.

# Summary and Conclusion

In this document we show how a short TTI and a faster grant acquisition may impact the end delay and corresponding performance for an example FTP download payload size. In summary the following observations and conclusions can be made:

A scaling of the TTI length has a large impact on the FTP download speed for smaller file sizes. This is due mainly to the TCP ramp up phase. The reduction in delay for files below 1 MB amounts to >40% with a 2 OFDM symbol TTI length.

Removing the delay due to UL grant acquisition reduces the delay for files below 1 MB with 20%.

Applying both techniques can reduce the delay even further.

It should be noted that not only TCP/IP as exemplified in this contribution benefit from reduced end to end packet delay; and assuming similar boundaries as a result of a shorter TTI, RTT and faster grant acquisition, also the performance impact on other application and transmission protocols and applications should be studied.

In view of the discussion and observations in this document it is proposed to:

Proposal: Capture the general findings on improvements based on reduced scheduling latency, shorter RTT and TTI.

# References

3GPP TS 36.213 section 8.3

3GPP TS 36.213 section 10.2

3GPP TS 36.213 section 8.0

R2-152326, Latency reductions in LTE, Ericsson, RAN2#90, May 25-29th 2015