3GPP TSG-RAN WG2 Meeting #90	R2-152440

# Fukuoka, Japan , 25th – 29th May 2015

Agenda item:		7.10

Source:	Sony

Title:	UE preferred eDRX behaviour

Document for:		Discussion and Decision

# 1	Introduction

SA2 is studying in Rel-13 the feasibility of extending DRX cycle for both idle and connected mode as part of the Study Item FS_eDRX. A number of issues are being discussed, and which are captured in TR 23.770 [2].

RAN has approved to start a new RAN2 lead WI, RAN enhancements for extended DRX in LTE, RP-150456 [1].

The general objective of this work item is to provide RAN specifications for extending the DRX cycles for both idle mode connected mode.

The work will continue the work started in Rel-12 in TR 23.887 and TR 37.869 for Extended DRX, and impact and work relates also to SA2/CT1 and RAN4.

We make the following observations;

A) The UE possibility to indicate its interest in extended DRX

In the SA2 report [2] the UE has the possibility to indicate its interest in using extended DRX.

This seems to be a good approach, but couldn’t there also be a possibility for the terminal to regularly send proposed DRX/DTX usage pattern, based on the existing transmission and reception needs from the ongoing services within the terminal? This could in particular be of use for MTC type of devices where the data traffic pattern and expected delay tolerance is well known in the UE.

B) Differentiation between DRX pattern for paging occasion and measurements

We notice that the DRX pattern is mainly designed for the UE to save power between periods when it has to listen to paging and system information, and when doing so the UE will also do some radio measurements to verify it is camping on a suitable cell.

Though, does the UE always have to do the combined activity of both listen for paging and doing measurements? In case the time between the paging occasions is long (a long eDRX value) there could be a system benefit of ensuring the UE makes one or more channel measurements in between paging interval.

# 2	Discussion on UE preferred eDRX behaviour

In TR 23.770 [2] a number of issues have been raised, where some are related to Idle mode DRX.

Issue 1:

-	Support NAS-protocol extensions to enable extended DRX cycle. There needs to be coordination between UE and Core Network (SGSN/MME) for use of extended DRX.

Issue 2:

-	Paging strategy in CN that fits the needs of the extended DRX cycle and normal DRX cycle.

Issue 6:

- Interaction between eDRX and PSM (Power Saving Mode).

Updated solution proposal for Issue 1

A solution for issue 1 is presented, see figure 1.

In an updated solution it could be clarified that the UE also has the possibility to indicate specific DRX cycle lengths.

In idle mode, the UE would transmit desired DRX pattering in Attach/TAU request. For connected mode case, this can be discussed later if found relevant.

UE

eNB

MME

UE upper layer decide using

long paging DRX and set corresponding

parameter if necessary

MME decides to transmit the UE

specific extended paging DRX in

S

1

AP Paging message

S1AP Paging (extended paging DRX)

MME obtains the maximum extended paging DRX info for the TA, either from configuration data or fromS1 Setup Request

According to extended long DRX parameters setting UE and

RAN perform the DRX for paging

MME decides DRX value to use using UE proposed value, DRX supported by RAN and policies etc.

.

.

1

3

NAS: Attach/TAU Accept (Applied DRX value)

UE may inform upper layer of the

expected delay for MT communication

2

NAS: Attach/TAU Accept (Applied DRX value)

NAS: Attach/TAU Request (eDRX value)

NAS: Attach/TAU Accept (eDRX value)

Figure1 Procedure of extended DRX determination in E-UTRAN

Proposal 1: Clarify that the UE does not only have the possibility to indicate its interest in extended DRX, but also when relevant being able to indicate its preference for a specific DRX value.

Additional solution proposal for Issue 2:

For issue 2, “Paging strategy in CN that fits the needs of the extended DRX cycle and normal DRX cycle”, solution for paging strategy was presented for the SGSN/MME at the last SA2#108 meeting in Los Cabos [3], but no strategy based on UE preference has been presented..

One potential solution could be to have it possible to differentiate between the DRX patterns used for paging occasion and when the UE is doing measurements. E.g. a UE that is fairly stationary, could do measurements more seldom than listen to the paging channel. On the other hand, a UE that is fairly mobile, but is seldom receiving MT calls, could listen to the paging channel less frequently, but needs to do measurements more often to be able to camp on a suitable cell and in order to be prepared for MO calls, i.e. initiation of the RACH procedure.

E.g. not all MTC devices are stationary, then it would be relevant for a UE in the border area of a TAU would benefit for doing measurements for frequently in order to signal correct TAYU to the network.

Proposal 2: Specify possibility to have different DRX schemes for Paging occasions and when doing measurements.

Conclusion for Issues 6:

For issue 6 we see that it might be possible to configure both PSM and extended DRX, but see no real use case or reason to do so.

Proposal 3: For issue 6 we propose to clarify the use case for configuring both PSM and extended DRX simultaneously.

# 3	Conclusion

In this contribution we have presented two proposals which we propose to initially be discussed, and if found appropriate to be included in the relevant specifications.

Proposal 1: Clarify that the UE does not only have the possibility to indicate its interest in extended DRX, but also when relevant being able to indicate its preference for a specific DRX value.

Proposal 2: Specify possibility to have different DRX schemes for Paging occasions and when doing measurements.

Proposal 3: For issue 6 we propose to clarify the use case for configuring both PSM and extended DRX simultaneously.

# 4	References

- RP-150493 New Work Item Proposal: RAN enhancements for extended DRX in LTE

- 3GPP TR 23.770. Ver. 0.1.0. Study on System Impacts of Extended DRX Cycle for Power Consumption Optimization.