3GPP TSG-RAN WG2 #90	Tdoc R2-152521

Fukuoka, Japan, 25–29 May 2015

Agenda Item:	7.8

Source:	Ericsson

Title:	Packet delay and drop rate for MDT QoS measurements

Document for:	Discussion, Decision

# Introduction

RAN plenary has agreed a new Study Item evaluating MDT procedures and capabilities to support QoS and its limiting factors for MMTEL voice and video traffic [1]. In subsequent work, RAN2 has in RAN2#89bis, agreed that latency and packet Loss metrics, for both UL and DL are desirable for GBR traffic.

In this contribution, some details on possible solutions providing latency and packet loss measurement are outlined.

# General

It can be assumed that a ideal goal from operators is to be able to operate e.g. VoLTE service with an end-to-end packet delay of less than 150ms with a minimum of 98% packet success rate as recommended in [2]. It is in view of this valuable to have a possibility of receiving regular information as input to the RRM and scheduling strategy decision processes. By defining UE measurements and reporting the above goal should be able to achieved. It should be noted though that limited complexity in solutions should be sought, so that statistical numbers can be achieved as a result of a large population of UEs support the enhanced reporting.

## Existing MDT and OAM functionality

In the current MDT functionality of Rel-12, we today have several means of collecting measurement reports and logs form UE that in parallel with traces and logs at the NW side (eNB) may assist in operators service performance evaluation. For example, In addition to the eNB specific Data Volume measurement and the Scheduled IP throughput measurements for MDT, 4.1.4 in [4] also defines L2 measurements for Packet Delay in the DL per QCI and in 4.1.5 Data Loss in the DL, UL and Uu per QCI.

# UL latency and packet loss measurement

As can be seen above, it can be assume that at least for DL, packet delay and packet loss rate is observable. Furthermore, shortcomings of the already defined measurements (e.g. in the eNB) should in many cases be possible to address by implementation.

The difference in relative arrival time between packets, or jitter, is an important aspect of conversational services like Voice. UL Scheduling Request (SR) based transmissions; including the aspects on varying delay between packets caused by HARQ, may introduce intermittent queuing delays due to coverage and load. As low jitter variation in general is beneficial for a stable system with steady packet stream, small delay variation of received packets is preferred. This reduces reordering delays and eases detection of lost packets. High jitter and packet loss on the other hand may result in poor service quality. It is hence of interest to be able to track this aspect in the NW and be able to determine what e.g. parameters need to be tuned. As As the UL packet delay variation (jitter) usually is not proportional to the DL delay and more influenced by coverage problems and scheduling delay, it requires additional information from the sending entity, UE.

The measurements provided by the UE to the NW, for example UE reporting per QCI should discriminate between losses on Uu, and due to packet discard in PDCP (SDU) caused by long queuing delay. Currently only the loss on Uu (c.f. UL) is possible to monitor at the eNB, and thus a packet discard mechanism report from the UE is beneficial.

The following possible measurements can be envisioned:

- A count of SDUs exceeding delay budget (i.e packet discard rate)

  - Based on configurable upper bound timer value or implementation specific (i.e UE specific)

  - Is a logged parameter per QCI per each meas. period (e.g. logging period)

  - Measurement period configurable, or logs are accompanied with the number of packets sent (c.f. MBMS BLER measurement)

- Measurement to account for/estimate jitter

  - With UE providing a histogram of packet delay, or

  - Concatenate each PDCP PDU with a time stamp, or

  - Include an avg. queuing delay

For the case when UE includes an avg. packet queuing delay, it can be assumed that the eNB could estimate the jitter if all packets have equal length and inter-arrival time at the transmitter. To this an assumption at the receiver is also needed for the probability of packet loss, i.e. estimation on that a packet is lost (with a resulting delay) at the transmitting end with probability p independently of other packets. Note that, if service multiplexing were present, the estimation would increase in complexity.

With the current DL queuing delay measurement in the eNB as base line, a similar to that measurement could be defined for UL:

Packet Delay in the UL per QCI: This measurement refers to packet delay for DRBs. For arrival of packets the reference point is PDCP upper SAP. For successful delivery the reference point is MAC lower SAP.

With the additional information, the NW could adjust RRM and scheduling strategies, for example HARQ op. point or even RLC parameters (RLC AM).

Proposal: For feMDT QoS measurements, RAN2 should evaluate the feasibility of jitter estimation based on a measurement such as avg. packet queuing delay and packet discard count of SDUs exceeding delay budget.

The logging should be periodic or otherwise related to a known time period for an ongoing service and reported per QCI.

## Complexity

The possible inclusion of the above measurements will naturally impact RRC connected mode UE measurements. The Study should thus also include a comprehensive evaluation of the complexity as well as the impact of existing MDT/OAM functionality.

# Conclusion

In this contribution, details on possible solutions providing latency and packet loss measurement for UL are outlined and it is proposed that:

Proposal: For feMDT QoS measurements, RAN2 should evaluate the feasibility of jitter estimation based on a measurement such as avg. packet queuing delay and packet discard count of SDUs exceeding delay budget.

# References

RP-150472, New Study Item Proposal: Further Enhancements of Minimization of Drive Tests for E-UTRAN, RAN#67 March 2015

ITU-T Recommendation G.114, “One way transmission time”, February 2003.

3GPP TS 37.320, Radio measurement collection for Minimization of Drive Tests (MDT); Overall description;

3GPP TS 36.314, Layer 2 – Measurements V12.0.0 (2014-09)

# Annex

# Excerpt from [4]

### 4.1.4	Packet Delay

#### 4.1.4.1	Packet Delay in the DL per QCI

The objective of this measurement is to measure L2 Packet Delay for OAM performance observability.

If there is one or more RNs served in a cell, for that cell the eNB performs each measurement separately for packets transmitted between the eNB and UEs and for packets transmitted between the eNB and RNs.

Protocol Layer: MAC, RLC, PDCP

| Definition | Packet Delay in the DL per QCI. This measurement refers to packet delay for DRBs. For arrival of packets the reference point is PDCP upper SAP. For successful reception the reference point is MAC lower SAP. The measurement is done separately per QCI. Detailed Definition: ,where explanations can be found in the table 4.1.4.1-1 below. |
| --- | --- |

Table 4.1.4.1-1

|  | Packet Delay in the DL per QCI, averaged during time period . Unit: Integer ms. |
| --- | --- |
|  | The point in time when PDCP SDU arrives. |
|  | The point in time when the last piece of PDCP SDU i was received by the UE according to received HARQ feedback information. |
|  | A PDCP SDU that arrives at the PDCP upper SAP during time period . PDCP SDU for which HARQ acknowledgement is not received for all parts shall not be included in the calculation. |
|  | Total number of PDCP SDUs . |
|  | Time Period during which the measurement is performed |

### 4.1.5	Data Loss

#### 4.1.5.1	Packet Discard Rate in the DL per QCI

The objective of this measurement is to measure packets that are dropped due to congestion, traffic management etc, for OAM performance observability.

For an eNB serving one or more RNs, packets transmitted between the eNB and RNs are excluded, i.e., only packets transmitted between the eNB and UEs are counted.

Protocol Layer: MAC, RLC, PDCP

| Definition | Packet Discard Rate in the DL per QCI. This measurement refers to discard for DRBs. One packet corresponds to one PDCP SDU. The reference point is PDCP upper SAP. The measurement is done separately per QCI. Detailed Definition: ,where explanations can be found in the table 4.1.5.1-1 below. |
| --- | --- |

NOTE:	Packet loss is expected to be small or very small The statistical accuracy of an individual discard rate measurement result is dependent on how many packets has been received, and thus the time for the measurement.

Table 4.1.5.1-1

|  | Packet Discard Rate in the DL per QCI, averaged during time period . Unit: number of discarded packets per received packets * 106, Integer. |
| --- | --- |
|  | Number of DL packets, for which no part has been transmitted over the air, of a data radio bearer with QCI = , that are discarded during time period in the PDCP, RLC or MAC layers due to reasons other than hand-over. |
|  | Number of DL packets of bearer with QCI = that has entered PDCP upper SAP during time period (NOTE). |
|  | Time Period during which the measurement is performed, Unit: minutes (NOTE). |

#### 4.1.5.2	Packet Uu Loss Rate in the DL per QCI

The objective of this measurement is to measure packets that are lost at Uu transmission, for OAM performance observability.

.If there is one or more RNs served in a cell, for that cell the eNB performs each measurement separately for packets transmitted between the eNB and UE and for packets transmitted between the eNB and RNs.

Protocol Layer: MAC, RLC, PDCP

| Definition | Packet Uu Loss Rate in the DL per QCI. This measurement refers to packet loss for DRBs. One packet corresponds to one PDCP SDU. The measurement is done separately per QCI. Detailed Definition: , where explanations can be found in the table 4.1.5.2-1 below. |
| --- | --- |

NOTE:	Packet loss is expected to be upper bounded by the PELR of the QCI which takes values between 10-6 and 10-2. The statistical accuracy of an individual packet loss rate measurement result is dependent on how many packets have been received, and thus the time for the measurement.

Table 4.1.5.2-1

|  | Packet Uu Loss Rate in the DL per QCI. Unit: number of lost packets per transmitted packets * 106, Integer. |
| --- | --- |
|  | Number of DL packets, of a data radio bearer with QCI = , for which at least a part has been transmitted over the air but not positively acknowledged, and it was decided during time period that no more transmission attempts will be done. If transmission of a packet might continue in another cell, it shall not be included in this count. |
|  | Number of DL packets, of a data radio bearer with QCI = , which has been transmitted over the air and positively acknowledged during time period . |
|  | Time Period during which the measurement is performed, Unit: minutes (NOTE). |

#### 4.1.5.3	Packet Loss Rate in the UL per QCI

The objective of this measurement is to measure packets that are lost in the UL, for OAM performance observability.

If there is one or more RNs served in a cell, for that cell the eNB performs each measurement separately for packets transmitted between the eNB and UE and for packets transmitted between the eNB and RNs.

Protocol Layer: PDCP

| Definition | Packet Loss Rate in the UL per QCI. This measurement refers to packet loss for DRBs. One packet corresponds to one PDCP SDU. Reference point is the PDCP upper SAP. The measurement is done separately per QCI. Detailed Definition: , where explanations can be found in the table 4.1.5.3-1 below. |
| --- | --- |

NOTE:	Packet loss is expected to be upper bounded by the PELR of the QCI which takes values between 10-6 and 10-2. The statistical accuracy of an individual packet loss rate measurement result is dependent on how many packets have been received, and thus the time for the measurement.

Table 4.1.5.3-1

|  | Packet Loss Rate in the UL per QCI. Unit: number of lost packets per transmitted packets * 106, Integer. |
| --- | --- |
|  | Number of missing UL PDCP sequence numbers, representing packets that are not delivered to higher layers, of a data radio bearer with QCI = during time period . If transmission of a packet might continue in another cell, it shall not be included in this count. |
|  | Total number of UL PDCP sequence numbers (also including missing sequence numbers) of a bearer with QCI = , starting from the sequence number of the first packet delivered by PDCP upper SAP to higher layers until the sequence number of the last packet during time period . |
|  | Time Period during which the measurement is performed, Unit: minutes (NOTE). |