3GPP TSG-RAN WG2#91bis	R2-154755								R2-15xxxx

5—9 October, Malmo, Sweden.

Agenda Item:	7.1.2

Source: 	Motorola Mobility

Title:  	RSSI measurement for LAA

Document for:	Discussion and Decision

# Introduction

Based on the understanding that RSRP measurements do not provide a clear picture of the load on the carrier, it has been agreed that the UE will perform RSSI measurements on LAA carriers. However, given the unpredictability of transmissions in LAA (discussed further below), RSSI may not provide an accurate representation of the load on the carrier. We discuss this issue and provide our observations about RSSI measurements in the presence of varying loads.

# Discussion

RSRP measurements do not give a clear picture of activity on the carrier. The UE may find a carrier which has a desired LAA cell (i.e., belonging to desired PLMN) and measure a high RSRP for the cell. On a different carrier the UE may find a desired LAA cell and measure a moderate RSRP for the cell. Based on the current measurement framework, the first carrier and the first cell would be chosen for LAA service. However, if the first carrier has a significantly higher level of activity/channel occupancy compared to the second carrier, the throughput that can be achieved on the first cell may be lower than the throughput that can be achieved on the second cell. Additionally, for some types of services (e.g., VoIP and other periodic packet arrival services) a carrier with low occupancy may be preferred.

It has been agreed that RSSI will be used to estimate load on carriers. The UE will report RSSI measurements for one or more LAA carriers. This will enable the network to choose whether and which carriers to configure as an LAA carrier for the UE. RSSI measurements will also be used to determine when to remove a carrier (due to excessive load from other nodes). Consequently, the measured RSSI has to effectively capture the time-varying load on the carrier.

In the current measurement framework, the UE generally performs measurements on discovery signal within periodically occurring DMTC (discovery signal measurement timing configuration) occasions. That is, for example, the UE can perform (legacy RSSI and other) measurements every 40 ms, which are then averaged/filtered. The periodic measurements generally do not allow the UE to obtain measurements that are representative of the time-varying load on the carrier.

## LAA-RSSI

[1] defines RSSI, as part of the RSRQ definition, as follows:

E-UTRA Carrier Received Signal Strength Indicator (RSSI) comprises the linear average of the total received power (in [W]) observed only in certain OFDM symbols of measurement subframes, in the measurement bandwidth, over N number of resource blocks by the UE from all sources, including co-channel serving and non-serving cells, adjacent channel interference, thermal noise etc.

Unless indicated otherwise by higher layers, RSSI is measured only from OFDM symbol containing reference symbols for antenna port 0 of measurement subframes. If higher layers indicate all OFDM symbols for for performing RSRQ measurements, then RSSI is measured from all OFDM symbols of the DL part of measurement subframes. If higher layers indicate certain subframes for performing RSRQ measurements, then RSSI is measured from all OFDM symbols of the DL part of the indicated subframes.

If higher layers indicate measurements based on discovery signals, RSSI is measured from all OFDM symbols of the DL part of the subframes in the configured discovery signal occasions.

For the purpose of measuring load on the carrier, the restriction to certain OFDM symbols of measurement subframes is not relevant. In fact, when the UE performs such a measurement, there may not even be a reference symbol/signal transmission present. So we note that there will need to be a new measurement quantity, which we will refer to as LAA-RSSI.

## Placement of Measurement Occasions

The DMTC window is the time duration during which all Discovery Signal signals (from the serving and neighbour LAA eNBs) are expected to be transmitted. Thus, measuring RSSI during the DMTC windows can result in an RSSI that indicates a load that is higher than the actual load on the carrier. For this reason, RSSI measurements should not be performed during DMTC windows.

Proposal 1: LAA-RSSI measurements are not performed during DMTC windows.

The UE needs gaps for performing measurements on “inter-frequency” LAA carriers – i.e., carriers without configured secondary cells. If the UE needs gaps to perform LAA-RSSI measurements, the network would need to configure specific subframes during which the LAA-RSSI measurements occur. Generally, it is expected that LAA-RSSI measurements will be configured in addition to RSRP measurements for a carrier. Therefore, in order to minimize the total number of times the UE switches between monitoring the DL and perform a measurement, it is preferable to have LAA-RSSI measurements occur immediately adjacent to the DMTC windows. This also has the benefit of not having to explicitly configure the LAA-RSSI measurement gaps.

Proposal 2: LAA-RSSI measurement gaps, if needed, are configured to occur immediately before or immediately after RSRP measurement gaps.

When the UE does not need a measurement gap to perform a measurement on an LAA carrier, the measurement can occur when the carrier is busy (due to transmissions from either the serving LAA eNB or from another node) or when the channel is idle. Measurements are considered to be representative of load on the channel, if the proportion of measurements that occur when the channel is busy is roughly the same as the proportion of time that the channel is busy. For convenience, we write this in terms of probabilities as follows:

Observation 1: For measurements to be representative of load on the channel:

P[Measurement occurring when Channel is Busy] – P[Channel being busy] should be minimized.

Note that the criterion described above is actually a high standard to meet. In fact, the only guaranteed way to have P[Measurement occurring when Channel is Busy] = P[Channel being busy] is to perform measurements continuously. However, the metric can be useful to compare among different measurement approaches to see which ones perform better.

Below, we illustrate how well the current measurement framework conforms to the criterion defined above. Consider a poisson packet arrival where packets arrive with an inter-arrival time of 40 ms. Each packet is assumed to be 10 ms long. For transmission over the channel, it is assumed that the packet is transmitted in 1 ms blocks which are separated in time by 8 ms. UE performs periodic measurements with a periodicity of 40 ms. The duration of the measurement is assumed to be 1 ms. Figure 1 shows a CDF corresponding to the criterion mentioned above. As can be seen from this, for periodic measurements, there is a rather large gap between P[Measurement occurring when Channel is Busy] and P[Channel being busy].

Observation: Periodic LAA-RSSI measurements may not provide an accurate representation of load on an LAA carrier.

Figure 1: CDF of Probability of Measurements occurring when Channel is busy vs Probability that the channel is busy, for Periodic and Random Measurements

Figure 1 also shows how well a random selection of measurement occasions performs with respect to the criterion above. Measurement occasions are selected randomly as follows. The duration between successive measurement occasions is chosen from the range of 10 – 70 ms, based on a uniform random distribution. The range of 10 –70 ms is picked to ensure that the mean duration between successive measurements is 40 ms (which is the periodicity used for the periodic measurements).

The random selection of measurement occasions can be modified to be more adaptive. In principle, if a measurement reveals that a channel is busy, a “busy channel inter-measurement duration” is used to determine the next measurement occasion. If a measurement reveals that the channel is not busy, a “quiet channel inter-measurement duration” is used to determine the next measurement occasion. The quiet channel inter-measurement duration is longer than the busy channel inter-measurement duration. Figure 1 further illustrates this in comparison to the periodic and random measurements. A quiet channel inter-measurement duration of 200 ms and a busy channel inter-measurement duration of 8 ms are used. The measurement duration during quiet periods is 6 ms and the measurement duration during busy periods is 1 ms. The adaptive measurements, as shown, result in measurements that are significantly more representative of the channel load.

Observation 2: Randomizing the measurement occasions can make the LAA-RSSI measurements more representative of the load on the channel.

Proposal 3: For “intra-frequency” LAA-RSSI measurements, randomization of measurement occasions should be considered to make the measurements representative of the load.

If measurement gaps are used for LAA-RSSI measurements, the time durations during which the UE performs the measurements can still be chosen randomly. For example, the UE could choose one of the subframes of the LAA-RSSI measurement gap pseudo-randomly based on a UE specific parameter such as C-RNTI or a cell specific parameter such as PCID.

Proposal 4: For “inter-frequency” LAA-RSSI measurements, the UE should pseudo-randomly choose a measurement occasion based on a UE specific parameter or a cell specific parameter.

# Summary

In this contribution we discussed various issues related to RSSI measurements for LAA. Below are our observations and proposals:

Proposal 1: LAA-RSSI measurements are not performed during DMTC windows.

Proposal 2: LAA-RSSI measurement gaps, if needed, are configured to occur immediately before or immediately after RSRP measurement gaps.

Observation 1: For measurements to be representative of load on the channel:

P[Measurement occurring when Channel is Busy] – P[Channel being busy] should not be minimized.

Observation 2: Periodic LAA-RSSI measurements may not provide an accurate representation of load on an LAA carrier.

Proposal 3: For “intra-frequency” LAA-RSSI measurements, randomization of measurement occasions should be considered to make the measurements representative of the load.

Proposal 4: For “inter-frequency” LAA-RSSI measurements, the UE should pseudo-randomly choose a measurement occasion based on a UE specific parameter or a cell specific parameter.

# References

- 3GPP TS 36.214