3GPP TSG-RAN WG2 Meeting #92	R2-156255

# Anaheim, USA, November 16-20, 2015

| CR-Form-v11 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.314 | CR |  | rev |  | Current version: | 12.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | New MDT measurement introduced by feMDT |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE-eMDT2 |  |  |  |  |  |  | Date: |  | 2015-10-29 |
| Category: | B |  |  |  |  |  |  | Release: |  | Rel-12 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-4 (Release 4) Rel-5 (Release 5) Rel-6 (Release 6) Rel-7 (Release 7) Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) |  |
| Reason for change: |  | feMDT is introducted in rel-13. And some new eNB measurements and UE measurements are agreed and should be captured in TS 36.314. UE measurements - UL PDCP Packet Delay per QCI eNB measurments - Packet Delay in the DL per QCI per UE - Packet Uu Loss rate in the DL per QCI per UE - Packet Loss rate in the UL per QCI per UE |  |  |  |  |  |  |  |  |
| Summary of change: |  | Definition of the new MDT measurements are added. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | New MDTmeasurements will not be reported to TCE for feMDT. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.1.4，4.1.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  | TS 37.320, TS 36.331 |  |  |  |
| affected: |  |  | x | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

Modified Subclause

1	Scope

The present document contains the description and definition of the measurements performed by E-UTRAN or the UE that are transferred over the standardised interfaces in order to support E-UTRA radio link operations, radio resource management (RRM), network operations and maintenance (OAM), mimization of drive tests (MDT) and self-organising networks (SON).

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

- References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

- For a specific reference, subsequent revisions do not apply.

- For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 36.321: "Evolved Universal Terrestrial Radio Access (E-UTRA) Medium Access Control (MAC) protocol specification ".

[3]	3GPP TS 36.322: "Evolved Universal Terrestrial Radio Access (E-UTRA) Radio Link Control (RLC) protocol specification".

[4]	3GPP TS 36.323: "Evolved Universal Terrestrial Radio Access (E-UTRA); Packet Data Convergence Protocol (PDCP) specification".

[5]	3GPP TS 36.331: "Evolved Universal Terrestrial Radio Access (E-UTRA); Radio Resource Control Protocol (RRC) specification".

Next Modified Subclause

4 Layer 2 measurements

4.1 E-UTRAN measurements performed by the eNB

4.1.4 Packet Delay

4.1.4.2 Packet Delay in the DL per QCI per UE

The objective of this measurement is to measure L2 Packet Delay for MDT for QoS verification of MMTEL video/voice traffic.

Protocol Layer: MAC, RLC, PDCP

| Definition | Packet Delay in the DL per QCI per UE. This measurement refers to packet delay for DRBs. For arrival of packets the reference point is PDCP upper SAP. For successful reception the reference point is MAC lower SAP. The measurement is done separately per QCI per UE. Detailed Definition: ,where explanations can be found in the table 4.1.4.1-1 below. |
| --- | --- |

Table 4.1.4.1-1

|  | Packet Delay in the DL per QCI per UE, averaged during time period . Unit: Integer ms. |
| --- | --- |
|  | The point in time when PDCP SDU arrives. |
|  | The point in time when the last piece of PDCP SDU i was received by the UE according to received HARQ feedback information. |
|  | A PDCP SDU that arrives at the PDCP upper SAP during time period . PDCP SDUs for which HARQ acknowledgement is not received for all parts shall not be included in the calculation. |
|  | Total number of PDCP SDUs . |
|  | Time period during which the measurement is performed. |

4.1.5	Data Loss

4.1.5.4	Packet Uu Loss Rate in the DL per QCI per UE

The objective of this measurement performed by eNB is to measure packets that are lost at Uu transmission, for MDT for QoS verification of MMTEL video/voice traffic.

Protocol Layer: MAC, RLC, PDCP

| Definition | Packet Uu Loss Rate in the DL per QCI per UE. This measurement refers to packet loss for DRBs. One packet corresponds to one PDCP SDU. The measurement is done separately per QCI per UE. Detailed Definition: , where explanations can be found in the table 4.1.5.4-1 below. |
| --- | --- |

NOTE:	Packet loss is expected to be upper bounded by the PELR of the QCI which takes values between 10-6 and 10-2. The statistical accuracy of an individual packet loss rate measurement result is dependent on how many packets have been received, and thus the time for the measurement.Table 4.1.5.4-1

|  | Packet Uu Loss Rate in the DL per QCI per UE. Unit: number of lost packets per transmitted packets * 106, Integer. |
| --- | --- |
|  | Number of DL packets, of a data radio bearer with QCI = and UE with index =, for which at least a part has been transmitted over the air but not positively acknowledged, and it was decided during time period that no more transmission attempts will be done. If transmission of a packet might continue in another cell, it shall not be included in this count. |
|  | Number of DL packets, of a data radio bearer with QCI = and UE with index =,which has been transmitted over the air and positively acknowledged during time period . |
|  | Time period during which the measurement is performed, Unit: minutes (NOTE). |

4.1.5.5	Packet Loss Rate in the UL per QCI per UE

The objective of this measurement is to measure packets that are lost in the UL, for MDT for QoS verification of MMTEL video/voice traffic.

Protocol Layer: PDCP

| Definition | Packet Loss Rate in the UL per QCI per UE. This measurement refers to packet loss for DRBs. One packet corresponds to one PDCP SDU. Reference point is the PDCP upper SAP. The measurement is done separately per QCI per UE. Detailed Definition: , where explanations can be found in the table 4.1.5.5-1 below. |
| --- | --- |

NOTE:	Packet loss is expected to be upper bounded by the PELR of the QCI which takes values between 10-6 and 10-2. The statistical accuracy of an individual packet loss rate measurement result is dependent on how many packets have been received, and thus the time for the measurement.

Table 4.1.5.5-1

|  | Packet Loss Rate in the UL per QCI per UE. Unit: number of lost packets per transmitted packets * 106, Integer. |
| --- | --- |
|  | Number of missing UL PDCP sequence numbers, representing packets that are not delivered to higher layers, of a data radio bearer with QCI = and UE with index =during time period . If transmission of a packet might continue in another cell, it shall not be included in this count. |
|  | Total number of UL PDCP sequence numbers (also including missing sequence numbers) of a bearer with QCI = and UE with index =, starting from the sequence number of the first packet delivered by PDCP upper SAP to higher layers until the sequence number of the last packet during time period . |
|  | Time period during which the measurement is performed, Unit: minutes (NOTE). |

4.2 E-UTRAN measurements performed by the UE

4.2.1 Packet Delay

4.2.1.1 UL PDCP Packet Delay per QCI

The objective of this measurement performed by UE is to measure Excess Packet Delay Ratio in Layer PDCP for MDT for QoS verification of MMTEL video/voice traffic.

Protocol Layer: RLC, PDCP

| Definition | PDCP Packet Delay in the UL per QCI. This measurement refers to packet delay for DRBs, which captures the delay from packet arrival at PDCP upper SAP until the packet starts to be delivered to RLC. The measurement is done separately per QCI. Detailed Definition: ,where explanations can be found in the table 4.2.1.1-1 below. |
| --- | --- |

Table 4.2.1.1-1

|  | Ratio of packets in UL per QCI exceeding the configured delay threshold among the UL PDCP SDUs transmitted. |
| --- | --- |
|  | Number of PDCP SDUs of a data radio bearer with QCI = ,for which ULdelay exceeded the configured delayThreshold as defined in TS36.331 [5] during the time period T. |
|  | Number of PDCP SDUs of a data radio bearer with QCI = , for which at least a part of SDU was transmitted during the time period T. |
|  | Queueing delay observed at the UE PDCP layer from packet arrival at PDCP upper SAP until the packet starts to be delivered to RLC, the packet belongs to a data radio bearer with QCI = . |
|  | The point in time when the PDCP SDU i of a data radio bearer with QCI =was delivered to lower layers. |
|  | The point in the time when the PDCP SDU i of a data radio bearer with QCI = arrives at PDCP upper SAP. |
|  | Index of PDCP SDU that arrives at the PDCP upper SAP during time period . |
|  | Time period during which the measurement is performed. |

4.2.1.1.1 Measurement report mapping for PDCP SDU queuing delay

UL PDCP SDU queuing delay shall be measured according to configuration as defined in TS36.331 [5].

The UE shall report UL PDCP SDU queuing delay as the ratio of SDUs exceeding the configured delay threshold and the total number of SDUs received by the UE during the measurement period.

The reported excess PDCP queuing delay ratio is mapped to 32 [16] with the quantities ranging between 0.079% to 100% [0.100% - 100%] with uniform quantization in log domain.

The mapping of measured quantity is defined in Table 4.2.1.1.1-1.

Table 4.2.2.1-1: EXCESS DELAY RATIO measurement report mapping (5 –bit value)

| Reported value | Measured quantity value | Unit |
| --- | --- | --- |
| EXCESS DELAY RATIO_00 | EXCESS DELAY RATIO < 0,079 | % |
| EXCESS DELAY RATIO_01 | 0,079 < EXCESS DELAY RATIO < 0,100 | % |
| EXCESS DELAY RATIO_02 | 0,100 < EXCESS DELAY RATIO < 0,126 | % |
| EXCESS DELAY RATIO_03 | 0,126 < EXCESS DELAY RATIO < 0,158 | % |
| EXCESS DELAY RATIO_04 | 0,158 < EXCESS DELAY RATIO < 0,199 | % |
| EXCESS DELAY RATIO_05 | 0,199 < EXCESS DELAY RATIO < 0,251 | % |
| EXCESS DELAY RATIO_06 | 0,251 < EXCESS DELAY RATIO < 0,316 | % |
| EXCESS DELAY RATIO_07 | 0,316 < EXCESS DELAY RATIO < 0,398 | % |
| EXCESS DELAY RATIO_08 | 0,398 < EXCESS DELAY RATIO < 0,501 | % |
| EXCESS DELAY RATIO_09 | 0,501 < EXCESS DELAY RATIO < 0,631 | % |
| EXCESS DELAY RATIO_10 | 0,631 < EXCESS DELAY RATIO < 0,794 | % |
| EXCESS DELAY RATIO_11 | 0,794 < EXCESS DELAY RATIO < 1,000 | % |
| EXCESS DELAY RATIO_12 | 1,000 < EXCESS DELAY RATIO < 1,259 | % |
| EXCESS DELAY RATIO_13 | 1,259 < EXCESS DELAY RATIO < 1,585 | % |
| EXCESS DELAY RATIO_14 | 1,585 < EXCESS DELAY RATIO < 1,995 | % |
| EXCESS DELAY RATIO_15 | 1,995 < EXCESS DELAY RATIO < 2,511 | % |
| EXCESS DELAY RATIO_16 | 2,511 < EXCESS DELAY RATIO < 3,161 | % |
| EXCESS DELAY RATIO_17 | 3,161 < EXCESS DELAY RATIO < 3,980 | % |
| EXCESS DELAY RATIO_18 | 3,980 < EXCESS DELAY RATIO < 5,011 | % |
| EXCESS DELAY RATIO_19 | 5,011 < EXCESS DELAY RATIO < 6,309 | % |
| EXCESS DELAY RATIO_20 | 6,309 < EXCESS DELAY RATIO < 7,943 | % |
| EXCESS DELAY RATIO_21 | 7,943 < EXCESS DELAY RATIO < 10,00 | % |
| EXCESS DELAY RATIO_22 | 10,00 < EXCESS DELAY RATIO < 12,589 | % |
| EXCESS DELAY RATIO_23 | 12,589 < EXCESS DELAY RATIO < 15,849 | % |
| EXCESS DELAY RATIO_24 | 15,849 < EXCESS DELAY RATIO < 19,953 | % |
| EXCESS DELAY RATIO_25 | 19,953 < EXCESS DELAY RATIO < 25,119 | % |
| EXCESS DELAY RATIO_26 | 25,119 < EXCESS DELAY RATIO < 31,623 | % |
| EXCESS DELAY RATIO_27 | 31,623 < EXCESS DELAY RATIO < 39,811 | % |
| EXCESS DELAY RATIO_28 | 39,811 < EXCESS DELAY RATIO < 50,119 | % |
| EXCESS DELAY RATIO_29 | 50,119 < EXCESS DELAY RATIO < 63,096 | % |
| EXCESS DELAY RATIO_30 | 63,096 < EXCESS DELAY RATIO < 79,433 | % |
| EXCESS DELAY RATIO_31 | 79,433 < EXCESS DELAY RATIO < 100 | % |