3GPP TSG RAN WG2 Meeting #92	R2-156404

Anaheim, USA, Nov 16th - 20th, 2015

Source:	CATT

Title:	Missing Packet due to Half-duplex in PC5

Agenda Item:	7.5.1.1

Document for:	Discussion and Decision

Introduction

A communication capable UE performs sidelink transmission and reception in half-duplex mode, which means UE can’t perform reception and transmission simultaneously on PC5 interface. Missing packet will occur in case of resource collisions in time-domain between the Relay UE uplink/sidelink transmission and Remote UE sidelink transmission.

In this contribution, we discuss the solution for solving the missing packet issue due to half-duplex for UE-to-Network Relay and give our proposals.

Discussion

The transmission and reception of ProSe communication is operated on the same frequency. Due to the physical layer restriction, a UE can’t transmit and receive simultaneously on the same carrier. ProSe UE-to-Network Relay is an important function in Rel-13 D2D WID [1]. Relay UE needs to relay the unicast traffic (UL and DL) between Remote UE and the network. Due to the current half-duplex transmission/reception mode, Relay UE and Remote UE may miss packets, since resource collisions in time-domain may happen between Relay UE uplink/sidelink transmission and Remote UE sidelink transmission.

Proposal 1：Remote UE and Relay UE should avoid transmission in the same subframe.

There are two solutions to avoid missing packets due to collisions in time-domain between Relay UE uplink/sidelink transmission and Remote UE sidelink transmission.

- Option 1: The sidelink transmission resource pool used for out-of-coverage Remote UE is pre-configured. Relay UE shall not use the sub-frames allocated in Remote UE transmission resource pool for both uplink and sidelink transmission. eNB also needs to know the pre-configured Remote UE transmission sub-frames to avoid schedule Relay UE uplink/sidelink transmission in that sub-frames. This is a simple solution with low resource efficiency and less flexibility.

- Option 2: Relay UE informs the resource allocation information to Remote UE.

Since Relay UE is an in coverage UE, its resources for both uplink and Tx sidelink are assigned by eNB. To avoid resource collision in time-domain between Relay UE transmission and reception, eNB shall avoid scheduling the Relay UE for both uplink and Tx sidelink in the subframes on which Remote UE transmits sidelink data. The resources coordination amongst Remote UE, Relay UE and eNB is required. One simple way is that eNB configures the sidelink transmission resource pool to Remote UEs via the Relay UE. eNB can configure different resource pool for the Remote UEs and Relay UE, which can avoid the collisions in time-domain between Relay UE uplink/sidelink transmission and Remote UE sidelink transmission.

Proposal 2：eNB configures the relay UE with the communication transmission resource pool for out-of-coverage Remote UE.

Proposal 3：Relay UE informs the remote UE communication transmission resource pool to its served Relay UEs.

There are several options to inform the communication transmission resources pool by Relay UE:

- Option 1: the transmission resource information is carried by PSBCH

Extend MIB-SL to include transmission resource pool indexes of Relay UE or Remote UE.

- Option 2: the transmission resource information is carried by PSDCH

Relay UE’s AS layer forwards the transmission resource information to higher layer, then it can be carried in discovery message.

Table 1: comparison of transmission resource information methods

|  | Resource efficiency | Flexibility | Inter-monitoring performance | Specification effort | Whether impact normal transmission |
| --- | --- | --- | --- | --- | --- |
| Option 1 | FFS | Low, the configuration granularity is pre-configured resource pool | High | Low | May impact the reliability of synchronization |
| Option 2 | High, need 1-4 sub-frames for once transmission. Upper layer needs to generate the message periodically. | High | Middle | Low | Share the bits with relay discovery message, |

Inter-monitoring performance is used to evaluate the inter-monitor proportion between Tx and Rx UE. It will improve by increasing resource in time-domain. For communication, the resource in time-domain is divided by PSCCH and PSSCH. Inter-monitoring is not realized for the case either PSCCH or PSSCH lost. For discovery, there are more off pattern can be used to monitor. Hence the inter-monitoring performance of PSDCH is higher than PSCCH + PSSCH.

Considering resource efficiency, flexibility, inter-monitoring performance, specification impact and whether impacts normal transmission, we prefer option 2.

Proposal 4: Relay UE can transmit the resource pool information through PSDCH channel.

Proposal 5: Relay UE’s AS layer forwards the transmission resource information to higher layer, and then it can be carried in the discovery message.

Conclusion

According to the analysis in section 2, it is proposed:

Proposal 1：Remote UE and Relay UE should avoid transmission in the same sub-frame.

Proposal 2：eNB configures the relay UE with the communication transmission resource pool for out-of-coverage Remote UE.

Proposal 3：Relay UE informs the remote UE communication transmission resource pool to its served Relay UEs.

Proposal 4: Relay UE can transmit the resource pool information through PSDCH channel.

Proposal 5: Relay UE’s AS layer forwards the transmission resource information to higher layer, and then it can be carried in the discovery message.

References

  - RP-142311 Work Item Proposal for Enhanced LTE Device to Device Proximity Services, Qualcomm Incorporated, RAN#66