3GPP TSG RAN2 #92		R2-156551

Anaheim, US, Nov. 16 – Nov. 20, 2015

Source:	Samsung Electronics

Title:	Discussion on Paging in NB-IoT

Document for:	Discussion

Agenda item:	7.16.2.4

Introduction

At RAN #69, New Work Item called by NB-IoT was agreed to develop a single “clean-slate solution” in TSG RAN based on techniques described in TR 45.820 [1]. The objectives of the CIoT study described in TR 45.820 were improved indoor coverage, support of massive connections, reduced complexity, improved power efficiency and latency. [2]. In addition, as already discussed in WID [1], the main design concept in high layer of NB-IoT is to maximize the reuse of current LTE procedure to NB-IoT.

This contribution discusses how to adapt the current LTE paging procedures into NB-IoT with considering both the minimal change of specification and improvement of the power efficiency.

Revisit of Current Paging Operation

In this section, we first revisit the current paging operation in LTE to provide the background for discussing how it can be modified for use in NB-IoT systems.

2.1 Determination of PF (Paging Frame) and PO (Paging Occasion)

From [3], the PF and PO of current LTE paging operation are determined by following formulae using the DRX parameters provided in SI (System Information):

- PF is given by following equation:

SFN mod T= (T div N)*(UE_ID mod N)

- Index i_s pointing to PO from subframe pattern defined in Table 1 will be derived from following calculation:

i_s = floor(UE_ID/N) mod Ns

where the following parameters are used for the calculation of the PF and i_s:

- SFN: System Frame Number.

- T: DRX cycle of the UE. T is determined by the shortest of the UE specific DRX value, if allocated by upper layers, and a default DRX value broadcast in system information. If UE specific DRX is not configured by upper layers, the default value is applied.

- nB: 4T, 2T, T, T/2, T/4, T/8, T/16, T/32.

- N: min(T,nB)

- Ns: max(1,nB/T)

- UE_ID: IMSI mod 1024.

Table 1. Subframe Pattern for pointing PO according to Ns and i_s

| Ns | PO when i_s=0 | PO when i_s=1 | PO when i_s=2 | PO when i_s=3 |
| --- | --- | --- | --- | --- |
| 1 | 9 | N/A | N/A | N/A |
| 2 | 4 | 9 | N/A | N/A |
| 4 | 0 | 4 | 5 | 9 |

Here, nB means the number of POs to be transmitted in a frame, and is reflected in the resulting values of N for PF, and of Ns for PO i.e. the value nB influences the number of paging messages to be conveyed in a cell during a day. In addition, when considering the current eDRX discussion, the range of nB needs to be reconsidered, e.g. more sparse PO distribution over PF, such as T/64, T/128, … as nB.

Observation 1. The value of nB needs to be determined considering the number of required paging messages in NB-IoT and the current eDRX discussion.

2.2 The amount of Network Commands for Paging

Paging is triggered by network commands, whose model will be referred to as Network Triggered Reporting (NTR). When it comes to the amount of traffic, CIoT has much smaller volume than LTE. In terms of traffic pattern, TR 45.820 models the NTR with inter-arrival time distributed between 2 hours to 1 day, meaning that CIoT devices transmit UL data at most 12 times per a day for NTR traffic. According to simulation, CIoT devices receive an average of just 2.25 paging messages per day for NTR traffic, which is smaller compared to paging messages of normal LTE devices. Table 3 shows the traffic model of NTR mode used for paging operation.

Table 3. Traffic mode of NTR mode [2]

| Response Probability about the Network Command | response of 50%, not response of 50% |
| --- | --- |
| The size of the downlink Network Command | 20 bytes |
| The distribution of the periodic inter-arrival time | Split of inter-arrival time periodicity: 1 day (40%), 2 hours (40%), 1 hour (15%), and 30 minutes (5%) |
| The distribution of the application payload size | Pareto distribution with shape parameter alpha = 2.5 and minimum application payload size = 20 bytes with a cut off of 200 bytes i.e. payloads higher than 200 bytes are assumed to be 200 bytes. |

Therefore, following proposal is derived:

Proposal 1. Compared to LTE, much fewer paging messages generated from the network are expected in NB-IoT, e.g., 2.25 paging messages per day. Therefore, when reusing the paging mechanism of LTE for NB-IoT, we need to figure out the impacts of the reduced paging demands.

2.3 Transmission of Paging PDCCH and Paging Message

In order to examine the impacts of the reduced paging demands, the procedure for LTE paging operation is revisited from waking up to receiving paging message.

Figure 1. Procedure of LTE paging operation

After checking that S-Criteria is satisfied in step 1, UE confirms whether the location is updated or not by reading MIB and SIBs of steps 2 and 3. If the location is not updated, UE awakened in arbitrary Paging Occasion (PO) will decode PDCCH scrambled with P-RNTI in step 4. Then, UE proceeds to decode the PDSCH message including MS ID list (e.g. S-TMSI).

Here, aside from obtaining the synch and system information, LTE UEs need to perform two separate steps in order to receive paging message: receiving PDCCH and then PDSCH. This is the right approach for high-end LTE network where there are lots of uplink and downlink traffic arriving: in other words, when there are a large number of paged UEs in one PO, it is more efficient to have paged UE list included in data channel. In particular, when considering the size of S-TMSI is 40bits, including more than two UE IDs means that size of paging message should be more than 80bits, which means that inserting the paged UE IDs to PDCCH is quite challenging.

Observation 2. The paging mechanism of current LTE consists of two steps: receiving PDCCH and then PDSCH. The list of paged UE IDs is included in the data channel, PDSCH, because there are a large number of paged devices in LTE.

Therefore, we can know that the current paging mechanism is well suited to LTE with a very large amount of traffic, which in turn means that a large number of paged devices are included in same PO group. However, if the current LTE paging procedure is employed for NB-IoT, it is expected that a lot of devices in one PO group would wake up and find that they have no paging message for them: they have been woken up only because paging messages arrived for a small number of devices in the same PO group. Nonetheless, the not-paged devices have had to read both PDCCH and PDSCH in this procedure. We call this phenomenon “False Paging” in this contribution, captured in the following observation.

Observation 3. If the current LTE paging procedure is employed for NB-IoT, lots of non-paged devices will wake up due to the paging messages arriving for other devices included in same PO group.

Table 4 shows the number of woken and paged UEs in each PO according to DRX. For the analysis, following setups are assumed.

- The number of UEs in a cell : 52547

- The system frame duration : 100ms

- Observed Coverage Level : 144dB

- The number of UEs in the Observed Coverage Class : 17516

- Traffic distribution for paging demand : followed by Table 3

- nB = T is assumed in Table 2

Table 4. The number of woken and paged UEs in each PO according to DRX

|  | DRX |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 2.56 sec | 10.24 sec | 20.48 sec | 81.92 sec (≈ 1.3 min) | 163.84 sec (≈ 2.7 min) | 327.68 sec (≈5.3 min) |
| The number of woken UEs (per PO), (a) | 547.4 | 136.8 | 68.4 | 17.1 | 8.6 | 4.3 |
| The number of paged UEs (per PO), (b) | 0.037 | 0.037 | 0.037 | 0.037 | 0.037 | 0.037 |
| Paging Efficiency, (b)/(a) | ≈ 0.0 | ≈ 0.0 | ≈ 0.0 (0.0005) | ≈ 0.0 (0.002) | ≈ 0.0 (0.004) | ≈ 0.0 (0.008) |

As shown in Table 4, we can know that paging efficiency is almost zero in the NB-IoT environment. The Main reason behind this is that a few minutes DRX period is too short considering paging demands, so the number of paged UEs in each PO is almost zero. This implies that most of the woken UEs in each PO have been falsely paged and would consume their power unnecessarily. On the other hand, it is also observed that the number of woken UEs decrease as the DRX value increases, i.e. by increasing the DRX value, the number of false paging UEs can decrease.

Observation 4. When current 2 step paging mechanism with DRX period up to 10.24s is applied to NB-IoT, the paging efficiency is very low. Therefore, it can cause unnecessary battery power consumption.

Proposal 2. For NB-IoT environment, employing eDRX is recommended for relaxing the false paging.

Way Forward of Current Paging for NB-IoT

A few options can be considered as a way of reducing the false paging in NB-IoT environment.

(1) The first option is to extend the DRX cycle as much as required.

In that case, the number of paged UEs in the PO will increase so that the ratio of false paging will decrease. Currently, eDRX WI is considering the period of 43min as the maximum value. However, the required response time for the network command depends on the type of applications. One might consider the required response time into a few seconds – 1 min for the real-time application and into 1min – 5min for the near real-time application [4]. Even though considering those kinds of loose response time, we can still observe the quite many false paging UEs as shown in Table 4. Therefore, we can say that eDRX may not be a complete solution for the false paging problem.

Observation 5. We need to consider another solution aside from the eDRX scheme, because eDRX may not be applicable to all CIoT applications depending on the latency requirement.

(2) The second option is to simplify the current paging procedure which consists of PDCCH and PDSCH into a single step paging, e.g., PDCCH with paged UE IDs or PDSCH with no or reduced PDCCH transmission.

The devices with relatively short latency requirements will suffer from false paging. Whenever paging PDCCH is detected, the devices need to decode another channel, PDSCH, to check paging arrival. Instead of using both PDCCH and PDSCH, one single step paging using only PDCCH can be considered for the devices. This may work mostly well in CIoT environment where the number of paged devices is limited.

In addition, we can omit or reduce the regular PDCCH transmissions for paging as already introduced in SPS (Semi-Persistent Scheduling). By using these two methods, we can solve the false paging problem with few specification changes.

Proposal 3. For the false paging issue, we can consider simplifying the current paging procedure, e.g. a single PDCCH based paging message which includes paged UE ID or reducing the regular PDCCH transmissions for paging as already introduced in SPS.

(3) The third option is to insert more information about paging to some channels ahead of PDSCH paging message so that the false paging problem can be relaxed somewhat.

For example, one possible option is to insert the paging group information (e.g. n Least Significant Bits of IMSI or S-TMSI) to MIB (Master Information Block) as introduced in NB-CIoT or to PDCCH as introduced in NB-M2M.

However, inserting paging control information in the MIB may not be appropriate: the fundamental purpose of the MIB is not to control devices dynamically, but is to broadcast system information with long update period. In addition, inserting paging control information in the MIB can cause unnecessary power consumption to non-NTR UEs, which means that MAR UE should decode the unnecessary paging related information in the MIB.

On the other hand, it has its own benefit. Because the MIB is the first channel for the woken UEs to read except the synchronization signal, inserting the paging control information in the MIB can make the woken UEs back to sleep fast without decoding PDCCH or PDSCH. Therefore, considering the pros and cons, we can think some compromise, which is 1 bit indication.

In fact, when considering the main requirement for the response time into 1min - 5 min (near real-time service [4]), the number of woken UEs is around 10 as shown in table 4. Therefore, we think that 1 bit paging indication in MIB is enough for handling around 10 UEs without giving serious power consumption impact to the other MAR UEs.

Proposal 4. Inserting 1 bit paging indication in MI can be beneficial to NB-IoT UEs, because it can make the woken UEs back to sleep fast without giving serious power consumption impact to the other MAR UEs.

Conclusions

In this document, we discussed issues on paging design in NB-IoT. For the paging design, we derived following observations and proposals.

Observation 1. The value of nB needs to be determined considering the number of required paging messages in NB-IoT and the current eDRX discussion.

Observation 2. The paging mechanism of current LTE consists of two steps: receiving PDCCH and then PDSCH. The list of paged UE IDs is included in the data channel, PDSCH, because there are a large number of paged devices in LTE.

Observation 3. If the current LTE paging procedure is employed for NB-IoT, lots of non-paged devices will wake up due to the paging messages arriving for other devices included in same PO group.

Observation 4. When current 2 step paging mechanism with DRX period up to 10.24s is applied to NB-IoT, the paging efficiency is very low. Therefore, it can cause unnecessary battery power consumption.

Observation 5. We need to consider another solution aside from the eDRX scheme, because eDRX may not be applicable to all CIoT applications depending on the latency requirement.

Proposal 1. Compared to LTE, much fewer paging messages generated from the network are expected in NB-IoT, e.g., 2.25 paging messages per day. Therefore, when reusing the paging mechanism of LTE for NB-IoT, we need to figure out the impacts of the reduced paging demands.

Proposal 2. For NB-IoT environment, employing eDRX is recommended for relaxing the false paging.

Proposal 3. For the false paging issue, we can consider simplifying the current paging procedure, e.g. a single PDCCH based paging message which includes paged UE ID or reducing the regular PDCCH transmissions for paging as already introduced in SPS.

Proposal 4. Inserting 1 bit paging indication in MI can be beneficial to NB-IoT UEs, because it can make the woken UEs back to sleep fast without giving serious power consumption impact to the other MAR UEs.

References

[1] RP-151621, New Work Item: NarrowBand IOT (NB-IoT), Qualcomm Incorporated, RAN#69, Phoenix, USA, Sep 2015.

[2] 3GPP TR 45.820 v2.0.0, Cellular System Support for Ultra Low Complexity and Low Throughput Internet of Things, August 2015.

[3] 3GPP TS 36.304, 3GPP TSG RAN E-UTRA UE procedures in idle mode

[4] Dave Hardin, "Smart Grid and Dynamic Power Management", www.intechopen.com/download/pdf/17061