3GPP TSG-RAN WG2 Meeting #92	R2-156903

Anaheim, USA, November 16 – November 20, 2015

Agenda item:	7.6.2.2 (LTE_WLAN_radio-Core)

Source:	LG Electronics Inc.

Title: 	Introduce a LWA specification

Document for:	Discussion and Decision

1.	Introduction

One of the main issues in LWA is where the adaptaion layer should be specified. In this document, we show how the new specification look like if RAN2 agreed to have a new specification for adaptaion layer.

2.	Proposal

Make a new specification for the LWA protocol based on the draft text attached below.

3rd Generation Partnership Project;

Technical Specification Group Radio Access Network;

Evolved Universal Terrestrial Radio Access (E-UTRA);

LTE-WLAN Adaptation (LWA) protocol specification

(Release 13)

3GPP TS 36.3xx V0.0.0 (2015-11)

Technical Specification

The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP.

The present document has not been subject to any approval process by the 3GPP Organisational Partners and shall not be implemented.

This Specification is provided for future development work within 3GPP only. The Organisational Partners accept no liability for any use of this Specification.
Specifications and reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organisational Partners' Publications Offices.

Keywords

UMTS, radio

3GPP

Postal address

3GPP support office address

650 Route des Lucioles - Sophia Antipolis

Valbonne - FRANCE

Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16

Internet

http://www.3gpp.org

Copyright Notification

No part may be reproduced except as authorized by written permission.
The copyright and the foregoing restriction extend to reproduction in all media.

© 2015, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

UMTS™ is a Trade Mark of ETSI registered for the benefit of its members

3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners

LTE™ is a Trade Mark of ETSI currently being registered for the benefit of its Members and of the 3GPP Organizational Partners

GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	4

1	Scope	5

2	References	5

3	Definitions, symbols and abbreviations	5

3.1	Definitions	5

3.2	Abbreviations	5

4	General	6

4.1	Introduction	6

4.2	LWA architecture	6

4.2.1	LWA entities	6

4.3	Services	7

4.3.1	Services provided to upper layers	7

4.3.2	Services expected from lower layers	7

4.4	Functions	7

5	Procedures	7

5.1	Data transfer procedures	7

5.2	Handling of unknown, unforeseen and erroneous protocol data	7

6	Protocol data units, formats and parameters	7

6.1	Protocol data units	7

6.1.1	General	7

6.1.2	LWA data PDU	8

6.2	Formats and parameters	8

6.2.1	LWA header	8

Annex A (informative): Change history	9

Foreword

This Technical Specification has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

1	Scope

The present document specifies the E-UTRA LTE-WLAN Adaptation (LWA) protocol.

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

- References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

- For a specific reference, subsequent revisions do not apply.

- For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 36.300: "E-UTRA and E-UTRAN Overall Description; Stage 2".

[3]	3GPP TS 36.321: "E-UTRA MAC protocol specification".

[4]	3GPP TS 36.323: "E-UTRA PDCP specification".

[5]	3GPP TS 36.331: "E-UTRA RRC Protocol specification".

3	Definitions, symbols and abbreviations

3.1	Definitions

For the purposes of the present document, the terms and definitions given in TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [1].

LWA bearer: in LTE-WLAN Aggregation, a bearer whose radio protocols are located in both the eNB and the WLAN to use both eNB and WLAN resources.

3.2	Abbreviations

For the purposes of the present document, the following abbreviations apply:

DL	DownLink

DRB	Data Radio Bearer

eNB	E-UTRAN Node B

E-UTRA	Evolved UMTS Terrestrial Radio Access

E-UTRAN	Evolved UMTS Terrestrial Radio Access Network

LWA	LTE-WLAN Adaptation

MAC	Medium Access Control

PDCP	Packet Data Convergence Protocol

PDU	Protocol Data Unit

RRC	Radio Resource Control

SAP	Service Access Point

SDU	Service Data Unit

UE	User Equipment

UL	UpLink

4	General

4.1	Introduction

The objective is to describe the LWA architecture and the LWA entities from a functional point of view.

4.2	LWA architecture

4.2.1	LWA entities

The description in this sub clause is a model and does not specify or restrict implementations.

RRC is generally in control of the LWA configuration.

Functions of the LWA sub layer are performed by LWA entities. For a LWA entity configured at the eNB, there is a peer LWA entity configured at the UE and vice versa. For all LWA bearers, there is one LWA entity in the eNB and one LWA entity in the UE.

An LWA entity receives/delivers LWA SDUs from/to upper layer (i.e. PDCP) and sends/receives LWA PDUs to/from its peer LWA entity via WLAN. In the eNB side, an LWA entity receives LWA SDUs from multiple upper layer entities through corresponding SAPs, and after forming LWA PDUs from the received LWA SDUs, the LWA entity delivers the LWA PDUs to a lower layer entity through a single SAP. In the UE side, an LWA entity receives LWA PDUs from a lower layer entity through a single SAP, and after forming LWA SDUs from the received LWA PDUs, the LWA entity delivers the LWA SDUs to upper layer entities identified by the LWA headers through a corresponding SAP.

Figure 4.2.1-1 illustrates the overview model of the LWA sub layer.

Figure 4.2.1-1: Overview model of the LWA sub layer

An LWA entity delivers/receives the following LWA PDU to/from a lower layer entity:

-	LWA data PDU.

4.3	Services

4.3.1	Services provided to upper layers

The following services are provided by LWA to upper layer (i.e. PDCP):

-	LWA data transfer;

4.3.2	Services expected from lower layers

The following services are expected by LWA from lower layer (i.e. WLAN):

-	data transfer;

4.4	Functions

The following functions are supported by the LWA sub layer:

-	transfer of upper layer PDUs;

-	identification of upper layer entity to which the LWA SDU is destined.

5	Procedures

5.1	Data transfer procedures

When receiving an LWA data PDU from lower layer, the LWA entity shall:

-	identify the upper layer entity to which the LWA SDU is destined based on the DRB ID included in the LWA header;

-	reassemble the LWA SDU from the LWA data PDU by removing the LWA header from the LWA data PDU;

-	deliver the reassembled LWA SDU to the upper layer entity identified by the DRB ID.

5.2	Handling of unknown, unforeseen and erroneous protocol data

When an LWA entity receives an LWA PDU that contains reserved or invalid values, the LWA entity shall:

-	discard the received PDU.

6	Protocol data units, formats and parameters

6.1	Protocol data units

6.1.1	General

An LWA PDU is a bit string that is byte aligned (i.e. multiple of 8 bits) in length. In the figures in subclause 6.1, bit strings are represented by tables in which the most significant bit is the leftmost bit of the first line of the table, the least significant bit is the rightmost bit on the last line of the table, and more generally the bit string is to be read from left to right and then in the reading order of the lines. The bit order of each parameter field within an LWA PDU is represented with the first and most significant bit in the leftmost bit and the last and least significant bit in the rightmost bit.

An LWA SDU is a bit string that is byte aligned (i.e. multiple of 8 bits) in length. An LWA SDU is included into an LWA PDU from the first bit onward.

The LWA entity shall ignore the value of Reserved bits in downlink LWA PDUs.

Only one type of LWA PDU is defined, i.e. LWA data PDU.

6.1.2	LWA data PDU

An LWA PDU consists of an LWA header, as described in Figure 6.1.2-1.

An LWA header is one byte, and consists of the four header fields R/R/R/DRBID.

A Data field is of variable size, and consists of an LWA SDU.

Figure 6.1.2-1: LWA data PDU

6.2	Formats and parameters

6.2.1	LWA header

The LWA header consists of the following fields:

-	R: Reserved bit, set to "0". The R field size is 1 bit;

-	DRBID: The Data Radio Bearer ID field indicates the ID of the DRB to which the corresponding LWA SDU belongs. The DRBID field size is 5 bits.

Annex A (informative):
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | TSG # | TSG Doc. | CR | Rev | Subject/Comment | Old | New |