3GPP TSG-RAN WG2 Meeting #92 	R2-157144

# Anaheim, U.S.A, 16-20 November 2015

| CR-Form-v11.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.304 | CR | 0281 | rev | 3 | Current version: | 12.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introduction of load redistribution in RRC_IDLE |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE,China Telecom |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_MC_load-Core |  |  |  |  |  |  | Date: |  | 2015-11-05 |
| Category: | B |  |  |  |  |  |  | Release: |  | Rel-13 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) |  |
| Reason for change: |  | Existing cell reselection mechanism may result in unbalanced UE distribution among carriers. A new scheme is needed to redistribute a fraction of UEs among carriers and/or among cells under network control. |  |  |  |  |  |  |  |  |
| Summary of change: |  | For an E-UTRAN frequency or cell configured with redistribution factor, the UE shall perform measurements according to the redistribution frequency/cell provided in SIB3/SIB5; and the UE selects its target redistribution frequency/cell by comparing IMSI mod 100 value with the redistribution range derived from the redistribution factor and considers it as the highest priority and start T360 timer. The priority handling for the redistribution shall be stopped when T360 expires or the UE reselects to a cell not belonging to the target redistribution frequency/cell. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | This feature can’t be supported. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Section 5.2.4.1, 5.2.4.2, 5.2.4.7,5.2.4.z |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | y |  | Other core specifications |  |  | TS36.331... CR 1955 ... |  |  |  |
| affected: |  |  |  | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

5.2.4		Cell Reselection evaluation process

5.2.4.1	Reselection priorities handling

Absolute priorities of different E-UTRAN frequencies or inter-RAT frequencies may be provided to the UE in the system information, in the RRCConnectionRelease message, or by inheriting from another RAT at inter-RAT cell (re)selection. In the case of system information, an E-UTRAN frequency or inter-RAT frequency may be listed without providing a priority (i.e. the field cellReselectionPriority is absent for that frequency). If priorities are provided in dedicated signalling, the UE shall ignore all the priorities provided in system information. If UE is in camped on any cell state, UE shall only apply the priorities provided by system information from current cell, and the UE preserves priorities provided by dedicated signalling and deprioritisationReq received in RRCConnectionReject unless specified otherwise. When the UE in camped normally state, has only dedicated priorities other than for the current frequency, the UE shall consider the current frequency to be the lowest priority frequency (i.e. lower than the eight network configured values). While the UE is camped on a suitable CSG cell, the UE shall always consider the current frequency to be the highest priority frequency (i.e. higher than the eight network configured values), irrespective of any other priority value allocated to this frequency. If the UE capable of sidelink communication is configured to perform sidelink communication and can only perform the sidelink communication while camping on a frequency, the UE may consider that frequency to be the highest priority.

NOTE:	The prioritization among the frequencies which UE considers to be the highest priority frequency is left to UE implementation.

If the UE is capable of MBMS Service Continuity and receiving or interested to receive an MBMS service and can only receive this MBMS service while camping on a frequency on which it is provided, the UE may consider that frequency to be the highest priority during the MBMS session [2] as long as the reselected cell is broadcasting SIB13 and as long as:

-	SIB15 of the serving cell indicates for that frequency one or more MBMS SAIs included in the MBMS User Service Description (USD) [22] of this service; or

-	SIB15 is not broadcast in the serving cell and that frequency is included in the USD of this service.

If the UE is capable of MBMS Service Continuity and receiving or interested to receive an MBMS service provided on a downlink only MBMS frequency, the UE may consider cell reselection candidate frequencies at which it can not receive the MBMS service to be of the lowest priority during the MBMS session [2], as long as the cell on the MBMS frequency which the UE monitors is broadcasting SIB13 and as long as the above mentioned SIB15 condition is fulfilled for the serving cell.

NOTE:	An example scenario in which the previous down-prioritisation may be needed concerns the case the MBMS frequency is a downlink only carrier on which camping is not possible, while the UE can only receive this MBMS frequency when camping on a subset of cell reselection candidate frequencies.

If the UE is not capable of MBMS Service Continuity but has knowledge on which frequency an MBMS service of interest is provided, it may consider that frequency to be the highest priority during the MBMS session [2] as long as the reselected cell is broadcasting SIB13.

If the UE is not capable of MBMS Service Continuity but has knowledge on which downlink only frequency an MBMS service of interest is provided, it may consider cell reselection candidate frequencies at which it can not receive the MBMS service to be of the lowest priority during the MBMS session [2] as long as the cell on the MBMS frequency which the UE monitors is broadcasting SIB13.

NOTE:	The UE considers that the MBMS session is ongoing using the session start and end times as provided by upper layers in the USD i.e. the UE does not verify if the session is indicated on MCCH.

In case UE receives RRCConnectionReject with deprioritisationReq, UE shall consider current carrier frequency and stored frequencies due to the previously received RRCConnectionReject with deprioritisationReq or all the frequencies of EUTRA to be the lowest priority frequency (i.e. lower than the eight network configured values) while T325 is running irrespective of camped RAT. The UE shall delete the stored deprioritisation request(s) when a PLMN selection is performed on request by NAS [5].

NOTE:	Connecting to CDMA2000 does not imply PLMN selection.

NOTE:	UE should search for a higher priority layer for cell reselection as soon as possible after the change of priority. The minimum related performance requirements specified in TS 36.133 [10] are still applicable.

The UE shall delete priorities provided by dedicated signalling when:

- the UE enters RRC_CONNECTED state; or

- the optional validity time of dedicated priorities (T320) expires; or

- a PLMN selection is performed on request by NAS [5].

NOTE:	Equal priorities between RATs are not supported.

The UE shall only perform cell reselection evaluation for E-UTRAN frequencies and inter-RAT frequencies that are given in system information and for which the UE has a priority provided.

The UE shall not consider any black listed cells as candidate for cell reselection.

The UE shall inherit the priorities provided by dedicated signalling and the remaining validity time (i.e., T320 in E-UTRA, T322 in UTRA and T3230 in GERAN), if configured, at inter-RAT cell (re)selection.

NOTE:	The network may assign dedicated cell reselection priorities for frequencies not configured by system information.

While T360 is running, redistribution target is considered to be the highest priority (i.e. higher than the any network configured values).

5.2.4.2	Measurement rules for cell re-selection

When evaluating Srxlev and Squal of non-serving cells for reselection purposes, the UE shall use parameters provided by the serving cell.

Following rules are used by the UE to limit needed measurements:

-	If the serving cell fulfils Srxlev > SIntraSearchP and Squal > SIntraSearchQ, the UE may choose not to perform intra-frequency measurements.

-	Otherwise, the UE shall perform intra-frequency measurements.

-	The UE shall apply the following rules for E-UTRAN inter-frequencies and inter-RAT frequencies which are indicated in system information and for which the UE has priority provided as defined in 5.2.4.1:

-	For an E-UTRAN inter-frequency or inter-RAT frequency with a reselection priority higher than the reselection priority of the current E-UTRA frequency the UE shall perform measurements of higher priority E-UTRAN inter-frequency or inter-RAT frequencies according to [10].

-	For an E-UTRAN inter-frequency with an equal or lower reselection priority than the reselection priority of the current E-UTRA frequency and for inter-RAT frequency with lower reselection priority than the reselection priority of the current E-UTRAN frequency:

-	If the serving cell fulfils Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ, the UE may choose not to perform measurements of E-UTRAN inter-frequencies or inter-RAT frequency cells of equal or lower priority  unless the UE is triggered to measure an E-UTRAN inter-frequency which is configured with redistributionInterFreqInfo.

-	Otherwise, the UE shall perform measurements of E-UTRAN inter-frequencies or inter-RAT frequency cells of equal or lower priority according to [10].

Next Modified Subclause

5.2.4.7		Cell reselection parameters in system information broadcasts

Cell reselection parameters are broadcast in system information and are read from the serving cell as follows:

cellReselectionPriority

This specifies the absolute priority for E-UTRAN frequeny or UTRAN frequency or group of GERAN frequencies or band class of CDMA2000 HRPD or band class of CDMA2000 1xRTT.

Qoffsets,n

This specifies the offset between the two cells.

Qoffsetfrequency

Frequency specific offset for equal priority E-UTRAN frequencies.

Qoffsettemp

This specifies the additional offset to be used for cell selection and re-selection. It is temporarily used in case the RRC Connection Establishment fails on the cell as specified in [3].

Qhyst

This specifies the hysteresis value for ranking criteria.

Qqualmin

This specifies the minimum required quality level in the cell in dB.

Qrxlevmin

This specifies the minimum required Rx level in the cell in dBm.

TreselectionRAT

This specifies the cell reselection timer value. For each target E-UTRA frequency and for each RAT (other than E-UTRA) a specific value for the cell reselection timer is defined, which is applicable when evaluating reselection within E-UTRAN or towards other RAT (i.e. TreselectionRAT for E-UTRAN is TreselectionEUTRA, for UTRAN TreselectionUTRA for GERAN TreselectionGERA, for TreselectionCDMA_HRPD, and for TreselectionCDMA_1xRTT).

Note: 	TreselectionRAT is not sent on system information, but used in reselection rules by the UE for each RAT.

RedistributionFactorFreq

This specifies the redistribution factor for a neighbour E-UTRAN frequency.

RedistributionFactorCell

This specifies the redistribution factor for a neighbour E-UTRAN cell.

RedistributionFactorServing

This specifies the redistribution factor for serving cell or serving frequency.

TreselectionEUTRA

This specifies the cell reselection timer value TreselectionRAT for E-UTRAN. The parameter can be set per E-UTRAN frequency [3].

TreselectionUTRA

This specifies the cell reselection timer value TreselectionRAT for UTRAN.

TreselectionGERA

This specifies the cell reselection timer value TreselectionRAT for GERAN.

TreselectionCDMA_HRPD

This specifies the cell reselection timer value TreselectionRAT for CDMA HRPD.

TreselectionCDMA_1xRTT

This specifies the cell reselection timer value TreselectionRAT for CDMA 1xRTT.

ThreshX, HighP

This specifies the Srxlev threshold (in dB) used by the UE when reselecting towards a higher priority RAT/ frequency than the current serving frequency. Each frequency of E-UTRAN and UTRAN, each group of GERAN frequencies, each band class of CDMA2000 HRPD and CDMA2000 1xRTT might have a specific threshold.

ThreshX, HighQ

This specifies the Squal threshold (in dB) used by the UE when reselecting towards a higher priority RAT/ frequency than the current serving frequency. Each frequency of E-UTRAN and UTRAN FDD might have a specific threshold.

ThreshX, LowP

This specifies the Srxlev threshold (in dB) used by the UE when reselecting towards a lower priority RAT/ frequency than the current serving frequency. Each frequency of E-UTRAN and UTRAN, each group of GERAN frequencies, each band class of CDMA2000 HRPD and CDMA2000 1xRTT might have a specific threshold.

ThreshX, LowQ

This specifies the Squal threshold (in dB) used by the UE when reselecting towards a lower priority RAT/ frequency than the current serving frequency. Each frequency of E-UTRAN and UTRAN FDD might have a specific threshold.

ThreshServing, LowP

This specifies the Srxlev threshold (in dB) used by the UE on the serving cell when reselecting towards a lower priority RAT/ frequency.

ThreshServing, LowQ

This specifies the Squal threshold (in dB) used by the UE on the serving cell when reselecting towards a lower priority RAT/ frequency.

SIntraSearchP

This specifies the Srxlev threshold (in dB) for intra-frequency measurements.

SIntraSearchQ

This specifies the Squal threshold (in dB) for intra-frequency measurements.

SnonIntraSearchP

This specifies the Srxlev threshold (in dB) for E-UTRAN inter-frequency and inter-RAT measurements.

SnonIntraSearchQ

This specifies the Squal threshold (in dB) for E-UTRAN inter-frequency and inter-RAT measurements.

Next Modified Subclause

5.2.4.z 	E-UTRAN Inter-frequency Redistribution procedure

If a UE is redistribution capable and redistributionServingInfo is included in SystemInformationBlockType3 and redistributionInterFreqInfo is included in SystemInformationBlockType5 and the UE is not configured with dedicated priorities and

-	if T360 is not running and if redistrOnPagingOnly is not present in SystemInformationBlockType3: or

-	if T360 expires: or

-	if Paging message is received and the redistributionIndication is included:

-	Perform inter-frequency measurement as specified in 5.2.4.2;

-	Once measurement results are available perform redistribution target selection as specified in 5.2.4.z.1;

-	Start T360;

The UE shall stop T360 and cease to consider a frequency or cell to be redistribution target when:

- the UE enters RRC_CONNECTED state; or

- T360 expires; or

- if Paging message is received and the redistributionIndication is included while T360 is running; or

- the UE reselects a cell not belonging to redistribution target.

5.2.4.z.1	Redistribution target selection

The UE shall compile a sorted list of valid redistFactor[j] in which entries are added in increasing index order starting with index 0 as follows:

-	the value of the redistributionFactorServing from SystemInformationBlockType3;

- 	for each entry in InterFreqCarrierFreqList and subsequent for each entry in InterFreqCarrierFreqListExt:

-	if present in RedistributionInterFreqInfo:  the value of the entry in redistributionNeighCellList corresponding to the cell ranked as the best cell among the cells on this frequency meeting the criteria according to section 5.2.4.6;

-		else the value of the entry in redistributionFactorFreq corresponding to the frequency if at least one detected cell on the frequency fullfills the cell selection criterion S defined in 5.2.3.2;

The UE shall choose a redistribution target as follows:

- If UE_ID≤100*redistrRange[0], the UE shall choose the frequency or the cell corresponding to redistrFactor[0] as its redistribution target or;

- If 100*j=0j=i-1redistrRangej<UE_ID≤100*j=0j=iredistrRange[j], then the UE shall choose the frequency or cell corresponding to redistrFactor[i] as its redistribution target;

- UE_ID=IMSI mod 100;

The redistrRangei of E-UTRAN frequency or cell is defined by:

redistrRangei=redistrFactori/j=0j=(maxCandidates-1)redistrFactor[j]

Where: maxCandidates is the total number of frequencies/cells with valid redistFactor[j].