3GPP TSG RAN WG2 Meeting #93bis															R2-162185

# Dubrovnik, Croatia, 11th  – 15th  April 2016

Source:	CATT

Title:	Text Proposal for LWAAP to Support of UL Bearer in Rel-14 LWA

Agenda Item:	8.5.1

Document for:	Decision

In accordance with the discussions and proposals in R2-162183, this paper provides the text proposals for TS36.360 LWAAP specification to support the UL spilt/switch bearer in Rel-14 LWA.

Begin of Text Proposal

4.2.1	LWAAP entities

The description in this sub clause is a model and does not specify or restrict implementations.

RRC is generally in control of the LWAAP configuration.

Functions of the LWAAP sublayer are performed by LWAAP entities. For an LWAAP entity configured at the eNB, there is a peer LWAAP entity configured at the UE and vice versa. For all LWA bearers, there is one LWAAP entity in the eNB and one LWAAP entity in the UE.

An LWAAP entity at the eNB or the UE receives/delivers LWAAP SDUs from/to upper layers (i.e. PDCP) and sends/receives LWAAP PDUs to/from its peer LWAAP entity via WLAN:

-	When an LWAAP entity receives an LWAAP SDU from upper layers, it constructs the corresponding LWAAP PDU and delivers it to lower layers;

-	When an LWAAP entity receives an LWAAP PDU from lower layers, it reassembles the corresponding LWAAP SDU and delivers it to upper layers.

Figure 4.2.1-1 illustrates the overview model of the LWAAP sublayer.

Figure 4.2.1-1: Overview model of the LWAAP sublayer

Skip unchanged parts

5.1.1	Data receiving procedures

When receiving an LWAAP data PDU from lower layers, the LWAAP entity shall:

-	identify the upper layer entity to which the LWAAP SDU is destined based on the DRB ID included in the LWAAP header;

-	reassemble the LWAAP SDU from the LWAAP data PDU by removing the LWAAP header from the LWAAP data PDU;

-	deliver the reassembled LWAAP SDU to the upper layer entity identified by the DRB ID.

5.1.2	Data sending procedures

When receiving an LWAAP SDU from upper layers, the LWAAP entity shall:

-	set the DRB ID field in the LWAAP header based on the DRB ID of the upper layer entity from which the LWAAP SDU is received;

-	assemble the LWAAP data PDU by adding the LWAAP header to the LWAAP SDU;

-	deliver the assembled LWAAP data PDU to the lower layer entity for transfer.

End of Text Proposal