3GPP TSG RAN WG2 Meeting #93bis	R2-162191

# Dubrovnik, Croatia, 11th  – 15th  April 2016

Source:	CATT

Title:	Discussion on UP Aspect to Support 60GHz and High-rate WLAN

Agenda Item:	8.5.3.2

Document for:	Discussion and Decision

Introduction

During RAN#71 meeting, new WI for LWA enhancement was approved [1]. And one objective of the new WI is: “Potential enhancements to support 60 GHz new band and channels (e.g. in measurements) and increased data rates for 802.11ax, 802.11ad, and 802.11ay (e.g. by PDCP optimizations) (RAN2, RAN3)” [1]. This contribution will discuss further details on the UP aspects.

Discussion

The following sections will discuss the potential impacts and several key issues on UP aspects to support 60GHz and new evolution of WLAN in Rel-14 LWA.

- Scenario Study

The new evolution of  WLAN (802.11ax, 802.11ad, and 802.11ay) is designed to support extreme high data rate (about 10Gbps or more) but with a rather limited coverage range and it probably mainly focus on the “personal usage” scenarios, which is not like the typical use cases of operator deployed WLAN with comparative larger coverage range and user capacity.  As the designed scenarios will decide the system architecture and functions, it is worthy to study first the applicable or targeted scenarios and use cases.

Proposal 1: It is proposed to discuss and make clarifications firstly on the typical use case/scenarios to support the high data rate WLAN (802.11ax, 802.11ad, and 802.11ay) in Rel-14 LWA.

- Bearer Split vs. Bearer Switch

IEEE 802.11ax/ay/ad could support up to 10Gbps or more peak data rate at WLAN side, which is a much higher than the data rate of LTE side. Under split bearer structure, it will create a big challenge to guarantee the “in-sequence” delivery of PDCP at the receiver side, while originally the DC structure in Rel-12 is only designed with the assumption that the two links should have similar data rate. Also, a very large PDCP re-order buffer is needed at UE side which unnecessarily increases the UE implementation complexity.

Alternatively, under the switch bearer structure, there is no such issue of PDCP re-ordering and not that strict requirement of UE re-ordering buffer as that under split bearer structure. To reduce implementation complexity, only to use switch bearer structure for LWA of high data rate WLAN (802.11ax, 802.11ad, and 802.11ay) will be more simple and effective.

Proposal 2: Only switch bearer structure is supported for LWA of high data rate WLAN (802.11ax, 802.11ad, and 802.11ay).

- PDCP SN Extension

For current 18 bit PDCP SN size, assuming IP packets of 1500 bytes and PDCP RTT of 50ms, the maximum supported data rate could be calculated as: (1500*8*2^(18-1))/0.05=31.5Gbps. That rate is still bigger than the maximum theoretical rate of 802.11ay. Considering a more realistic scenarios, the actual data rate of 802.11ay could be much less due to radio conditions or user distributions. Thus it is suggest that the using current PDCP SN size should be enough.

Proposal 3: There is no need to extend current 18 bit PDCP SN size to support LWA of high data rate WLAN (802.11ax, 802.11ad, and 802.11ay).

- PDCP Encryption Issue

At RAN#71, there are some discussions to remove the PDCP encryption function to support LWA of high data rate WLAN, to save UE processing load and power consumptions. However, we think this may be a bit deviated from the original LWA structure based on DC architecture and may have some security concerns. Thus consultation with SA3 is needed.

Proposal 4: It is proposed RAN2 send LS to SA3 to consult the possibility to remove PDCP encryption function to support LWA of high data rate WLAN.

- Layer 2 Buffer Size

Obviously, under the high data rate of WLAN, the UE layer 2 buffer size needs to be extended to support the higher data rate transmitting/receiving.

Proposal 5: UE layer 2 buffer size needs to be expanded accordingly to support LWA of high data rate WLAN (802.11ax, 802.11ad, and 802.11ay).

Conclusion

It is proposed to discuss and capture the following proposals at RAN2:

Proposal 1: It is proposed to discuss and make clarifications firstly on the typical use case/scenarios to support the high data rate WLAN (802.11ax, 802.11ad, and 802.11ay) in Rel-14 LWA.

Proposal 2: Only switch bearer structure is supported for LWA of high data rate WLAN (802.11ax, 802.11ad, and 802.11ay).

Proposal 3: There is no need to extend current 18 bit PDCP SN size to support LWA of high data rate WLAN (802.11ax, 802.11ad, and 802.11ay).

Proposal 4: It is proposed RAN2 send LS to SA3 to consult the possibility to remove PDCP encryption function to support LWA of high data rate WLAN.

Proposal 5: UE layer 2 buffer size needs to be expanded accordingly to support LWA of high data rate WLAN (802.11ax, 802.11ad, and 802.11ay).

References

  - RP-160600, New Work Item on enhanced LTE-WLAN Aggregation (LWA), by Intel, Qualcomm, China Telecom