3GPP TSG-RAN WG2 #93bis	Tdoc R2-162781

Dubrovnik, Croatia, 11–15 April 2016

Agenda Item:	8.8.3

Source:	Ericsson

Title:	Acknowledgements for SPS commands

Document for:	Discussion, Decision

# Introduction

A new work item for L2 latency reduction techniques had been approved by RAN plenary [2]. With the intended technique of short SPS periods and skipping of padding transmissions, faster UL access is achieved, while UE power consumption and UL interference is kept low.

While we discuss the feature in general in our companion contributions [3], [5], in this contribution we discuss in particular the need for acknowledgements for SPS commands, such as activation and explicit release, as well as implicit release.

# Discussion

In current SPS, as response to the PDCCH grant activating SPS, the UE transmits on the first SPS resource either with a regular data transmission, or in case of empty UE buffer, with a new MAC PDU containing zero MAC SDUs. This way, the activation of SPS is acknowledged to the eNB. As response to the PDCCH grant releasing SPS, the UE stops all transmissions on the SPS resources, and by the absence of transmissions, the eNB is implicitly informed of successful SPS release.

For the solution of SPS with skipped padding transmissions, the need and different means for acknowledgement of activation and release have been discussed in [1].

We believe that a main concern in Rel-13 was the strict timeframe for standardizing acknowledgements for activation and release. In the complete solution of Rel-14, such acknowledgments should be included. It is important from network point of view to be sure about the UE state in order to operate efficiently. The potential state mismatch when no means of acknowledgment is introduced is thus avoided. Acknowledgements to eNB about SPS activation and release in the UE are important from network operational point of view.

In the following we discuss further how such acknowledgement solutions can be realized.

## Acknowledgement for SPS activation

In legacy SPS, the acknowledgment of the SPS activation is given by the first PUSCH transmission on the activated SPS resources. It is straightforward to reuse this method also in the solution of SPS with skipped padding transmissions. Therefore, it is important to specify an exception of skipping padding for the first transmission on SPS resources after receiving the SPS activation grant, i.e. to apply legacy behavior in this case.

UE does not skip padding transmission on the first SPS resource after receiving the SPS activation grant.

We believe that by all means, this is the simplest solution, as can also be seen from a specification impact perspective. A draft CR can be found in [4]. Alternative solutions based on e.g. ACK on PUCCH seem overly complex here, as they would have also considerable RAN1 impact.

Also, since the main motivation of SPS in general is to decrease PDCCH load, we believe that the SPS activation is not triggered very frequently and thus the additional overhead and UL interference from this acknowledgment is negligible. With this in mind, both repetition of the PDCCH grant activating the resource and using a more robust PDCCH resource allocation would increase the PDCCH load and resource utilization. Furthermore, one can assume that as SPS functionality become more efficient, the need for very frequent SR opportunity occasions is smaller, thus making the recovery of missed activation detection rather slow.

When it comes to the timing of SPS activation, it is assumed that the first SPS resource can be used within the same time as a dynamic uplink grant becomes usable (4TTI). This way, the first PUSCH (data or padding) acknowledging the SPS grant can be received in the eNB within 5TTI. From this time on the eNB can be sure that SPS has been successfully activated, thus sends an ACK on PHICH. Otherwise the eNB may retransmit the PDCCH activating the SPS grant. The UE on the other side, if no ACK is received on PHICH, a non-adaptive retransmission could be sent after 8TTI of the PUSCH on the SPS resources [3]. This way, error cases and state mismatch between eNB and UE are resolved.

## Acknowledgement for SPS explicit release

In legacy SPS, the PDCCH grant indicating SPS release is not explicitly acknowledged. In the legacy solution, invalid values in the DCI are used to indicate SPS release. After receiving this command, the UE immediately releases the SPS resources, thus transmission on the resources is not possible anymore (and the invalid values in the DCI are simply ignored).

Different alternative solutions for acknowledgment of SPS release could be standardized:

- Solution based on ACK on PUCCH/PUSCH after SPS release on PDCCH.

- Solution based on DL grant, so that UE would send ACK on PUCCH/PUSCH for DL reception.

- Solution based on single PUSCH padding transmission delaying SPS release.

Also here, we believe that solutions not based on padding transmissions are overly complex. With solution 3, in case of SPS with skipped padding transmissions, the SPS resources can be released just one SPS interval later, and a padding transmission on the last SPS resources is issued (this means that padding is not skipped on this last SPS resource). Thereby the valid DCI from the SPS activation is used (as in all other transmissions on SPS resources) and the DCI with the release-format (and it’s invalid values) from the SPS release is ignored. Also this solution is closest to legacy behavior and aligned with the solution of acknowledging SPS activation (Proposal 1).

Upon SPS release for SPS with skipped padding, UE delays the release of SPS resources by one period, and does not skip padding on the last SPS resource.

Afterwards, the UE releases its SPS uplink grant configuration. The eNB, upon receiving the last PUSCH transmission considers the SPS resource as successfully released and can schedule the resources to other UEs; otherwise the eNB resends the explicit release command to the UE. Assuming here also a minimum delay of 4TTI (or higher, if SPS period is higher) to the PUSCH transmission after receiving the SPS release, the eNB is informed of the SPS release after minimum 5TTI.

## Acknowledgement for SPS implicit release

In our companion paper [3] we discuss the support of implicit release of SPS with skipping padding. Two mechanism are identified as beneficial:

- Implicit release after number of skipped transmission, following the legacy implicit release after number of padding transmission

- Implicit release after number of subframes, independent of skipped transmissions

Whether the implicit release by the UE needs to be acknowledged to the eNB depends on whether a mismatch between eNB and UE state could occur. I.e. the eNB should be informed of whether the UE has released its SPS resources or not for the same reasons as for the explicit release acknowledge.

For the case of SPS with skipped padding and implicit release after a number of skipped padding transmissions, a mismatch of UE and eNB state can occur, since the eNB might not detect some of the UEs transmissions on SPS resources. Therefore, the counting of skipped transmission opportunities in eNB and UE might have different results. Therefore, it is important that the implicit release is acknowledged in this case. Following legacy behaviour, where padding is sent in the last SPS resource before releasing it, it can be mandated that the UE also transmit on the last SPS resource before implicit release, i.e. it does not skip padding in this case.

If implicit release after number of SPS-granted TTIs with skipped padding transmissions is supported, the UE does not skip padding in the last SPS resource before releasing it.

In case that the implicit release is done after a number of configurable subframes/time a mismatch of state between UE and eNB can only occur if the assumed starting time of SPS activation is not the same between eNB and UE. This can however be ensured with the SPS activation acknowledgement according to Proposal 1.

If implicit release after a configured number of TTIs is supported, the UE acknowledges only the SPS grant. Then implicit release of SPS is not acknowledged additionally.

# Conclusion

Based on the discussion in the previous section we make the following proposals:

- UE does not skip padding transmission on the first SPS resource after receiving the SPS activation grant.

Upon SPS release for SPS with skipped padding, UE delays the release of SPS resources by one period, and does not skip padding on the last SPS resource.

If implicit release after number of SPS-granted TTIs with skipped padding transmissions is supported, the UE does not skip padding in the last SPS resource before releasing it.

If implicit release after a configured number of TTIs is supported, the UE acknowledges only the SPS grant. Then implicit release of SPS is not acknowledged additionally.

# References

TR 36.881

RP-160667, New WI proposal: L2 latency reduction techniques for LTE

R2-162782 Skip padding option for UL grants, RAN2#93bis, April 2016

R2-162780, 36321_draftCR_(Rel-14)_R2-162780 - Skipping padding, RAN2#93bis, April 2016

R2-162768, Introduction of short SPS period, RAN2#93bis, April 2016