3GPP TSG-RAN WG2 Meeting #93bis	R2-162856

Dubrovnik, Croatia, April 11 – April 15, 2016

Agenda item:	7.14.2.1 (NB_IOT-Core)

Source:	LG Electronics Inc.

Title: 	Introducing SRB3 for NB-IOT

Document for:	Discussion and Decision

1.	Introduction

The document explains the need for introducing SRB3 that can be used for CP solution in NB-IoT. It is supplementary document to the one presented in SA2-RAN joint CC1 [1] for better understanding of new SRB3.

2.	Discussion

In LTE, the SRB1 is used for transmitting RRC messages with integrity protection applied, i.e. 1 byte PDCP header and 4 bytes MAC-I are always included even if the AS security is not applied. If the SRB1 is used for CP solution in NB-IoT (i.e. Data over NAS) where the AS security is not applied, 5 bytes overhead are always transmitted without having any gain.

To avoid this unnecessary 5 byte overhead for CP solution, some companies proposed to make the use of PDCP configurable for SRB1 [2]. In detail, SRB1 uses PDCP TM before AS security is activated, and uses legacy PDCP (i.e. PDCP for SRB) when AS security is activated. The SRB1 does not switch back to PDCP TM as long as the UE stays in RRC connected.

However, we think making PDCP configurable for SRB1 has following defects.

- In LTE, one RB has one single L2 configuration. Making PDCP configurable for one RB introduces a new concept for a radio bearer, i.e. SRB1 w/ PDCP and SRB1 w/o PDCP.

- In LTE, the UE cannot change RB configuration without network command. Making the UE to activate or deactivate PDCP depending on the security activation introduces UE autonomous change of RB configuration without network command.

- The LCID is the logical channel identifier which identifies a radio bearer. Using LCID to differentiate the existence of PDCP introduces a new concept for LCID, i.e. the LCID is used to identify whether a special L2 configuration is used for a radio bearer.

With above reasons, we think it should be avoided to make PDCP configurable for SRB1. If PDCP is not needed for SRB, it should be defined as a new SRB.

Moreover, defining a new SRB, i.e. SRB3, has following benefits.

- No impacts to SRB1 which is used for legacy UE.

- Efficient support for simultaneous CP and UP solutions

  - No packet-by-packet identification for SRB1 messages whether security is applied or not

  - No interaction between PDCP and MAC for LCID attachment

- Allow separation of signaling and data for CP solution

  - Signaling is transmitted on SRB1, and data is transmitted on SRB3

Proposal: Define SRB3 to support “Data over NAS”. The SRB3 is same as SRB1 except the PDCP is applied.

3.	Proposal

In this document, we have explained that introducing SRB3 is better than changing SRB1 configuration, and propose that:

Proposal: Define SRB3 to support “Data over NAS”. The SRB3 is same as SRB1 except the PDCP is applied.

4.	References

[1] SA2-RAN2 Joint CC1 (Mar22, 2016) Tdoc#07	PDCP handling for CP-UP solution		LG Electronics Inc.

[2] SA2-RAN2 Joint CC1 (Mar22, 2016) Tdoc#05	PDCP handling for SRB1	ZTE