3GPP TSG-RAN WG2 Meeting #95			     R2- 164730

Goteborg, Sweden, August 22-26, 2016

Agenda item:	8.12

Source: 	Apple

Title: 	Higher performance targets for Further Enhanced MTC.

Document for:		Discussion and Decision

# Introduction

Working Item for Release-14 “Further Enhanced MTC” [1] has been approved in RAN Meeting #72. This Working Item has, among others, the following Objectives:

“Higher data rates [RAN1, RAN2, RAN4]

- Specify HARQ-ACK bundling in CE mode A in HD-FDD

- Larger maximum TBS

- Larger max. PDSCH/PUSCH channel bandwidth in connected mode at least in CE mode A in order to enhance support e.g. voice and audio streaming or other applications and scenarios

- Up to 10 DL HARQ processes in CE mode A in FD-FDD”

This contribution discusses the RAN2 impacts of higher data rate target and the issues that should be analyzed.

# Discussion and proposals

3GPP work in Release-13 eMTC specified new features allowing cellular modem form-factor cost and complexity reduction, battery life extension and coverage enhancements.

These features were designed for a set of devices/use cases, such as smart meters, sensors and smart readers that requires neither real time services nor high data rates.

Today, wearable devices usage is expanding at fast pace and these devices provide their own set of services and applications that could be summarized as follows:

- HD Voice and Multimedia Calls

- HD Music streaming

- Video streaming

- Human-interface originated operations.

It is undeniable that wearable devices share some common requirements with other MTC devices such as form factor cost & complexity, battery life and coverage enhancements. On top to these requirements, wearables enable new services that are delay-sensitive and require a higher peak data rate in both normal and enhanced coverage modes.

Proposal 1: RAN2 to work on enhancing R-13 eMTC features based on wearables services/applications requirements when operating in normal coverage and enhanced coverage in TDD, FDD and HD-FDD.

## Larger Maximum TBS

Applications such HD music could require a large peak unicast traffic-throughput in normal coverage (at least in downlink).

In addition to that, a user’s access to short videos or files download through wearable interfaces requires a high peak data rate for a short time. This aspect of response delay is really important for an acceptable user experience.

The first limitation of R-13 eMTC devices is the max TBS size of 1000 bits [2]. RAN1 is tasked in the working item to relax the maximum TBS size beyond 1000 bits. In order, for the UE, to inform the eNB with supported Max TBS in UL-SCH and DL-SCH, UE capabilities have to be updated with new Max TBS sizes and the buffer sizes updated accordingly.  In addition to that, the impact of such extension on System Information for BL and UE in CE should be investigated.

Proposal 2: RAN2 to extend UE capabilities with support of larger TBS and its corresponding buffer size.

## Larger PDSCH/PUSCH Bandwidth

The second limitation on data rates in Release 13 eMTC devices is the 6PRB restriction, especially in downlink transmission. In order to support higher peak data rates, eMTC devices should be able to support larger PDSCH/PUSCH bandwidth in both normal and enhanced coverage.

The scalability of PDSCH/PUSCH bandwidth should be signaled between the UE and the network. It is within RAN2 scope to determine signaling mechanisms enabling the support and scalability of larger PDSCH/PUSCH bandwidth.

Proposal 3: RAN2 to investigate necessary signalling mechanisms to enable larger PDSCH/PUSCH bandwidth for eMTC in both normal coverage and enhanced coverage.

## Higher Data Rates Optimizations

HD-FDD form-factors provide certain advantages such as cost and power consumption when compared to Full duplex FDD form factors.  On the other hand, this mode of operation does not allow PDSCH reception while PUSCH transmission, hence a lower peak throughput.

Some optimisations could be considered to improve HD-FDD operation such as:

- Bundling of HARQ-ACK (at least for Downlink), could unleash higher pick throughput [3].

Proposal 4: RAN2 to investigate signalling impact of HARQ-ACK bundling.

# Conclusion

In this contribution, we listed some important design targets for eMTC based in wearable use-case and we propose following directions to enable necessary higher performances:

Proposal 1: RAN2 to work on enhancing R-13 eMTC features based on wearables services/applications requirements when operating in normal coverage and enhanced coverage in TDD, FDD and HD-FDD.

Proposal 2: RAN2 to extend UE capabilities with support of larger TBS and its corresponding buffer size.

Proposal 3: RAN2 to investigate necessary signalling mechanisms to enable larger PDSCH/PUSCH bandwidth for eMTC in both normal coverage and enhanced coverage.

Proposal 4: RAN2 to investigate signalling impact of HARQ-ACK bundling.

# 4. Reference

[1] RP-161321 “New WI proposal on Further Enhanced MTC”, Ericsson

[2] 3GPP TS 36.306 v13.2.0  “User Equipment (UE) radio access capabilities”

[3] R1-153111 “M-PDCCH and HARQ Options for HD-FDD in Normal Coverage”, Sierra Wireless