3GPP TSG-RAN WG2 Meeting #95                                                                     				          R2-165217

Göteborg, Sweden, 22nd – 26th August 2016

Agenda item:	8.3.4

Source:	CMCC

Title:                     TP for VoLTE/video enhancements to improve quality

Document for:	Discussion

Introduction

In RAN2#94 meeting, the following agreements regarding the VoLTE/video quality related enhancements were agreed [1]:

- => Focus the study of VoLTE/video quality related enhancements on the techniques introduced by the eMTC work item.

- => Focus the study of VoLTE/video quality related enhancements on the techniques available in CE mode A (with/without SPS) for Cat-M1 and other UE categories.

As a consequence, a LS [3] with recommended evaluation assumptions provided in Table 1 [2] was agreed to be provided to RAN1 for further evaluation.

However, these agreements and progress has not been captured into the TR [4] and hence we try to make it in this contribution.

Discussion

Capture agreements

As mentioned above, the following agreements should be captured properly [1]:

- => Focus the study of VoLTE/video quality related enhancements on the techniques introduced by the eMTC work item.

- => Focus the study of VoLTE/video quality related enhancements on the techniques available in CE mode A (with/without SPS) for Cat-M1 and other UE categories.

Especially, in [3], it was further indicated that 3GPP should investigate the trade-off between packet delay budget and achievable coverage to assess the benefits of VoLTE/video quality related enhancements. In order to make it clear what the coverage enhancement is related to the packet delay budget and what the techniques introduced for eMTC are, it would be beneficial to further clarify the candidate solutions with the following elaborations:

Solution 1. Coverage extension with relaxed delay budget

Several steps of this solution are:

- eNB may get the VoLTE packet E2E delay information report from UE.

- When the voice packet loss rate is high than a threshold, eNB can increase the maximum HARQ re-transmission times if the E2E delay budget is allowed, or increase the RTT to achieve the time diversity gain in longer transmission time.

Solution 2. Use eMTC technologies at CE mode A for VoLTE

For the coverage enhancement technologies of eMTC CE mode A, they are already specified in Rel-13, e.g. PUSCH coverage enhancement with asynchronous HARQ, the narrowband hopping and multi-SF channel estimation etc. and other technology e.g. denser DMRS was also discussed in eMTC WI and will probably useful for VoLTE coverage enhancement.

Capture evaluations

RAN1 is expected to do the evaluation based on RAN2 recommendation [3]. Considering the quite limited meeting time allocated, RAN1 is not likely to have a comprehensive analysis and evaluation but a general consideration and determination for further work.

From the TR point of view, it would be useful to also record the recommended evaluation assumptions and analysis. In light of this consideration, we propose to capture the recommended evaluation assumptions and analysis.

Draft conclusions

In order to help progress the convergence, a drafted conclusion is also provided. Note this may also depend on the RAN1 evaluation and feedback. Once the feedback is received, the conclusion may be revised accordingly.

Conclusions

In this paper, we summarized the agreements and progress, and accordingly propose to capture them in the TR.

A Text Proposal is provided below.

References

- 3GPP RAN2#94 Meeting Report from LTE Break-Out session (eVoLTE, Light conn, Mobility enh, eMBMS)

- R2-164566. LS on VoLTE and video quality related enhancements. Ericsson

- R2-164579. VoLTE/video quality related enhancements. Ericsson

- 3GPP TR 36.750 Study on Voice and Video enhancement for LTE

------------------------------------------------------------Text Proposals---------------------------------------------------------------

- 7	VoLTE/video enhancements to improve quality

- 7.1	Identified problems of existing mechanisms

VoLTE is not perfectly designed to fulfill the voice specific requirements and to take full advantage of EVS codec when it comes to coverage aspects. HD voice quality strongly relies on the radio transmission reliability, which may be critical in certain environments, such as indoor situations. 3GPP previously enhanced coverage specifically for 328bits MAC PDUs of AMR12.2kbps and did not consider EVS, which can provide better quality with lower codec rates such as, for example7.2kbps with 224 bits MAC PDUs. Therefore it turns to be useful to study possible improvements in the coverage enhancements area.

- 7.2	Design Principles

During the study, following design principles should be considered when developing the candidate solutions:

- Focus the study of VoLTE/video quality related enhancements on the techniques introduced by the eMTC work item.

- Focus the study of VoLTE/video quality related enhancements on the techniques available in CE mode A (with/without SPS) for Cat-M1 and other UE categories.

- The study should investigate the trade-off between packet delay budget and achievable coverage to assess the benefits of VoLTE/video quality related enhancements.

- 7.3	Candidate solutions

Editor’s note: This section is intended to capture the candidate solutions driven by the contributions and agreed by the meetings.

In order to improve the quality of VoLTE/Video, the candidate techniques were considered:

- Solution 1. Coverage extension with relaxed delay budget

Several steps of this solution are:

- eNB may get the VoLTE packet E2E delay information report from UE.

- When the voice packet loss rate is high than a threshold, eNB can increase the maximum HARQ re-transmission times if the E2E delay budget is allowed, or increase the RTT to achieve the time diversity gain in longer transmission time.

Solution 2. Use eMTC technologies at CE mode A for VoLTE

For the coverage enhancement technologies of eMTC CE mode A, they are already specified in Rel-13, e.g. PUSCH coverage enhancement with asynchronous HARQ, the narrowband hopping and multi-SF channel estimation etc. and other technology e.g. denser DMRS was also discussed in eMTC WI and will probably useful for VoLTE coverage enhancement.

- 7.4	Evaluations and Conclusions

As stated in [3] for the recommended evaluation assumptions, the EVS 7.2 kbps is used as the baseline speech codec for evaluation and evaluate the effect of handling up to four speech frames in one transmission. Besides, in order to evaluate possible coverage enhancements benefits, both RAN aggregates up to four RTP packets with one speech frame encapsulated in one RTP packet and the application encapsulating up to four packets in one RTP packet are considered. Furthermore, full ROHC header compression is assumed, and BSR and PHR in each transmission. The resulting required transport block size including PDCP, RLC, and MAC headers for the different packet bundling strategies is presented in Table 1.

Table 1: Proposed recommended evaluation assumptions for VoLTE quality related enhancements

| Codec | RAN aggregation (Note 1) | Application encapsulation (Note 2) | Payload (Note 3) | RTP header (ROHC) | PDCP | RLC | MAC | BSR | PHR | Inter tx time (ms) | Total (bytes) | TBS (bits) (Note 4) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EVS 7.2kbps | 1 | 1 | 18 | 3 | 2 | 2 | 1 | 4 | 2 | 20 | 32 | 256 |
| EVS 7.2kbps | 2 | 1 | 18*2 | 3*2 | 2*2 | 4 | 1 | 4 | 2 | 40 | 57 | 456 |
| EVS 7.2kbps | 3 | 1 | 18*3 | 3*3 | 2*3 | 5 | 1 | 4 | 2 | 60 | 81 | 648 |
| EVS 7.2kbps | 4 | 1 | 18*4 | 3*4 | 2*4 | 7 | 1 | 4 | 2 | 80 | 106 | 848 |
| EVS 7.2kbps | 1 | 2 | 19*2 | 3 | 2 | 2 | 1 | 4 | 2 | 40 | 52 | 416 |
| EVS 7.2kbps | 1 | 3 | 19*3 | 3 | 2 | 2 | 1 | 4 | 2 | 60 | 71 | 568 |
| EVS 7.2kbps | 1 | 4 | 19*4 | 3 | 2 | 2 | 1 | 4 | 2 | 80 | 90 | 720 |

Note 1:	RAN aggregation of RTP packets each composed of one speech frame

Note 2:	Application encapsulation of speech frames in one RTP packet may require updates to the VoLTE service description [6]

Note 3:	For Payload, Compact Format is used for RAN aggregation and Header-Full Format is used for Application encapsulation as described in Annex A of TS 26.445

Note 4:	Some of the TBS values do not match the valid TBS values in Table 7.1.7.2.1-1 in 36.213. It is assumed that lower layers will choose a suitable TBS value, equal to or greater than the values in this column.

Typically a rather tight packet delay budget is assumed for conversational services. While this is often required, there are also scenarios where a more relaxed packet delay budget can be considered acceptable (perhaps allowing for one-way delays of several hundred ms instead of 50-100 ms) if this means that the service can be provided with a larger coverage. Accordingly, the study should investigate the possible trade-off between packet delay budget and achievable coverage to provide a better understanding of this aspect.

After evaluation, the study concluded that the following solutions are beneficial and feasible to improve the VoLTE coverage:

Solution 1:  relaxing the delay budget over the air interface and then increasing the maximum HARQ re-transmission times and/or increasing the RTT to achieve the time diversity gain in longer transmission time.

Solution 2: re-using the technologies of eMTC CE mode A in Rel-13, e.g. PUSCH coverage enhancement with asynchronous HARQ, the narrowband hopping and multi-SF channel estimation, denser DMRS.

- 8	Conclusions and Recommendations

Regarding the VoLTE/video enhancements to improve quality, the study concluded that the following solutions are beneficial and feasible to improve the VoLTE coverage:

Solution 1:  relaxing the delay budget over the air interface and then increasing the maximum HARQ re-transmission times and/or increasing the RTT to achieve the time diversity gain in longer transmission time.

Solution 2: re-using the technologies of eMTC CE mode A in Rel-13, e.g. PUSCH coverage enhancement with asynchronous HARQ, the narrowband hopping and multi-SF channel estimation, denser DMRS.

Therefore, the above solutions are recommended to be further developed for the normative specification.