3GPP TSG-RAN WG2 Meeting #95 	R2-165309

# Gothenburg, Sweden, 22nd – 26th August 2016

| CR-Form-v11.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.304 | CR | 0328 | rev | - | Current version: | 13.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification of UE redistribution target selection |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_MC_load-Core |  |  |  |  |  |  | Date: |  | 2016-08-22 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-13 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) |  |
| Reason for change: |  | 1) If the redistribution factor of the serving frequency or cell is configured to zero, which is possible and useful in certain applications, and the UE does not find any cell qualifying the cell selection criterion S at the carrier frequencies in the inter-frequency carrier frequency lists in SIB5 associated with a redistribution factor, the calculation of the redistribution range for the serving frequency or cell yields a division zero by zero, which is not defined. 2) The modulo operation on IMSI results in an integer UEID in the range 0 .. 99. The total redistribution range, including all targets and multiplied with 100, spans a range of rational numbers from 0 to 100. Hence, there is a slight assymmetry between the UEID range and the redistribution range, where the UEID values are shifted half a unit towards zero. For instance, if UE shall be distributed between four frequencies, all with equal redistribution factors, one would expect it to split the IMSI range in four equally large sub-ranges (25 IMSI values in each). With the current specification, IMSI 0 .. 25 do select the serving frequency or cell as the redistribution target, IMSI 26 .. 50, the next frequency, IMSI 51 .. 75 the third frequency and IMSI 76 .. 99 the last and fourth frequency. Hence, there are 26 IMSI values in the first group, 25 in each of the next two groups and only 24 in the last group, not 25 in each. |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1) Clarify that if the serving frequency or cell is the only possible redistribution target, the redistribution range of it is = 1. No calculation is required. 2) Expand the total redistribution range to any rational numbers within range 0 to 200 (multiply by 200 instead of 100). Do a remapping of the UEID range from every integer 0 .. 99 onto every odd integer 1 .. 199 (multiply by 2 and add 1). Impact Analysis Impacted functionality Inter-frequency redistribution in idle mode. Inter-operability There are no interoperability issues as the CR impacts UE behaviour only. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The UE behaviour is undefined when the redistribution range calculations yieds a division of zero by zero. The assymmetric division of the IMSI range prevails. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | x | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

### 5.2.4		Cell Reselection evaluation process

##### 5.2.4.10.1	Redistribution target selection

The UE shall compile a sorted list of one or more candidate redistribution targets, and for each candidate entry [j] a valid redistrFactor[j], in which entries are added in increasing index order starting with index 0 as follows:

-	for the serving frequency (redistributionFactorServing is included in SystemInformationBlockType3 whenever redistribution is configured):

-	the serving cell if redistributionFactorCell is included;

-	otherwise the serving frequency;

-	In both cases, redistrFactor[0] is set to redistributionFactorServing;

-	for each entry in InterFreqCarrierFreqList and subsequent for each entry in InterFreqCarrierFreqListExt:

-	the cell ranked as the best cell on this frequency according to section 5.2.4.6 if redistributionNeighCellList is configured and includes this cell;

-	otherwise, the concerned frequency if redistributionFactorFreq is configured and if at least one cell on the frequency fullfills the cell selection criterion S defined in 5.2.3.2;

-	If the cell is included, redistrFactor[j] is set to the corresponding redistributionFactorCell; If the frequency is included, redistrFactor[j] is set to the corresponding redistributionFactorFreq;

The UE shall choose a redistribution target as follows:

-	If ueID≤100*redistrRangeueID≤200∙redistrRange[0],  the UE shall choose the frequency or the cell corresponding to redistrFactor[0] as its redistribution target or;

100*j=0j=i-1redistrRange[j]<ueID≤j=0j=iredistrRange[j]-	If 200∙j=0j=i-1redistrRange[j]<ueID≤200∙j=0j=iredistrRange[j], then the UE shall choose the frequency or cell corresponding to redistrFactor[i] as its redistribution target;

-	ueID=IMSI mod 100∙2+1;

If there are no redistribution candidates apart from the serving frequency or cell, the redistrRange[0] = 1.

Otherwise, the redistrRangeiredistrRange[i] of E-UTRAN frequency or cell is defined by:

Where: maxCandidates is the total number of frequencies/cells with valid redistrFactor[j].