3GPP TSG-RAN WG2 #95	Tdoc R2-165337

# Gothenburg, Sweden, 22nd – 26th August 2016

Agenda Item:	9.4.2.1

Source:	Ericsson

Title:	Second level retransmissions in NR

Document for:	Discussion

# Introduction

NR should address new use cases for mobile networks compared to LTE with new requirements such as:

Higher speed - Which may result in that the link deteriorates quickly

Higher reliability with low latency - Which may require reliability up to 10-5 error within a 1 ms.

Higher data rates and higher frequency operation - With throughputs up to 20 Gbps and frequencies up to 100 GHz.

In the previous meeting and an in the email discussion [94#38], it was discussed if second level retransmissions should be supported in NR and if so, where it should be placed. In this contribution we give some further analysis w.r.t. the open issues in that email discussion.

# Discussion

## Need for second level retransmissions

HSDPA and in particular LTE aimed to enhance the performance for data services. It had been understood that data rates required by applications varies significantly over time. A key requirement was therefore, that the available resource (frequency, power) can be allocated instantaneously to the UEs that need them. Fast link adaptation was used to adjust the modulation and coding scheme equally instantaneously to the available radio conditions.

Many data services use TCP as the underlying transport protocol. TCP aims to adjust the application’s send rate to the capacity of the bottleneck link along the end to end path. It increases the send rate (i.e., the so-called congestion window) for data successfully acknowledged by its receiving peer, and it decreases the congestion window when it observes packet losses – assuming that losses are (only) caused by congestion. Therefore, TCP requires low end-to-end latency to increase the congestion window quickly and low loss rates to avoid unintended reduction of the congestion window.

TCP requires low IP loss rates and low end-to-end latency to benefit from high data rates offered by an underlying link.

In contrast to that, the physical layer’s capacity and throughput can be maximized when aiming for an initial block error rate of about 10%, provided that these errors are corrected efficiently by means of HARQ (soft-combining). Since TCP requires low end-to-end latency, the HARQ protocol has to perform these retransmissions as quickly as possible. In LTE, this is achieved by sending instantaneous HARQ ACK/NACK as soon as the decoding attempt completes, instead of sending combined status report feedback for many packets (as e.g. in UMTS RLC). So, in LTE only a single ACK/NACK-bit is used. This single bit could not be encoded in any way other than by repetition.  Further, this single bit could not be protected by a CRC. It was concluded that the fast HARQ protocol can efficiently correct most of the losses but cannot achieve the low residual error rate required by TCP (~10-6) with reasonably low overhead due to HARQ feedback errors.

The single-bit ACK/NACK feedback in LTE Rel-8 was not considered sufficiently reliable to achieve the required low residual loss rate with reasonable overhead.

Instead, the RLC ARQ protocol corrects the few remaining losses observed above MAC. The overhead introduced by RLC status messages is small since status messages are not sent for every RLC PDU and the status messages can often be piggybacked with RLC SDUs e.g. TCP ACKs. For some types of traffic the overhead of RLC AM is however significant, e.g. small packets. Furthermore, RLC receiver needs some time to reliably detect lost MAC PDUs and to request and obtain the RLC retransmission. In the meantime it needs to store all other received RLC PDUs and in order to deliver RLC SDUs in the correct order. Hence, RLC AM causes some latency spikes and requires a correspondingly larger L2 buffer to maintain in-sequence delivery.

RLC provides in-sequence delivery service to higher layers. This implies that, after the RLC entity detects a missing RLC PDU, the RLC entity stores subsequent received RLC PDUs. When the missing RLC PDU has been received, that PDU and subsequent RLC PDUs are delivered in-sequence to higher layers. A L2 buffer is, therefore, needed to provide in-sequence delivery and coping with packet losses.

LTE RLC AM corrects residual HARQ errors reliably with relatively low overhead but, as any protocol for ensuring in-order-delivery, at the expense of latency spikes and hence L2 memory.

Modern TCP variants cope better with packet re-ordering since they restore the previous congestion window size as soon as they detect that the gap in the sequence of received TCP segments was actually not a packet loss. With these new TCP capabilities, removing or reducing the in-sequence delivery functionality from the L2 protocol stack could be considered in order to save L2 memory. However, the effects should be studied carefully before taking such a decision.

Modern TCP variants cope better with packet re-ordering since they do not interpret re-ordering as packet loss.

In LTE Rel-13 aggregation of up to 32 carriers has been introduced. In order to cope with the resulting number of HARQ ACK/NACKs, new UCI formats have been introduced. Those formats protect the ACK/NACK array by means of CRC which allows the eNB to detect transmission errors (ACKNACK & NACKACK) and to perform another retransmission of the affected processes. Thereby, the CRC protection of the HARQ feedback makes the HARQ protocol more robust.

CRC protection of HARQ feedback (as in PUCCH format 4 and 5) allows the eNB to detect NACKACK errors and thereby to reduce residual MAC errors.

For downlink HARQ operation in NR, it should be investigated whether enhancements such as CRC protection of UL HARQ feedback can make the HARQ protocol sufficiently reliable so that the additional RLC ARQ is no longer needed in downlink. While the additional overhead (for CRC and HARQ bitmaps) may be affordable in small cells with relatively few UEs and high cell edge data rates, the two level-ARQ may be preferable in wide area macro cells.

UL in LAA and eMTC relies entirely on asynchronous adaptive HARQ, i.e., the UE performs HARQ retransmissions only upon receiving an UL grant with non-toggled NDI from the eNB. Even if the UE misses such a grant, the eNB has the possibility to request another retransmission in a subsequent subframe. In principle there is also no need for the UE to flush the UL HARQ buffer unless the eNB issues a grant for a new data transmission (toggled NDI). Therefore, the eNB could request even a large number of HARQ retransmissions for the same process. Due to these properties asynchronous adaptive HARQ is expected to show a very low residual error rate.

If it can be shown that enhancements such as CRC protection of UL HARQ feedback as well as Asynchronous Adaptive UL HARQ could make the HARQ protocol sufficiently reliable and resource efficient, it can be considered to no longer use RLC ARQ.

It should also be taken into account that LTE allows using (RLC AM) or not using (RLC UM) the second ARQ level. Thereby, it provides a simple means to trade between latency/jitter and residual loss rate. If RAN2 decides to substitute the RLC ARQ by a more robust MAC HARQ scheme, it would by default apply to all MAC transmissions. While the network could possibly decide to terminate a HARQ process after a certain time depending on the contained user data, this is more challenging to achieve in uplink direction.

The possibility to configure RLC AM or UM for a DRB allows trading between latency/jitter and residual loss rate depending on the service requirements. This trade-off would become more difficult if RLC ARQ is substituted by a robust HARQ protocol.

## Placement of second level retransmissions

As discussed above, there may be scenarios where retransmissions above HARQ are needed and whether this is needed depends for example on HARQ reliability in NR, which require RAN1 input and RAN2 would therefore need to wait for RAN1 progress.

If however a second level retransmission scheme is needed, it is up to RAN2 to decide where in the protocol stack it should be placed and in order to be able to answer this question, we lay down some principles with which a retransmission method should comply to:

- Retransmissions should avoid recovery from congestion.

- Retransmissions should retransmit only the lost data, but not received data.

- Retransmissions should be possible without too large overhead.

- Retransmissions should be configurable per bearer/service.

### Retransmissions over transport network

In UMTS, retransmissions above MAC HARQ are handled by RLC ARQ. This function is placed on the RNC. Placing RLC ARQ in the RNC has certain advantages but also few disadvantages. One of the major disadvantages is that there is a transport network (TN) between the RNC and the Node B and this resulted that ARQ kicked in when either the TN or the Uu showed problems to deliver the packets. There could be at least two different types of issues in the TN. One of the issues is that the TN link could experience transport losses. The second of them is that the TN may be congested and, therefore, packets will be lost. While the first issue can be solved by the RLC ARQ, the second issue cannot be solved by the RLC ARQ. On the contrary, retransmissions of packets which were lost due to congestion will contribute to increased congestion which creates even more losses which will, in turn, lead to more retransmissions, and so on...

What is wanted instead is that if there is congestion on the transport network this should be visible to higher layers such that higher layers adapt to the congestion, e.g. TPC performs back off.

ARQ

RNC

Node B

UE

It is unwanted that L2 ARQ protocol recovers from congestion (occurring e.g. on RAN internal interfaces)

### Retransmission efficiency

Retransmissions should be efficient and adaptive so that it increases the probability for the retransmitted packet to be received. In LTE, RLC receives the PDCP PDUs (containing one IP packet per PDU) and may segments or concatenate these. RLC then performs retransmission of missing segments, not complete PDCP PDUs, this means that only missing data is retransmitted.

In the example in Figure 2, a PDCP PDU is segmented in to three segments and the first and the last part are correctly received while the middle part is not. In this case only PDU_2 needs to be retransmitted. Naturally, to retransmit all segments (including PDU_1 and PDU_3) would not only waste radio resources but would also introduce additional delay since several segments would need to be re-sent instead of just the missing segments.

Figure 2

It is beneficial that the L2 retransmission scheme only retransmits lost data.

The LTE RLC protocol assigns a sequence number for each RLC PDU. The size of the RLC PDU is chosen when MAC indicates how much data each RLC entity may send. This principle minimizes the RLC protocol overhead and the RLC sequence number space compared to e.g. older versions of UMTS that used a fixed (small) RLC PDU size. In UMTS, the RLC PDU size was fixed and rather small. To transmit larger packets several RLC PDUs are concatenated in to one larger transport block. This meant that, since there are many RLC PDUs in each transport block, there would be many RLC headers in each transport block and this would create unnecessary overhead.

From overhead and processing point of view, it is beneficial to assign one sequence number per transport block.

### Retransmissions configurable per service

One may also have considered ARQ in MAC, i.e. that MAC would have HARQ and ARQ on top of that. This may be fine in cases the UE has only one service and whether ARQ should be applied or not can be configured based on this single service. But as soon as the UE has several different services this becomes infeasible since the UE may have traffic which needs ARQ (e.g. TCP traffic) and traffic which does not (e.g. streaming). In LTE it is therefore possible to configure whether ARQ should be used per bearer (i.e. per RLC entity). It would not be feasible to have ARQ in MAC since MAC is, in general, configured per UE, not per service.

Whether L2 retransmissions are needed depends on services and hence MAC is not a suitable place for ARQ.

# Text proposal

Below a text proposal is providing for capturing the observations in this contribution.

### 5.4.2 RLC Sublayer #### 5.4.2.X ARQ functionality Regarding the need of an ARQ protocol (on top of HARQ): - Some services, like TCP, require low IP loss rates and low end-to-end latency to benefit from high data rates offered by an underlying link. - The single-bit ACK/NACK feedback in LTE Rel-8 was not considered to be reliable enough to achieve the required low residual loss rate with reasonable overhead. - LTE RLC AM corrects residual HARQ errors reliably with relatively low overhead. - CRC protection of HARQ feedback (as in PUCCH format 4 and 5) allows the eNB to detect NACK-to-ACK errors and thereby to reduce residual MAC errors. - Some modern TCP variants cope better with packet re-ordering since they do not interpret re-ordering as packet loss anymore. - If it can be shown that enhancements such as CRC protection of UL HARQ feedback as well as Asynchronous Adaptive UL HARQ could make the HARQ protocol sufficiently reliable and resource efficient, it can be considered to no longer use RLC ARQ. Regarding placement of an ARQ protocol (on top of HARQ): - It is unwanted that L2 ARQ protocol recovers from congestion (occurring e.g. on RAN internal interfaces). - It is beneficial that L2 retransmission scheme should retransmit only lost data. - From an overhead and processing point of view, it is beneficial to assign one sequence number per transport block. - The possibility to configure RLC AM or UM for a DRB allows trading between latency/jitter and residual loss rate depending on the service requirements. This trade-off would become more difficult if RLC ARQ is substituted by a robust HARQ protocol. Further, whether L2 retransmissions are needed depends on the different services and hence MAC is not a suitable place for ARQ.

Adopt the text proposal for the TR.

# Conclusion

In section 2 we made the following observations:

Observation 1	TCP requires low IP loss rates and low end-to-end latency to benefit from high data rates offered by an underlying link.

Observation 2	The single-bit ACK/NACK feedback in LTE Rel-8 was not considered sufficiently reliable to achieve the required low residual loss rate with reasonable overhead.

Observation 3	LTE RLC AM corrects residual HARQ errors reliably with relatively low overhead but, as any protocol for ensuring in-order-delivery, at the expense of latency spikes and hence L2 memory.

Observation 4	Modern TCP variants cope better with packet re-ordering since they do not interpret re-ordering as packet loss.

Observation 5	CRC protection of HARQ feedback (as in PUCCH format 4 and 5) allows the eNB to detect NACKACK errors and thereby to reduce residual MAC errors.

Observation 6	If it can be shown that enhancements such as CRC protection of UL HARQ feedback as well as Asynchronous Adaptive UL HARQ could make the HARQ protocol sufficiently reliable and resource efficient, it can be considered to no longer use RLC ARQ.

Observation 7	The possibility to configure RLC AM or UM for a DRB allows trading between latency/jitter and residual loss rate depending on the service requirements. This trade-off would become more difficult if RLC ARQ is substituted by a robust HARQ protocol.

Observation 8	It is unwanted that L2 ARQ protocol recovers from congestion (occurring e.g. on RAN internal interfaces)

Observation 9	It is beneficial that the L2 retransmission scheme only retransmits lost data.

Observation 10	From overhead and processing point of view, it is beneficial to assign one sequence number per transport block.

Observation 11	Whether L2 retransmissions are needed depends on services and hence MAC is not a suitable place for ARQ.

Based on the discussion above we propose the following:

Proposal 1	Adopt the text proposal for the TR.

In LTE Rel-8 the processing delays were unfortunately, agreed to be long compared to the subframe duration and compared to a symbol duration. For NR there seems to be a clear desire to reduce these processing times to one or at most a few symbol durations.