3GPP TSG-RAN WG2 #95	Tdoc R2-165514

Gothenburg, Sweden, 22nd – 26th May 2016

Agenda Item:	8.10.4

Source:	Ericsson

Title:	Authentication in eMBMS

Document for:	Discussion, Decision

# Introduction

A new work item for MBMS enhancements been approved by RAN plenary [2].

The WI objectives to be specified as well as a study part as follows:

The objective of the work item is to evaluate and specify the following eMBMS enhancements for LTE:

- Specify means of using a longer cyclic prefix (e.g. greater than 33.33µs) for use in a mixed unicast/eMBMS carrier for large SFN delay spread environment (e.g. 15km or larger inter-site distance), which guarantees coexistence of the legacy and new prefixes on the same carrier, while achieving a spectral efficiency of at least 2 bps/Hz. This objective includes evaluation. (RAN1)

- Specify means of using subframes 0, 4, 5, 9 (FS1) and 0, 1, 5, 6 (FS2) for MBSFN. (RAN2, RAN1)

  - The non-MBSFN subframes for unicast can only be used as Scell

- Specify means of configuring MBSFN subframes without a unicast control region and cell-specific reference signals. (RAN1, RAN2)

Study the following:

- Support for standalone carrier with all DL subframes dedicated to MBSFN transmission and self-contained eMBMS signaling including information of SIB13, SIB15, SIB16. (RAN2)

- Support for multi-carrier eMBMS/unicast operation involving reception from one or more eMBMS cells that may be non-collocated and asynchronous with one or more cells that are simultaneously used for unicast. (RAN4, RAN2)

- Solutions where a UE can receive the TV transport service without being authenticated (RAN1, RAN2, RAN3)

No new TDD DL/UL configuration for FS2 is considered in this WI

This work will consider the outcome of the corresponding SA2 work

In this contribution, we discuss the study phase objective f) which refers to a authentication.

# Discussion

The UE interest in a service is determined by the information from the application layer, USD (start/stop time, TMGI, frequencies, SAI, etc); which is later combined with SIB15 information.

Thus, to receive a transport TV service the interaction between application instances in the UE and NW is needed.

Currently, there is not any notion of authentication in current RAN2 specs for the specific purpose of receiving a unicast or broadcast bearer. Also, it seems there are no related functions required in eNB/MCE to support UE authentication in MBMS.

For the UE side application however, looking into 33.220, this suggests the use of bootstrapping in order to enable application functions in the network (HSS, BM-Sc) and on the user side (USD) e.g. to establish shared keys. This is performed using a Generic Bootstrapping Architecture and also include procedures for the UE establishing the HTTP procedures towards BM-Sc. This is outside the domain of RAN.

Naturally, common security procedures to reach RRC connected is always needed and even before that; for instance TA update procedures may be performed should the UE move outside the TAC list.

In any case, there seems no explicit relation of the connection (cause) that explicitly maps an authenticated user to a MBMS TV service in RAN.

Another use case that may impact authentication is where Mobile Network Operators have agreed that UEs subscribed to one network shall be able to receive certain eMBMS service from another network. It may be also so that to accommodate this functionality, enhancements to counting is needed, and that those enhancements implicitly may impact the requirement of authentication.

Progress work within this WI with the base line assumption that authentication is not impacted.

Enhancements that may be pursued due to improved counting should consider solutions that allow a UE receiving a TV transport service without being authenticated.

# Conclusion

Based on the discussion in the previous section we make the following proposals:

- Progress work within this WI with the base line assumption that authentication is not impacted.

Enhancements that may be pursued due to improved counting should consider solutions that allow a UE receiving a TV transport service without being authenticated.

# References

- RP-160675, "WID eMBMS enhancements for LTE", 3GPP, RP#71, Gothenburg, Sweden, 7 - 10 March 2016

- 3GPP TR 36.743, “eMBMS enhancements for LTE”, Release 14, v0.0.1, May 2016