3GPP TSG RAN WG2 Meeting #95			R2-165586

Gothenburg, Sweden, 22nd – 26th Aug 2016

Agenda item:		8.4.2.1

Source:	Sequans Communications

Title:	Traffic Scenarios for feD2D

Document for:		Discussion and decision

# 1	Introduction

In RAN#72 a revised SID for Further Enhancements to LTE Device to Device, UE to Network Relays for IoT and Wearables [1] was agreed. The primary objective of the study is to address power efficiency for evolved Remote UEs (e.g. wearable devices), and with the following specific objectives listed:

- Study necessary LTE sidelink enhancements.

  - Introduce additional evaluation assumptions to the sidelink evaluation methodology defined in TR 36.843 focussing on analysis of wearable use cases [RAN1, RAN2].

While some of the evaluation assumptions are more L1 based and expected to be evaluated by RAN1, it was deemed beneficial to evaluate the traffic scenarios as soon as possible by RAN2, to allow taking the traffic scenario into account while discussing the feD2D Architecture.

In this document we assess the applicability of some of the traffic scenarios already used by 3GPP RAN in previous SI/WI and propose additional traffic scenarios which are specifically relevant for the wearables.

# 2	Discussion

## 2.1 New Traffic scenarios for wearables

Location Tracking

The tracker applications are of the earliest and widest adopted applications for wearables. Trackers for kids, pets or fitness could be considered.

The tracker traffic typically includes Periodic position reports which provide the current tracker position and in order to support the position calculation which is typically GPS based, there are also Periodic GPS assistance data acquisition.

The periodic positioning report could be characterized by the size (e.g. 50Bytes) and the periodicity (e.g. 1 min and up to 15 min) of the report.

The periodic GPS assistance data acquisition is typically few hundreds of DL bytes and rather infrequent e.g. once a day. The frequency and size may vary according to the positioning accuracy requirement but generally it is much less frequent than the position reporting and therefore could be neglected for traffic scenarios.

Proposal 1: Location tracking traffic scenario is used to evaluate the feD2D design.

Voice command

Voice command is another popular use case for wearables. It is a common application for smartwatch from different brands and operating systems.

Voice command traffic scenario could be characterized as unidirectional UL voice burst of a few seconds e.g. 5s for each command. Voice commands are initiated by the human user and therefore quite sparsely distributed in time with some average value of voice commands per hour e.g. 2 commands per hour.

During the voice burst we can assume a similar to VoIP traffic, e.g. AMR WB12.65 packetized as 328bit/packet and 20ms periodicity between packets or AMR WB6.6 packetized as 328bit/packet and 40ms periodicity between packets

Proposal 2: Voice command traffic scenario is used to evaluate the feD2D design.

Audio Streaming

Audio streaming is an established use case for smartphones, and some wearables (for example wireless earbuds) already apply audio streaming as well. However the application endpoint in this case would be the smartphone, the mp3 player or the smart watch that is interfacing the audio server over the cellular NW on one side and on the other side interfacing the earbuds over a BT/ A2DP audio connection or another RAT.

If the audio streaming is terminated at the smartwatch, then it would be beneficial for the smartwatch to use relayed communication whenever the smartphone is in proximity.

Future applications may also implement the audio streaming application directly on the earbuds to allow autonomous functionality. In this case it is easy to imagine that the same earbuds are used autonomously for an outdoor fitness training and using NW relay while the smartphone is in proximity.

In terms of traffic scenarios, the audio stream is relatively fixed in bit rate, ranging from slightly less than 100kbps and up to few hundreds of kbps for a very high quality audio. For this study we can use e.g. 128kbps.

Due to the moderate required bitrate, the audio streaming could be buffered at the application endpoint and some audio services already take advantage of it. The audio stream is divided into segments of a few seconds worth of audio and each segment is sent over the NW in a burst of fewer seconds with maximum throughput, which allows the UE to enter DRX (IDLE or Connected) until the next segment. An example of this behavior was shown for the Pandora service [2].

The length of each audio streaming session is relatively long on average (e.g. 30min).

Proposal 3: Audio Streaming traffic scenario is used to evaluate the feD2D design.

Table 1 below could be used to summarize the discussed traffic scenarios.

| Traffic Scenario | Statistical description | Example |
| --- | --- | --- |
| Location Tracking | Periodic UL positioning report - Size of the positioning report - Periodicity of the position report | 50Bytes every 1min or 15min |
| Voice Command | - Unidirectional UL voice burst of few seconds - Randomly distributed bursts - Voice coding during the burst | 5 sec long voice command, 2 commands per hour on average. Voice command is codded with AMR WB12.65 packetized as 328bit/packet and 20ms periodicity between packets or Voice command is codded with AMR WB6.6 packetized as 328bit/packet and 40ms periodicity between packets |
| Audio Streaming | Continuous Audio DL bit stream characterized by - Audio streaming bitrate - The bit stream is segmented into audio worth of specified seconds - Each segment is transmitted at the highest applicable T/P | 30 min audio secession at 128kbps segmented into 60sec segments. (for 60s*128kbps we get 7.5Mb/segment) |

Table 1: Traffic scenarios for wearables

Proposal 4: RAN2 to agree on values for Location Tracking, Voice Command and Audio Streaming traffic scenario.

Proposal 5: Include a table with the agreed new traffic scenarios in the TR.

## 2.2 Legacy releases Traffic Scenarios

In the following section we will review some of the traffic scenarios already used by 3GPP RAN in previous SI/WI and assess their applicability for the feD2D study.

The Rel-11 RAN2 SI on enhancements for diverse data applications was summarized in [3]. The top priority scenarios are extracted below

| A | Background Traffic | Traffic from an unattended phone with applications not in “active phase” (i.e. not including email retrieval, no IM sending etc…) |
| --- | --- | --- |
| B | IM | Instant Messaging. Includes IM background traffic. |

Table 2: Top priority scenarios in SI for eDDA.

Background Traffic

According to [3] Background traffic refers to the autonomous exchange of user plane data packets between the UE and the network, generally in the absence of a specific user interaction with the device.

In observed scenarios with a relatively small number of open background applications and processes on a typical device, the mean volume of data (aggregated uplink plus downlink) ranges from approximately 5 Bytes/s through to approximately 250 Bytes/s. and divided into two categories: “light background” or “heavier background” according to the mean data rate and mean packets per second.

-	Light Background:  Traces for which the packet inter-arrival CDFs lie mainly to the right of a straight line drawn from 25% at 100ms through to 75% at 10secs (on a plot with logarithmic X-axis and linear Y-axis)

-	Heavier Background:  Traces for which the packet inter-arrival CDFs lie mainly to the left of a line drawn from 25% at 100ms through to 75% at 10secs (on a plot with logarithmic X-axis and linear Y-axis)

RAN2 did not specify traffic scenarios for evaluation in [3] but rather gathered all of the traffic sources from different companies into the TR. An attempt to deduce a single traffic scenario for light BG and another for heavier BG is not trivial; however, a rough approximation for the packet size and an exponential distribution of the packet inter-arrival time with some mean arrival rate could be achieved by averaging over the different sources.

Instant Messaging (IM)

The mean data rate of the IM traffic typically lies within a range of 30 - 100 Bytes/s (for aggregated uplink plus downlink).

The calculated value for mean packets per sec over the relevant sources in [3] is 0.52 packets/sec with ~140 bytes per packet.

The traffic scenarios originated in [3] could be summarized in the following table.

| Traffic Scenario | Mean packet size [Bytes] |  | Mean packets per sec (exponentially distributed) |  |
| --- | --- | --- | --- | --- |
|  | DL | UL | DL | UL |
| Light Background | 120 | 60 | 0.06 | 0.08 |
| Heavier Background | 255 | 115 | 0.23 | 0.30 |
| IM | 140 |  | 0.52 |  |

Table 3: BG and IM traffic characteristics

Proposal 6: Light and heavier background traffic and IM traffic with exponential distribution of packet arrival time may be used to evaluate the feD2D design.

Proposal 7: Include a table with agreed statistical values for background and IM traffic in the TR.

# 3	Conclusion

Proposal 1: Location tracking traffic scenario is used to evaluate the feD2D design

Proposal 2: Voice command traffic scenario is used to evaluate the feD2D design

Proposal 3: Audio Streaming traffic scenario is used to evaluate the feD2D design.

Proposal 4: RAN2 to agree on values for location Tracking, Voice command and Audio Streaming traffic scenario.

Proposal 5: Include a table with the agreed new traffic scenarios in the TR

Proposal 6: Light and heavier background traffic and IM traffic with exponential distribution of packet arrival time may be used to evaluate the feD2D design

Proposal 7: Include a table with agreed statistical values for background and IM traffic in the TR

# References

- RP-161303 	Revision of SI: Further Enhancements to LTE Device to Device, UE to Network Relays for IoT and Wearables,	Qualcomm Incorporated

- R2-113065 	Evaluation Framework for RAN Enhancements for Diverse Data Applications, Intel Corporation

- 3GPP TR 36.822 enhancements for diverse data applications (V11.0.0)

- 3GPP TR 36.888 Study on provision of low-cost Machine-Type Communication (MTC) User Equipments (UEs) based on LTE (V12.0.0)