3GPP TSG RAN WG2 Meeting #95bis	R2-166083

Kaohsiung, 10th –14th October 2016

Source:	CATT

Title:	Remaining Issues for UL Bearer Support of eLWA

Agenda Item:	8.5.2

Document for:	Discussion and Decision

Introduction

At RAN2#94 and RAN2#95 meeting, eLWA UL scheduling issue was discussed and the principles for UL scheduling are agreed. However there are still some missing issues for UL bearer for eLWA. This contribution will discuss further details.

Discussion

The following sections will discuss the remaining issues for UL bearer support in Rel-14 LWA and provide candidate solutions and proposals.

- PDCP SR for UL

There are proposals [1] to introduce the PDCP SR message also in UL for flow control and retransmission purpose, i.e. the eNB could send PDCP legacy or LWA SR messages to UE periodical or on demand. However, the “flow control” issue in UL is not that significant as that in DL, thus it could be assumed that UE could autonomously handle the issue well. Besides, the cost over air interface of UL PDCP or LWA SR is not negligible. Regards to the “retransmission” function as referred by contribution [1], we think traditionally, PDCP retransmission is only utilized under limited scenarios and introducing a “systematic” retransmission function for eLWA UL at PDCP level is a bit against the original protocol stack design principles, while its performance and overhead has not yet been carefully analysed. Thus it is proposed not to introduce such PDCP SR reporting mechanism for UL.

Proposal 1: There is no need to introduce UL PDCP or LWA SR for eLWA.

Proposal 2: For UL in Rel-14 eLWA, UE should ensure that no more than half the PDCP SN space is in flight.

- DL PDCP SR transmitted over WLAN

In Rel-13 LWA, to overcome the limitations of WT implementations, UE based flow control is introduced with the new PDCP status report format used for LWA. To achieve a better flow control performance, the periodicity of UE based PDCP SR may be as frequent as 5ms. However, that kind of highly-frequent reporting will bring significant burden on Uu interface. With the support of UL split/switch bearer, such PDCP SR packets could be transferred on WLAN link to save the precious Uu interface resource. The eNB may configure UE to send back PDCP SR only on WLAN UL under certain conditions (e.g. when LTE UL is congested).

Even though the WLAN link is not that reliable as LTE link, but since PDCP SR is only used for flow control over WLAN link, and also considering the frequent reporting of the message, there should be not much issues or impacts to the flow control. Of course it could also be done by UE implementations, but to achieve a uniformed performance, direct eNB configuration should be supported.

Proposal 3: Rel-14 eLWA capable UE should deliver all PDCP or LWA SR used for LWA flow control via WLAN UL only, if requested by eNB configuration.

- Configuration of MAC Address for UE

There are some proposals [2] for eNB to configure UE with a specific WLAN MAC Address for UL traffic routings. However we don't think this is needed as the existing WLAN mechanism could take care of the routing issue well, besides it is not within 3GPP’s scope. More details could be found in our contribution [3].

Proposal 4: There is no need for eNB to signal a specific MAC address for eLWA UL routing identifications.

- System architecture and Protocol Stack

With the support of UL split/switch bearer in Rel14 LWA, the complete system architecture and protocol stack is illustrated in figure 1 and figure 2 respectively. The stage 2 figures should also be updated accordingly.

Figure 1 System Architecture for Rel-14 eLWA with UL Bearer Support

Figure 2 UP Protocol Stack for eLWA

Proposal 5: It is proposed to adopt figure 1 and figure 2 as Rel-14 eLWA system architecture and UP protocol stack baseline.

Conclusion

It is proposed to discuss and capture the following proposals at RAN2:

Proposal 1: There is no need to introduce UL PDCP or LWA SR for eLWA.

Proposal 2: For UL in Rel-14 eLWA, UE should ensure that no more than half the PDCP SN space is in flight.

Proposal 3: Rel-14 eLWA capable UE should deliver all PDCP or LWA SR used for LWA flow control via WLAN UL only, if requested by eNB configuration.

Proposal 4: There is no need for eNB to signal a specific MAC address for eLWA UL routing identification

Proposal 5: It is proposed to adopt figure 1 and figure 2 as Rel-14 eLWA system architecture and UP protocol stack baseline.

References

  - R2-162903,  Uplink transmission on WLAN for LWA, QUALCOMM CDMA Technologies

  - R2-164135, Uplink routing for enhanced LWA, MediaTek Inc., Beijing Xiaomi Mobile Software Co.

  - R2-166084, Uplink Discussion on WLAN MAC Address Configuration for UL Routing, CATT