3GPP TSG RAN WG2 Meeting #95bis	R2-166085

Kaohsiung, 10th –14th October 2016

Source:	CATT

Title:	Discussion on CP Aspect to Support 60GHz WLAN

Agenda Item:	8.5.4.1

Document for:	Discussion and Decision

Introduction

During RAN2#94 meeting it was agreed [1]: “Add support for 60Ghz band in WLAN measurements (at least RSSI) and LWA configuration (i.e. mobility set).” This contribution will discuss further details on the CP aspects.

Discussion

The following sections will discuss the potential impacts and several key issues on CP aspects to support 60GHz WLAN in Rel-14 LWA.

- Scenario Study for 60GHz WLAN

The new evolution of 60GHz WLAN (802.11ad) is designed to support extreme high data rate but with a rather limited coverage range and it probably mainly focuses on the “personal usage” scenarios or short-range device-to-device communication (like from iPad to a LCD screen), which is not like the typical use cases of operator deployed WLAN with comparative larger coverage range and user capacity. While initially the LWA WI is only targeted to support “operator deployed WLAN”. As the designed scenarios will decide the system architecture and functions, it is worthy to make some clarifications on the applicable or targeted scenarios and use cases for 60GHz WLAN.

Proposal 1: It is proposed RAN2 to discuss and make clarifications firstly on the typical use case/scenarios to support the 60GHz WLAN in Rel-14 eLWA.

- Mobility Issue

For 60GHz WLAN, the cellular size is rather small and the frequent UE handovers in WLAN will have bad impacts to LWA performance and user experience. And originally the 60GHz WLAN is only designed to support a “stationary” user scenarios.

If mobility is to be supported, during the WLAN HO procedures, the packet loss issues will become more serious than Rel-13 as the data rate is increased dramatically. However under current architecture where UE fully controls its WLAN mobility in WLAN mobility set and eNB is not aware of UE’s intra-mobility set mobility, there is no room to make further enhancement.

One natural solution is that, for HO at 60GHz WLAN, the eNB may temporarily stop/pause the transmitting over WLAN link and switch all PDCP PDUs to LTE link during HO phase, so that the packets loss could reduced. But that requires the eNB to be aware of the UE’s mobility. And two possible options are:

Option 1: UE reports its incoming 60GHz WLAN HO to eNB.

Option 2: eNB configure the 60GHz WLAN mobility set with only 1 single AP.

For option 1, new signalling and link control mechanism (like link pause, resume) needs to be defined while option 2 does not have any specification impact. And for option 2, since mobility set is always configured with just 1 single AP, UE will report measurement event W2 during each WLAN HO and thus eNB may take the corresponding actions (like to switch all traffic to LTE link).

Proposal 2: It is proposed to discuss the 2 options to address the mobility issue of 60GHz WLAN for eLWA.

Proposal 3: It is proposed to discuss whether additional link control mechanism (like pause, resume) should be introduced to improve HO performance for 60GHz WLAN in Rel-14 eLWA.

As discussed before, seamless or fast HO may not be supported of 60GHz WLAN, thus it is better not to mix the 60GHz AP with legacy APs by configuring them in the same WLAN mobility set.

Proposal 4: 60GHz APs and legacy APs should not be configure in the same WLAN mobility set.

Conclusion

It is proposed to discuss and capture the following proposals at RAN2:

Proposal 1: It is proposed RAN2 to discuss and make clarifications firstly on the typical use case/scenarios to support the 60GHz WLAN in Rel-14 eLWA.

Proposal 2: It is proposed to discuss the 2 options to address the mobility issue of 60GHz WLAN for eLWA.

Proposal 3: It is proposed to discuss whether additional link control mechanism (like pause, resume) should be introduced to improve HO performance for 60GHz WLAN in Rel-14 eLWA.

Proposal 4: 60GHz APs and legacy APs should not be configure in the same WLAN mobility set.

References

  - RAN2#94 Chairman Notes