3GPP TSG-RAN WG2 Meeting #95bis															 R2-166553

Kaohsiung, 10th – 14th October 2016

| CR-Form-v11.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.302 | CR |  | rev |  | Current version: | 14.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on TS 36.302 for V2V |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_SL_V2V-Core |  |  |  |  |  |  | Date: |  | 2016-09-30 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-14 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) |  |
| Reason for change: |  | 1 The ‘not correponsidng to V2X sidelink communication’ may lead to misunderstanding. 2 The UE supporting V2X sidelink communication also needs to transmit MasterInformationBlock-SL messages. 3 The description of Sidelink Received Signal Strength Indicator (S-RSSI) is missing. |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1. change ‘not correponsidng to V2X sidelink communication’ to ‘correponding to sidelink communication and Sidelink discovery’ 2. add ‘ or V2X sidelink communication’ to ‘The UE supporting sidelink communication transmits MasterInformationBlock-SL messages’ 3. add the description of Sidelink Received Signal Strength Indicator (S-RSSI). |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Some descriptions lead to misunderstanding. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.3.1, 8.3, 9.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

CHANGE START

6.3	Sidelink model

6.3.1	Sidelink Broadcast Channel

The physical-layer model for Sidelink Broadcast Channel transmission is characterized by a fixed pre-defined transport format. The TTI (repetition rate) of the SL-BCH correponsidng to sidelink communication and Sidelink discovery is 40ms whereas the TTI (repetition rate) of the SL-BCH corresponding to V2X sidelink communication is 160 ms, if a UE is configured to transmit on SL-BCH. The SL-BCH physical-layer model is described based on the corresponding SL-BCH physical-layer-processing chain, see Figure 6.3.1-1.

-	Higher-layer data passed to/from the physical layer

-	A single (fixed-size) transport block per TTI.

-	CRC and transport-block-error indication

-	Transport-block-error indication delivered to higher layers.

-	FEC and rate matching

-	Channel coding rate is implicitly given by the combination of transport block size, modulation scheme and resource assignment;

-	No SL-BCH Hybrid ARQ, i.e. no higher-layer control of redundancy version.

-	Data modulation

-	Fixed modulation scheme (QPSK), i.e. no higher-layer control.

-	Mapping to physical resource

-	Fixed pre-determined transport format i.e. no higher-layer control.

-	RRC controlled semi-static resource assignment.

-	Multi-antenna processing

-	Single antenna port is used.

-	Support for Hybrid-ARQ-related signalling

-	No Hybrid ARQ.

Figure 6.3.1-1: Physical-layer model for SL-BCH transmission

NEXT CHANGE

8.3	Sidelink

The table 8.3-1 describes the possible combinations of physical channels that can be sent in parallel from UE perspective in the sidelink within the same subframe. Table 8.3-2 describes the possible combinations of physical channels that can be received in parallel from UE perspective in the sidelink within the same subframe.

Table 8.3-1: Sidelink transmission

|  | Physical Channel Combination | Transport Channel Combination | Mandatory dependent on UE radio access capabilities | Comment |
| --- | --- | --- | --- | --- |
| 1 | PSDCH | SL-DCH | Mandatory for UE supporting sidelink discovery | The UE supporting sidelink discovery transmits sidelink discovery messages on the camped cell (idle) or PCell (connected). |
| 2 | PSBCH | SL-BCH | Mandatory for UE supporting sidelink communication or V2X sidelink communication | The UE supporting sidelink communication or V2X sidelink communication transmits MasterInformationBlock-SL messages in PSBCH on one preconfigured frequency. |
| 3 | PSSCH | SL-SCH | Mandatory for UE supporting sidelink communication or V2X sidelink communication | The UE supporting sidelink communication or V2X sidelink communication transmits sidelink data in PSSCH on one preconfigured frequency. |
| 4 | PSCCH | N/A | Mandatory for UE supporting sidelink communication or V2X sidelink communication | The UE supporting sidelink communication or V2X sidelink communication transmits sidelink control information in PSCCH on one preconfigured frequency. |
| NOTE: Depending on the UE capability, the UE may be able to perform simultaneous Uplink and Sidelink transmissions. If the UE is unable to perform simultaneous Uplink and Sidelink transmissions, transmissions are prioritized according to [12]. NOTE: Depending on the UE capability, the UE may be able to perform simultaneous sidelink communication transmissions (PSBCH or PSSCH or PSCCH) and sidelink discovery transmission (PSDCH). If the UE is unable to perform simultaneous transmission of sidelink communication and discovery, transmissions are prioritized according to [12]. |  |  |  |  |

Table 8.3-2: Sidelink reception

|  | Physical Channel Combination | Transport Channel Combination | Mandatory dependent on UE radio access capabilities | Comment |
| --- | --- | --- | --- | --- |
| 1 | PSDCH | SL-DCH | Mandatory for UE supporting sidelink discovery |  |
| 2 | PSBCH | SL-BCH | Mandatory for UE supporting sidelink communication or V2X sidelink communication |  |
| 3 | PSSCH | SL-SCH | Mandatory for UE supporting sidelink communication or V2X sidelink communication |  |
| 4 | PSCCH | N/A | Mandatory for UE supporting sidelink communication or V2X sidelink communication |  |
| NOTE: For sidelink communication, the UE shall be able to perform simultaneous Downlink and sidelink communication reception. For sidelink discovery, depending on the UE capability, the UE may be able to perform simultaneous Downlink and sidelink discovery receptions. If the UE is unable to perform simultaneous Downlink and sidelink discovery receptions, receptions are prioritized according to [12]. For V2X sidelink communication, the UE shall be able to perform simultaneous Downlink and V2X sidelink communication reception. NOTE: If the configured resources for reception of sidelink communication and sidelink discovery are overlapped, receptions are prioritized according to [12]. |  |  |  |  |

NEXT CHANGE

9	Measurements provided by the physical layer

9.1	Void

9.2	UE Measurements

The detailed UE measurements definition is provided in [11]:

UE measurement: Reference signal received power (RSRP): Reference signal received power (RSRP) is determined for a considered cell as the linear average over the power contributions (in [W]) of the resource elements that carry cell-specific reference signals within the considered measurement frequency bandwidth. For RSRP determination the cell-specific reference signals R0 and if available R1 according to [8] can be used.

If receiver diversity is in use by the UE, the reported value shall be in accordance with [11].

UE measurement: Sidelink Discovery Reference Signal Received Power (SD-RSRP): Sidelink Discovery Reference Signal Received Power (SD-RSRP) is defined as the linear average over the power contributions (in [W]) of the resource elements that carry demodulation reference signals associated with PSDCH for which CRC has been validated.

If receiver diversity is in use by the UE, the reported value shall be in accordance with [11].

UE measurement: Sidelink Reference Signal Received Power (S-RSRP): Sidelink Reference Signal Received Power (S-RSRP) is defined as the linear average over the power contributions (in [W]) of the resource elements that carry demodulation reference signals associated with PSBCH, within the central 6 PRBs of the applicable subframes.

If receiver diversity is in use by the UE, the reported value shall be in accordance with [11].

UE measurement: PSSCH Reference Signal Received Power (PSSCH-RSRP): PSSCH Reference Signal Received Power (PSSCH-RSRP) is defined as the linear average over the power contributions (in [W]) of the resource elements that carry demodulation reference signals associated with PSSCH, within the PRBs indicated by the associated PSCCH.

If receiver diversity is in use by the UE, the reported value shall be in accordance with [11].

UE measurement: Reference Signal Received Quality (RSRQ): Reference Signal Received Quality (RSRQ) is defined as the ratio N×RSRP / (E-UTRA carrier RSSI), where N is the number of RB’s of the E-UTRA carrier RSSI measurement bandwidth. The measurements in the numerator and denominator shall be made over the same set of resource blocks.

E-UTRA Carrier Received Signal Strength Indicator (RSSI), comprises the linear average of the total received power (in [W]) observed only in OFDM symbols containing reference symbols for antenna port 0, in the measurement bandwidth, over N number of resource blocks by the UE from all sources, including co-channel serving and non-serving cells, adjacent channel interference, thermal noise etc.

If receiver diversity is in use by the UE, the reported value shall be in accordance with [11].

UE measurement: Sidelink Received Signal Strength Indicator (S-RSSI)：Sidelink Received Signal Strength Indicator (S-RSSI) is defined as the linear average of the total received power (in [W]) observed only in the configured sub-channel by the UE.

The reference point for the S-RSSI shall be the antenna connector of the UE.

If receiver diversity is in use by the UE, the reported value shall be in accordance with [11].

CHANGE END