3GPP TSG-RAN WG2 Meeting #95bis 	R2-166574

# Kaohsiung, Taiwan, 10th-14th Oct, 2016

Title: 	Radio-aware TCP optimization

Source: 	Qualcomm Incorporated, CMCC, Huawei

Agenda item:	8.16

Document for:	Discussion and Approval

# Introduction

In the last RAN3 #92 meeting, the TCP related issue has been identified under the Context Aware Service Delivery in RAN for LTE study item [1]. The TCP E2E delay can decrease the throughput and the issue is:

The behavior of TCP assumes that network congestion is the primary cause for packet loss and high delay.  In cellular networks the bandwidth available for each UE can vary by an order of magnitude on a TTI basis due to changes in the underlying radio channel conditions. Such changes can be caused by the movement of devices or interference, as well as changes in system load due to bursty traffic sources or when other UEs enter and leave the network. TCP has difficulties adapting to these rapidly varying conditions.

If the E2E delay increases, the TCP RTT increases and the TCP throughput may decrease, which may impact the user experience.

In this contribution, we analyze the TCP issues in wireless network and propose one solution to solve these issues. This solution helps the TCP layer to adjust the congestion window according to the UE reported predicted information, and TCP protocol still works on the congestion control mechanism.

# Current TCP Issues

TCP uses packet loss or delay spike as congestion signal to control TCP to L3/L2 data delivery speed. However, these congestion signals may mislead TCP. Packet loss in wireless link may be caused by random interference or decoding error instead of congestion. Long RTT may be caused by RLC/HARQ retransmission, multi-user scheduling or handover instead of congestion.

On the other hand, TCP congestion control is used to control the TCP to L2/L3 data delivery speed. Too fast delivery may lead to congestion and packet loss. Too slow delivery may lead to slow service provisioning and overprovisioning. The available bandwidth can vary significantly due to the changes in the underlying radio channel conditions, and TCP congestion control has difficulties in adapting to these rapidly varying conditions. So current TCP congestion control may result in a slow adaptation to radio channel changes, which leads to low throughput and long delay.

# Radio-Aware TCP Optimization

As analyzed above, TCP congestion control is based on the congestion signals, such as packet loss or TCP RTT delays. However, packet loss in wireless link may also be caused by random interference, instead of congestion. Long TCP RTT latency may be caused by handover, multi-user scheduling or low layer retransmission. So the inaccurate congestion signals lead to inaccurate TCP congestion control and finally lead to low TCP throughput, i.e. low radio link capability utilization and long TCP transmission delay.

Figure 1 gives the normalized TCP goodput and TCP RTT vs. normalized bandwidth-delay product (BDP). The normalized TCP goodput is defined as received TCP goodput scaled by LTE link throughput. It is actually the LTE link utilization rate. The normalized TCP RTT is the average TCP RTT scaled by ideal LTE RTT (5ms). Therefore, the upper boundary of normalized goodput is 1 and lower boundary of normalized delay is 1. The normalized BDP is the layer 2 buffer scaled by actual bandwidth-delay product, which is defined as LTE throughput times LTE RTT. This figure illustrates the normalized TCP RTT increases and approaches to 1 with increased buffer. This is because the larger buffer absorbs the wireless channel fluctuation and congestion control error. However, normalized TCP delay increases with the increased buffer.

Figure 1 Normalized TCP goodput & TCP RTT vs. nomalized BDP

Observation 1: TCP throughput and LTE link utilization by TCP could be improved by enlarging the buffer size. However, large buffer size increases the TCP RTT.

Typical TCP protocol transmission has two major phases: slow start phase and congestion avoidance phase. In slow start phase, current TCP exponentially increases congestion window (cwnd), and leads to low TCP throughput at the beginning.

Observation 2: Bandwidth probe in slow start phase leads to low TCP throughput and long delay.

While in congestion avoidance phase, current TCP increases cwnd until packet loss or packet error, and leads to unnecessary TCP retransmission and layer 2 buffer level fluctuation. Figure 2 is an example of TCP congestion control including slow start phase and congestion avoidance phase. Current TCP adjusts the congestion window size only based on the congestion signal in the two phases. The fluctuated wireless bandwidth causes the mismatch of layer 2 buffer and TCP congestion window, and also limits the TCP goodput.

Observation 3: In congestion avoidance phase, cwnd increase leads to packet loss.

Figure 2 TCP congestion control example with slow start and congestion avoidance phase

Ideally, TCP congestion control shall control layer 2 buffer at an optimal level. That means the TCP congestion control should minimize the layer 2 buffer level to reduce queuing delay and also minimize the layer 2 buffer empty probability to improve TCP throughput.

Figure 3 Layer 2 buffer model as leaky bucket model

Layer 2 buffer can be modeled as a leaky bucket model in Figure 3, and the buffer level is impacted by the incoming packets from upper layer and the air interface throughput which fluctuates with the radio channel status, cell load, the other UEs’ channel status, and the mobility. If more information can assist the TCP sender, the layer 2 buffer could be expected to be controlled better to adapt to the fluctuated wireless environment.

Several network based throughput prediction schemes have been studied. In these solutions, the eNB estimates and delivers the mobile throughput guidance information or radio network information of the target UE to TCP sender. The TCP sender uses these information to adjust the sending rate to adapt to the radio channel. But these methods will bring the extra work load of DPI to eNB, and these solutions also need to setup a separate connection (such as TCP proxy) between eNB and TCP sender.

Radio-aware TCP congestion control mechanism adjusts its congestion window and delivers packets to lower layer (i.e. layer 2) according to the assisted layer 2 information, which may consider the achievable data rate or layer 2 buffer level or ECN message. In this solution, the UE Achievable Data Rate (ADR) could be calculated in UE modem based on the UE CQI information, and network loading message which may broadcast from eNB. Due to the unavoidable error in achievable data rate prediction, the estimated layer 2 buffer level gradually drifts from the actual layer 2 buffer level. So the actual layer 2 buffer level should be sent to TCP sender to correct the drift.

The UE based TCP throughput prediction with eNodeB assistance method not only estimates the available throughput, but also predict the future UE assignment bandwidth from the RAN network assist information, i.e. cell loading. The predict throughput, ADR, can be expressed as:

ADR=W∙f(CQI)∙load,

Where W is the system bandwidth which can be acquired from SIB1 in LTE system; the UE spectral efficiency can looks up the CQI table in [2]. The load information gets from the eNB potential resource usage for this client, and the RAN need broadcast the cell loading information to assist all the served UE to calculate load information.

UE based throughput prediction works as below in Figure 4:

- The RAN network shares its cell level information to UE, such as resource status etc.

- UE predicts its future available throughout according to the cell information.

- The predicted throughput is proactively signalled to server from uplink, i.e. TCP sender.

Figure 4 UE based throughput prediction procedure

It should be noted that the filter mechanism should be considered to avoid large fluctuation of predicated ADR.

By using the predicted UE throughput, the recommended TCP congestion window size can be re-calculated together with smoothed TCP RTT. In Figure 3 the calculated throughput can roughly predict the required buffer level, and the actual layer 2 buffer level will further accurately adjust the TCP congestion window. The proposed TCP congestion window can be defined as,

Qthr-Qlent+R(t)∙RTT,

where Qthr is the predefined threshold of layer 2 buffer queue length, which is determined per the delay requirement of the TCP flow. Qlent is the sampled layer 2 buffer queue length. The parameters R(t) and RTT are the predicted UE throughput, i.e. ADR and predicted TCP RTT, respectively. The TCP RTT is predicted per history information and potentially radio status information, e.g. handover, RLF, UL grant, and cell load.

Figure 5 introduces the procedure of layer 2 information delivery in radio-aware TCP solution in downlink path. At the beginning, eNB sends achievable data rate assistance information and layer 2 buffer information to the served UEs. Then the UE predicts achievable data rate in modem based on the assistance information and measured radio information, i.e. CQI. After that, the low layer information (i.e. achievable data rate) is sent to TCP sender. Finally the TCP updates the congestion window (cwnd) according to the layer 2 queue information and UE predicted achieve data rate. The TCP server sends packets to UE with the delivery speed controlled by the updated cwnd.

Figure 5 Layer 2 information delivery in downlink TCP

The performance is evaluated under the general assumption in appendix, and several traffic modellings are implemented in the simulation. The burst traffic with short flow (i.e. one 0.5MB burst traffic) or long flow (i.e. 2MB burst packet with configurable interval during 60 minutes simulation) refers to 3GPP FTP modeling [3], and the full buffer is also considered to evaluate the TCP performance.

Figure 6 shows the performance benefits for radio-aware TCP solution. For a short traffic flow with single 0.5MB burst size, the TCP latency can be reduced by about 66% than the original TCP. The reason is that radio-aware TCP design can faster download small web page than original TCP in slow start phase, while blind additive increase of cwnd in original TCP causes performance degradation. Besides, the low layer rate prediction also helps to probe the appropriate available bandwidth.

Figure 6 TCP performance enhancement for radio-aware TCP solution for short flow

Table 1 gives the enhanced TCP performance results in long flow traffic. The typical layer 2 buffer size is assumed based on the 1 ~ 1.5x bandwidth-delay product, i.e. BDP. If only the R(t) * RTT is considered in radio-aware TCP solution, nearly 10% performance gain can be acquired. More than 10% performance enhancement is achieved when radio-aware TCP solution (i.e. ADR and layer 2 buffer level) is adopted in the system.

Table 1 Enhanced TCP performance for radio-aware TCP solution for long flow

| L2 Buffer Size (Bytes) | Normal TCP (Mbps) | Radio aware TCP: R(t)*RTT (Mbps) | Radio aware TCP w/ L2 Q info (Mbps) | Gain |
| --- | --- | --- | --- | --- |
| 45KB | 14.5 | 15.9 | 16.6 | 14.5% |
| 60KB | 16.57 | 17.33 | 18.59 | 12.1% |

Observation 4: Radio-aware TCP solution provides significant performance gain compared with the original.

Observation 5: UE based throughput prediction and layer 2 buffer level can support TCP to adjust the congestion window more accurately.

Observation 6: Network assistance information can help UE to predict the accurate throughput.

Proposal 1: Network to broadcast assistance information for throughput prediction.

# Network assisted radio-aware TCP solution

Figure 5 Network assisted radio-aware TCP solution

- At reception of optimization request, CN may perform UE eligibility check. “UE eligibility check” is necessary in case the optimization is performed for only some kind of user subscription and differentiate charging may be performed. After UE eligibility check, CN sends TCP optimization response to UE to accept or reject the TCP optimization request.

- Upon decision to perform TCP optimisation, the CN will request eNB to report achievable data rate (ADR), for this UE.

In this step, the CN can also configure the reporting criteria and other related information.

During handover procedure, CN may configure the target eNB to report optimization parameters by CN request. No more eligibility check is needed.

- According to the ADR assistance info report configuration, eNB reports the required ADR assistance info to CN.

This can be reported periodically or event triggered by CN configuration (e.g. ADR drops below a certain threshold).

- CN sends ADR assistance info to TCP server; then TCP server set/update its congestion window accordingly.

# Conclusion

Based on above discussion, we have following observations and proposals.

Observation 1: TCP throughput and LTE link utilization by TCP could be improved by enlarging the buffer size. However, large buffer size increases the TCP RTT.

Observation 2: Bandwidth probe in slow start phase leads to low TCP throughput and long delay.

Observation 3: In congestion avoidance phase, cwnd increase leads to packet loss.

Observation 4: Radio-aware TCP solution provides significant performance gain compared with the original.

Observation 5: UE based throughput prediction and layer 2 buffer level can support TCP to adjust the congestion window more accurately.

Observation 6: Network assistance information can help UE to predict the accurate throughput.

Proposal 1: Network to broadcast assistance information for throughput prediction.

# References

- R3-161473, “Update of Draft TR 36.933”, CMCC (Rapporteur).

- 3GPP TS36.213, “Evolved Universal Terrestrial Radio Access (E-UTRA); Physical layer procedures”, 3GPP, v13.2.0.

- 3GPP TR36.814, “Further advancements for E-UTRA physical layer aspects”, 3GPP, v9.0.0.

# Appendix

Below table gives the key simulation parameters.

| Parameter | Value |
| --- | --- |
| Bandwidth | 20MHz |
| MIMO configuration | MIMO TM2 |
| BS Tx power | 43dBm |
| MAC scheduler | Proportional fair |
| Large scale channel model | Cost231 |
| Fast fading | EVA channel |
| UE Speed | 60kmh |
| HARQ | Yes |
| RLC | AM |