3GPP TSG-RAN WG2 #95bis	Tdoc R2-166775

# Kaohsiung, Taiwan, 10th – 14th October 2016

Agenda Item:	9.2.2.3

Source:	Ericsson

Title:	RRC configuration in LTE-NR tight-interworking

Document for:	Discussion, Decision

# Introduction

In RAN2#95, some aspects are remained to be discussed. One of these issues is “Whether NR RRC messages generated by NR node are the final RRC messages?”.

In this contribution, we will be seeking answers to this open issue; as well as present and propose how RRC configuration should be handled overall for LTE-NR tight-interworking.

# Discussion

In this section, we will discuss reasons “why the final RRC message should be transmitted only from the master node” and present proposals accordingly.

The agreement from RAN2#94 “the UE has a single RRC state based on the master” already motivates that the final RRC message should be transmitted only from the master node. Yet, there are also other reasons behind this choice, which need to be brought more attention to. First of all, if (final) RRC messages could be generated by both master and secondary nodes and even if master node message could only be sent via master node Uu and secondary node messages via secondary node Uu, it would be difficult to process them in the correct order on the UE side as concluded in LTE DC discussions in Rel-12 [1][2]. Moreover, if final RRC messages generated by master and secondary nodes are supposed to be transmittable via any of the two paths, it becomes even more difficult to ensure the correct order. Whether or not processing multiple final RRC message in the wrong order leads to problems, such as connection failure, would need to be investigated on a case by case basis. Furthermore, it also becomes difficult to have a joint error handling for NR and LTE inter-working.

It becomes difficult to ensure the correct order if final RRC messages generated by master and secondary nodes are supposed to be transmittable via different paths.

Whether or not processing multiple final RRC messages in the wrong order leads to problems, such as connection failure, would need to be investigated on a case by case basis.

If final RRC messages generated by master and secondary nodes are supposed to be transmittable via different paths, it also becomes difficult to have a joint error handling for NR and LTE inter-working.

If secondary node could send an RRC message (e.g., only affecting the “SCG” or corresponding NR access configuration) directly to the UE, this may in some cases reduce the latency. However, this is only the case when there is a significant latency between the entities generating LTE and NR configuration. If the latency is negligible e.g., compared to the RRC processing in the UE or compared to the transmission delay, the benefit diminishes. Secondly, the latency benefit disappears when “SCG” (or corresponding NR access) reconfiguration may have an impact on the master node configuration. For instance, a concurrent master node reconfiguration may lead to a race condition or to an invalid configuration when exceeding the UE capabilities. To avoid that, the secondary node would need to handshake with master node anyway before sending the message to the UE. For all those cases, a latency enhancement may not be expected as concluded in LTE DC Rel-12 discussions [1][2].

Latency enhancement may not be expected if secondary node would need to handshake with master node before sending a message to UE or if the backhaul latency is negligible compared to the Uu transmission time and the UE RRC processing time.

Therefore, instead of generating final RRC messages both from master and secondary nodes, we propose to reuse the LTE DC approach where the “SCG” configuration is generated by the RRC entity in the secondary node, transported to the RRC entity in the master node within a container, and transmitted by a final RRC message from the master node to the UE, as shown in the signaling diagram in Figure 1.

Either NR or LTE node, which is of the master node role, should be generating the final RRC message, which includes both LTE and NR configurations, to ensure in-order delivery and joint error handling of LTE and NR configurations.

“SCG” or corresponding NR access configuration are generated by the RRC entity in secondary node, transported to the RRC entity in master node within a container, and transmitted by a final RRC message from master node to UE.

Figure 1: RRC signaling assuming an interworking scenario where the master node is an LTE eNB and the secondary node is an NR gNB.

# Conclusion

Based on the discussion in section 2 we have the following observations and proposals:

Observation 1	It becomes difficult to ensure the correct order if final RRC messages generated by master and secondary nodes are supposed to be transmittable via different paths.

Observation 2	Whether or not processing multiple final RRC messages in the wrong order leads to problems, such as connection failure, would need to be investigated on a case by case basis.

Observation 3	If final RRC messages generated by master and secondary nodes are supposed to be transmittable via different paths, it also becomes difficult to have a joint error handling for NR and LTE inter-working.

Observation 4	Latency enhancement may not be expected if secondary node would need to handshake with master node before sending a message to UE or if the backhaul latency is negligible compared to the Uu transmission time and the UE RRC processing time.

Proposal 1	Either NR or LTE node, which is of the master node role, should be generating the final RRC message, which includes both LTE and NR configurations, to ensure in-order delivery and joint error handling of LTE and NR configurations.

Proposal 2	“SCG” or corresponding NR access configuration are generated by the RRC entity in secondary node, transported to the RRC entity in master node within a container, and transmitted by a final RRC message from master node to UE.

# References

R2-131673, Summary of email discussion [81bis#18][LTE/SCE-HL] CP protocol and architecture alternatives, Ericsson (Rapporteur), RAN2#82, May 20 - 24, 2013.

R2-132691, Summary of email discussion [82#17][LTE/SCE] Control plane aspects, Ericsson (Rapporteur), RAN2#83, August 19 - 23, 2013.