3GPP TSG-RAN WG2 #95bis	Tdoc R2-166801

# Kaohsiung, Taiwan, 10th – 14th October 2016

Agenda Item:	9.2.1.2

Source:	Ericsson

Title:	QoS framework for NR

Document for:	Discussion, Decision

# Introduction

RAN2-95 discussed the basic principles of the NR QoS framework and reached the following agreements:

Agreements

1	For DL for a non-GBR flow, the eNB sees an indication over NG-u and based on the indication the eNB maps the packet to a DRB of an appropriate QoS. RAN2 understanding of SA2 agreements is that eNB has a QoS profile associated with the indication.

FFS whether there is a requirement for every different QoS indication to be mapped to a different radio bearer.

2	Functionality is required to differentiate flows from different PDN-connections over the radio interface (e.g. by using separate DRBs or by an explicit indication in a header)

3	For DL, the eNB establishes DRBs for the UE taking the QoS profiles in to account.

FFS how the DRB is established in the first packet is an UL packet.

In this document we address the remaining open issues taking into account recent discussions and agreements in SA2.

# Discussion

## Legacy QoS mapping/filtering

Figure 1 depicts the mapping of data (IP) packets to EPS bearers and to S1- and Data Radio Bearers as it is done in E-UTRA/EPC. The DL and UL packet filters (TFTs) determine (based on source and destination IP Addresses and Port Numbers) in which bearer to carry each packet.

Figure 1: QoS Mapping in E-UTRA

## QoS mapping/filtering in the NextGen CN

Figure 2 shows the changes expected for NR and the next generation CN according to discussions in SA2. Instead of mapping IP packets to EPS bearers, the Next Generation core network is supposed to group packets into flows. We expect that this may be done by packet filters similar to the TFTs defined in EPS, i.e., the next generation CN and the UE could ensure that all packets to and from, for example, the same IP/Port number tuple belong to a “flow”. On their way through the transport network each packet may be marked with some sort of “Flow ID”. In Figure 2 we denote these filters as “NAS filters”. Like in E-UTRA/EPC, the core network determines and applies the downlink filters locally and it may configure the UE by means of NAS signalling with a set of uplink “NAS filters”. Besides these explicitly configured uplink TFTs, SA2 discussed also a reflective QoS function. As a basic principle, the UE detects which DL packets appear in which DL flow and creates TFTs (filters) that identify corresponding uplink packets and map those to the same flow in UL direction. Hence no explicit NAS signalling to the UE is needed for configuring the “NAS” filters, which has large potential to decrease the control signalling for services where the filter criteria are subject to frequent changes. Explicitly adding and removing filters on port numbers and IP addresses could be avoided by such a mechanism.

Figure 2: Mapping of Packets to Flows to Radio Bearers

Similarly to the PDN Connections in EPS there NextGen CN will support multiple PDU sessions. Each PDU session is mapped to a separate transport network bearer in order to separate them even if the contained packets have an overlapping IP address range. Also the UE must be able to determine which IP packet belongs to which PDN session in order to route packets correctly. This may also need to be taken into account in the reflective QoS filtering.

## QoS mapping/filtering in NR

While SA2 intends to distinguish and mark many different flows, RAN2 already agreed to handle different priorities and QoS requirements by means of different DRBs whereas all packets within a DRB experience the same QoS treatment. In order to keep UE and RAN complexity to a reasonable level, the number of DRBs must be kept reasonably low. Considering that RAN2 agreed to use the LTE UP stack as baseline for the NR design, we also consider a similar number of DRBs as baseline.

In order to keep UE and RAN complexity to a reasonable level, the number of DRBs must be kept reasonably low.

Unless SA2 decides to limit the number of flows to such number, it must hence be possible to map several flows onto a common data radio bearer.

As discussed at RAN2-95, the eNB determines e.g. based on the Flow ID the desired QoS profile for each flow. Furthermore, “the eNB establishes DRBs for the UE taking the QoS profiles in to account”. If the eNB intends to map packets with different QoS requirements onto the same bearer, those packets will (as agreed by RAN2) obtain the same QoS treatment. To ensure that all packets experience the desired quality level, the eNB target the most demanding QoS requirements of all packets mapped to that bearer.

The eNB may decide to map packets of multiple flows and with different QoS requirements onto a common DRB provided that the QoS requirements of all packets (i.e., also the most demanding) can be fulfilled.

Note that this does not intend to change RAN2’s early agreement that “all packets within a DRB experience the same QoS treatment”.

During RAN2-95 some concerns were raised that different Flows may be associated with different ARP (Allocation and Retention Priority) values and that those priorities could not be handled properly when flows with similar QoS requirements but different ARP were mapped to the same DRB. We would like to point out that the DRB determines primarily the packet forwarding treatment, i.e., the targeted packet delay budget and loss rates. Flows mapped to one DRB will observe the same treatment with this respect. The ARP determines the priority with which the RAN should admit or remove a flow in case of overload. However, even if flows with different ARP are mapped to a common DRB, the RAN may distinguish the flows in terms of their ARP. It should be noted that this is equivalent to the EPS QoS concept where an ARP value is associated with each EPS bearer.

Upon addition of a new Flow the eNB determines, based on the Flow’s ARP value, whether or not to admit the Flow.

When the eNB is no longer able to fulfil negotiated QoS contracts of one or more DRBs, it should remove Flows in accordance with their ARP.

If the RAN decides to reject a flow in accordance with its ARP, there need to means for ensuring that the corresponding traffic of that flow is barred as well. Whether to achieve this on AS or NAS level should be discussed further among RAN2 and SA2.

As explained in section 2.2, SA2 intends to rely primarily on “reflective filters” and we believe that this concept should also be available on AS level as shown in Figure 2. Determining reflective QoS filters necessitates that the eNB includes the flow IDs into the DL packets (e.g. into the PDCP header) so that the UE can determine the “AS filters” mapping from flow ID to UL DRB. Furthermore, the NAS layer uses this information to create uplink “NAS filters” that map IP packets to flows.

It may also be desirable to include the flow IDs in the UL PDCP header so that the RAN can mark UL packets with the correct DSCP for their way through the transport network. Secondly, it allows the network to verify that the UE mapped packets to flow IDs correctly.

The reflective QoS concept intends to reduce the signalling overhead but the need to include the flow ID in each PDCP header increases the user plane protocol overhead. The relative overhead is marginal for large IP packets but could be considered significant for services such as VoIP.

The UE determines “reflective QoS filters” based on the downlink packets received within a DRB and applies those filters for mapping uplink packets or “flows” to DRBs.

For the downlink direction the eNB determines and applies the “AS” filter by itself. For many scenarios the above-mentioned reflective QoS filters is a signalling efficient means for uplink QoS mapping. A drawback of that scheme is however, that uplink packets arriving prior to the first downlink packet of the flow will not be mapped as intended by the RAN. In order to guarantee correct mapping also for those initial packets, the eNB should be able to configure the UE with appropriate “flow-to-DRB filters”. It should also be possible to define a dedicated mapping of flows to DRBs even though the NAS level applies reflective filters only.

The eNB may configure the UE with uplink AS filters that determine the mapping of uplink “flows” to data radio bearers (DRB). These filters are configured by means of RRC. An explicitly configured AS filter overrides any reflective filter.

In the EPS QoS framework, an uplink packet that does not match any packet filter should be transmitted on the default bearer. We think that the corresponding behaviour should be ensured in NR/NG CN.

If an incoming UL packet matches neither a configured nor a reflective uplink filter, the UE shall map that packet to the default DRB.

At RAN2-95 it was also left “FFS whether there is a requirement for GBR flows and non GBR flows to be mapped to different DRBs”. From eNB scheduling point of view there is no real difference between GBR and non-GBR bearers. The scheduler aims to fulfil the QoS contract, i.e., the packet delay budget and the required loss rate in both cases. GBR bearers (or flows) differ from Non-GBR bearers (or flows) primarily in the way they are requested and established: For a Non-GBR bearer the eNB cannot and does not need to make any assumptions on the anticipated service data rate and it does not need to make any commitments with that respect. Hence, it does not need to verify whether or not it expects to have sufficient radio resources to serve a non-GBR bearer. On the other hand, when the CN requests establishment of a GBR bearer (or flow), it also requests a “Guaranteed Bit Rate”. The RAN needs to assess whether it will (likely) be able to fulfil this request and if so, admit the request. This is known as Admission Control. Once the GBR flow has been admitted, the eNB will serve it in accordance with the negotiated QoS contract up to the guaranteed bit rate. Whether the eNB does this by means of a dedicated DRB or whether it maps the GBR traffic onto another existing DRB does not matter from CN/NSA perspective provided that it fulfils the negotiated QoS contract.

Whether the eNB maps an admitted GBR flow to a separate DRB or onto another existing DRB is up to RAN implementation provided that the RAN fulfils the QoS contract negotiated with the CN.

# Conclusion

Based on the discussion in section 2 we propose the following:

Proposal 1	The eNB may decide to map packets of multiple flows and with different QoS requirements onto a common DRB provided that the QoS requirements of all packets (i.e., also the most demanding) can be fulfilled.

Proposal 2	Upon addition of a new Flow the eNB determines, based on the Flow’s ARP value, whether or not to admit the Flow.

Proposal 3	When the eNB is no longer able to fulfil negotiated QoS contracts of one or more DRBs, it should remove Flows in accordance with their ARP.

Proposal 4	The UE determines “reflective QoS filters” based on the downlink packets received within a DRB and applies those filters for mapping uplink packets or “flows” to DRBs.

Proposal 5	The eNB may configure the UE with uplink AS filters that determine the mapping of uplink “flows” to data radio bearers (DRB). These filters are configured by means of RRC. An explicitly configured AS filter overrides any reflective filter.

Proposal 6	If an incoming UL packet matches neither a configured nor a reflective uplink filter, the UE shall map that packet to the default DRB.

Proposal 7	Whether the eNB maps an admitted GBR flow to a separate DRB or onto another existing DRB is up to RAN implementation provided that the RAN fulfils the QoS contract negotiated with the CN.

# References

R2-165336, “QoS framework for NR”, Ericsson, Gothenburg, Sweden, 22nd – 26th August 2016