3GPP TSG-RAN WG2 Meeting #95bis 	R2-166993

# Kaohsiung, 10 – 14 Oct 2016

| CR-Form-v11.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 25.331 | CR |  | rev | - | Current version: | 11.17.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | LTE CA capability inquiry. |
| --- | --- |
| Source to WG: | Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI11 |  |  |  |  |  | Date: |  |  | 2016-09-30 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-11 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) |  |
| Reason for change: |  | CR5754 was agreed to allow the network to ask up to 16 LTE bands for the CA capability. The intention was to reduce the message size of RRC CONNECTION SETUP COMPLETE and UE CAPABILITY ENQUIRY. However, even with 16 LTE bands, with 2 DL CA, 3 DL CA, and more DL CA capabilities, the message size is still large. For RRC CONNECTION SETUP COMPLETE, the large message size can severely affect call setup time. In some cases, a CS call setup may be failure. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add an optional IE to allow the network only ask LTE non-CA band capability. Impact analysis Impacted functionality: LTE CA band combination capability reporting. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The UE sends all its supported LTE carrier aggregation combinations to the network, which may result in large Capability IE leading to SRB overload and possible call failures. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.6.3.12, 10.3.3.2, 11.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

****************************************** TEXT START *****************************************

#### 8.6.3.12	Capability Update Requirement

If the IE "Capability Update Requirement" is included the UE shall:

1>	if the IE "UE radio access FDD capability update requirement" has the value TRUE:

2>	if the UE supports FDD mode:

3>	store its UTRA FDD capabilities and its UTRA capabilities common to FDD and TDD in the IE "UE radio access capability" and the IE "UE radio access capability extension" in variable UE_CAPABILITY_REQUESTED as specified below:

4>	if the UE supports any radio access capability included in IE "UE radio access capability extension" that is not included in IE "UE radio access capability":

NOTE:	This is valid e.g. for UE that supports multiple UTRA FDD Bands, UE that supports a single UTRA FDD Band different from Band I [21] or UE that supports E-UTRA.

5>	store the IE "UE radio access capability", excluding IEs "RF capability FDD" and "Measurement capability";

5>	store the IE "UE radio access capability extension", including the IEs "RF capability FDD extension", the "Measurement capability extension", the "Additional Secondary Cells"  and the "Non-contiguous multi-cell" associated with each supported UTRA FDD frequency band indicated in the IE "Frequency band".

4>	else:

5>	store the IE "UE radio access capability", including the IEs "RF capability FDD" and "Measurement capability" associated with the Band I [21].

1>	if the IE "UE radio access 3.84 Mcps TDD capability update requirement" has the value TRUE:

2>	if the UE supports 3.84 Mcps TDD mode:

3>	store its UTRAN-specific 3.84 Mcps TDD capabilities and its UTRAN–specific capabilities common to FDD and TDD in the variable UE_CAPABILITY_REQUESTED.

1>	if the IE "UE radio access 7.68 Mcps TDD capability update requirement" has the value TRUE:

2>	if the UE supports 7.68 Mcps TDD mode:

3>	store its UTRAN-specific 7.68 Mcps TDD capabilities and its UTRAN–specific capabilities common to FDD and TDD in the variable UE_CAPABILITY_REQUESTED.

1>	if the IE "UE radio access 1.28 Mcps TDD capability update requirement" has the value TRUE:

2>	if the UE supports 1.28 Mcps TDD mode:

3>	store its UTRAN-specific 1.28 Mcps TDD capabilities and its UTRAN–specific capabilities common to FDD and TDD in the variable UE_CAPABILITY_REQUESTED;

3>	if the UE supports E-UTRA:

4>	store the IE "UE radio access capability", including "Measurement capability TDD" associated with each supported E-UTRA band.

1>	if the IE "System specific capability update requirement list" is present:

2>	for each of the RAT requested in the IE "UE system specific capability":

3>	if the UE supports the listed RAT:

4>	include its inter-RAT radio access capabilities for the listed RAT in the IE "UE system specific capability" from the variable UE_CAPABILITY_REQUESTED;

4>	if the listed RAT is GSM and PS Handover to GPRS is supported:

5>	include the IE ''MS Radio Access Capability'' in the variable UE_CAPABILITY_REQUESTED.

4> if the listed RAT is E-UTRA:

5>	if the IE "Requested E-UTRA Frequency Band list" is present and supported by the UE:

6>	include in IE "UE system specific capability" in the variable UE_CAPABILITY_REQUESTED all supported E-UTRA bands according to sub-clause 5.6.3.3 in [67];

6>	include in IE "UE system specific capability" in the variable UE_CAPABILITY_REQUESTED all supported non-CA bands according to sub-clause 5.6.3.3 in [67];

6>	include in IE "UE system specific capability" in the variable UE_CAPABILITY_REQUESTED CA band combinations according to sub-clause 5.6.3.3 in [67] as if the UE supports only those E-UTRA frequency bands signaled in IE "Requested E-UTRA Frequency Band list";

6>	include in IE "UE system specific capability" in the variable UE_CAPABILITY_REQUESTED other E-UTRA capabilities.

4>	if the listed RAT is E-UTRA no carrier aggregation:

5>	do not include SupportedBandCombination in the UE-EUTRA-Capability Information element as defined in [67] in order to create the UE-EUTRA-Capability Information element.

If the IE "Capability update requirement" is not present, the UE shall:

1>	assume the default values as specified in subclause 10.3.3.2 and act in accordance with the above.

#### 10.3.3.2	Capability Update Requirement

This IE indicates to the UE which specific capabilities to transfer to the network.

| Information Element/Group name | Need | Multi | Type and reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| UE radio access FDD capability update requirement | MP |  | Boolean | TRUE indicates update required |  |
| UE radio access 3.84 Mcps TDD capability update requirement | MP |  | Boolean | TRUE indicates update required | Name changed in REL-4 |
| UE radio access 7.68 Mcps TDD capability update requirement | MP |  | Boolean | TRUE indicates update required | REL-7 |
| UE radio access 1.28 Mcps TDD capability update requirement | MP |  | Boolean | TRUE indicates update required | REL-4 |
| System specific capability update requirement list | OP | 1 to <maxSystemCapability> |  | In this version, a maximum size of 4 of the list shall be applied and any items after the 4th item in the list shall be ignored. |  |
| >System specific capability update requirement | MP |  | Enumerated (GSM | Four spare values needed. |  |
|  |  |  | , GERAN Iu |  | REL-5 |
|  |  |  | , E-UTRA |  | REL-8 |
|  |  |  | , E-UTRA no carrier aggregation) |  |  |
|  |  |  |  |  | REL-11 |
| Requested E-UTRA Frequency Band list | CV-BCHopt | 1 to 16 |  |  | REL-10 |
| >E-UTRA Frequency band | MP |  | Integer (1..256) | As defined in [64]. The value of 64 is reserved and shall not be used | REL-10 |

| Condition | Explanation |
| --- | --- |
| BCHopt | This IE is not needed when sent in SYSTEM INFORMATION. Otherwise, the IE is optional |

Default value is:

"UE radio capability FDD update requirement" = FALSE

"UE radio capability 3.84 Mcps TDD update requirement" = FALSE

"UE radio capability 7.68 Mcps TDD update requirement" = FALSE

"UE radio capability 1.28 Mcps TDD update requirement" = FALSE

"System specific capability update requirement" not present.

"Requested E-UTRA Frequency Band list" not present.

## 11.2	PDU definitions

--**************************************************************

--

-- TABULAR: The message type and integrity check info are not

-- visible in this module as they are defined in the class module.

-- Also, all FDD/TDD specific choices have the FDD option first

-- and TDD second, just for consistency.

--

--**************************************************************

PDU-definitions DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

--**************************************************************

--

-- IE parameter types from other modules

--

--**************************************************************

IMPORTS

-- Core Network IEs :

CN-DomainIdentity,

CN-InformationInfo,

CN-InformationInfo-r6,

CN-InformationInfoFull,

NAS-Message,

PagingRecordTypeID,

PLMN-Identity,

-- UTRAN Mobility IEs :

CellIdentity,

CellIdentity-PerRL-List,

DedicatedPriorityInformation,

DedicatedPriorityInformation-r11,

URA-Identity,

-- User Equipment IEs :

AccessStratumReleaseIndicator,

ActivationTime,

C-RNTI,

CapabilityUpdateRequirement,

CapabilityUpdateRequirement-r4,

CapabilityUpdateRequirement-r4-ext,

CapabilityUpdateRequirement-r5,

CapabilityUpdateRequirement-v770ext,

CapabilityUpdateRequirement-v860ext,

CapabilityUpdateRequirement-vaj0ext,

CapabilityUpdateRequirement-r7,

CapabilityUpdateRequirement-r8,

CapabilityUpdateRequirement-vbi0ext,

CellUpdateCause,

-- ***************************************************

--

-- RRC CONNECTION SETUP

--

-- ***************************************************

RRCConnectionSetup ::= CHOICE {

r3								SEQUENCE {

rrcConnectionSetup-r3			RRCConnectionSetup-r3-IEs,

laterNonCriticalExtensions		SEQUENCE {

-- Container for additional R99 extensions

rrcConnectionSetup-r3-add-ext	BIT STRING		OPTIONAL,

v4b0NonCriticalExtensions		SEQUENCE {

rrcConnectionSetup-v4b0ext		RRCConnectionSetup-v4b0ext-IEs,

v590NonCriticalExtensions		SEQUENCE {

rrcConnectionSetup-v590ext		RRCConnectionSetup-v590ext-IEs,

v690NonCriticalExtensions		SEQUENCE {

rrcConnectionSetup-v690ext		RRCConnectionSetup-v690ext-IEs,

nonCriticalExtensions			SEQUENCE {}		OPTIONAL

}	OPTIONAL

}	OPTIONAL

}	OPTIONAL

}	OPTIONAL

},

later-than-r3					SEQUENCE {

initialUE-Identity				InitialUE-Identity,

rrc-TransactionIdentifier		RRC-TransactionIdentifier,

criticalExtensions				CHOICE {

r4								SEQUENCE {

rrcConnectionSetup-r4			RRCConnectionSetup-r4-IEs,

v4d0NonCriticalExtensions			SEQUENCE {

-- Container for adding non critical extensions after freezing REL-5

rrcConnectionSetup-r4-add-ext	BIT STRING		OPTIONAL,

v590NonCriticalExtensions		SEQUENCE {

rrcConnectionSetup-v590ext		RRCConnectionSetup-v590ext-IEs,

v690NonCriticalExtensions		SEQUENCE {

rrcConnectionSetup-v690ext		RRCConnectionSetup-v690ext-IEs,

nonCriticalExtensions			SEQUENCE {}		OPTIONAL

}	OPTIONAL

}	OPTIONAL

}	OPTIONAL

},

criticalExtensions					CHOICE {

r5								SEQUENCE {

rrcConnectionSetup-r5			RRCConnectionSetup-r5-IEs,

-- Container for adding non critical extensions after freezing REL-6

rrcConnectionSetup-r5-add-ext	BIT STRING		OPTIONAL,

v690NonCriticalExtensions		SEQUENCE {

rrcConnectionSetup-v690ext		RRCConnectionSetup-v690ext-IEs,

nonCriticalExtensions			SEQUENCE {}		OPTIONAL

}	OPTIONAL

},

criticalExtensions				CHOICE {

r6								SEQUENCE {

rrcConnectionSetup-r6			RRCConnectionSetup-r6-IEs,

-- Container for adding non critical extensions after freezing REL-7

rrcConnectionSetup-r6-add-ext	BIT STRING					OPTIONAL,

v6b0NonCriticalExtensions		SEQUENCE {

rrcConnectionSetup-v6b0ext		RRCConnectionSetup-v6b0ext-IEs,

nonCriticalExtensions			SEQUENCE {}				OPTIONAL

}	OPTIONAL

},

criticalExtensions				CHOICE {

r7								SEQUENCE {

rrcConnectionSetup-r7			RRCConnectionSetup-r7-IEs,

-- Container for adding non critical extensions after freezing REL-8

rrcConnectionSetup-r7-add-ext	BIT STRING					OPTIONAL,

v780NonCriticalExtensions		SEQUENCE {

rrcConnectionSetup-v780ext		RRCConnectionSetup-v780ext-IEs,

v7d0NonCriticalExtensions		SEQUENCE {

rrcConnectionSetup-v7d0ext		RRCConnectionSetup-v7d0ext-IEs,

nonCriticalExtensions			SEQUENCE {}		OPTIONAL

}	OPTIONAL

}	OPTIONAL

},

criticalExtensions				CHOICE {

r8								SEQUENCE {

rrcConnectionSetup-r8			RRCConnectionSetup-r8-IEs,

-- Container for adding non critical extensions after freezing REL-9

rrcConnectionSetup-r8-add-ext	BIT STRING				OPTIONAL,

v7d0NonCriticalExtensions		SEQUENCE {

rrcConnectionSetup-v7d0ext		RRCConnectionSetup-v7d0ext-IEs,

v890NonCriticalExtensions		SEQUENCE {

rrcConnectionSetup-v890ext

RRCConnectionSetup-v890ext-IEs,

v8a0NonCriticalExtensions		SEQUENCE {

rrcConnectionSetup-v8a0ext

RRCConnectionSetup-v8a0ext-IEs,

nonCriticalExtensions			SEQUENCE {}		OPTIONAL

}	OPTIONAL

}	OPTIONAL

}	OPTIONAL

},

criticalExtensions				CHOICE {

r9								SEQUENCE {

rrcConnectionSetup-r9			RRCConnectionSetup-r9-IEs,

-- Container for adding non critical extensions after

-- freezing REL-10

rrcConnectionSetup-r9-add-ext

BIT STRING		OPTIONAL,

v950NonCriticalExtensions		SEQUENCE {

rrcConnectionSetup-v950ext	RRCConnectionSetup-v950ext-IEs,

nonCriticalExtensions			SEQUENCE {}		OPTIONAL

}	OPTIONAL

},

criticalExtensions				CHOICE {

r10								SEQUENCE {

rrcConnectionSetup-r10			RRCConnectionSetup-r10-IEs,

-- Container for adding non critical extensions after

-- freezing REL-11

rrcConnectionSetup-r10-add-ext

BIT STRING	OPTIONAL,

vaj0NonCriticalExtensions		SEQUENCE {

rrcConnectionSetup-vaj0ext	RRCConnectionSetup-vaj0ext-IEs,

nonCriticalExtensions			SEQUENCE {}		OPTIONAL

}	OPTIONAL

},

criticalExtensions				CHOICE {

r11								SEQUENCE {

rrcConnectionSetup-r11

RRCConnectionSetup-r11-IEs,

-- Container for adding non critical extensions after

-- freezing REL-12

rrcConnectionSetup-r11-add-ext

BIT STRING	OPTIONAL,

vaj0NonCriticalExtensions		SEQUENCE {

rrcConnectionSetup-vaj0ext	RRCConnectionSetup-vaj0ext-IEs,

vbi0NonCriticalExtensions		SEQUENCE {

rrcConnectionSetup-vbi0ext	RRCConnectionSetup-vbi0ext-IEs,

nonCriticalExtensions			SEQUENCE {}		OPTIONAL

} OPTIONAL

}	OPTIONAL

},

criticalExtensions				SEQUENCE {}

}

}

}

}

}

}

}

}

}

}

RRCConnectionSetup-vbi0ext-IEs ::= SEQUENCE {

capabilityUpdateRequirement		CapabilityUpdateRequirementvbi0ext

}

CapabilityUpdateRequirement-vbi0ext ::=	SEQUENCE {

systemSpecificCapUpdateReqList		SystemSpecificCapUpdateReqList-vbi0ext

}

SystemSpecificCapUpdateReqList- vbi0ext::= SEQUENCE (SIZE (1..maxSystemCapability)) OF

SystemSpecificCapUpdateReq- vbi0ext

SystemSpecificCapUpdateReq- vbi0ext::=	ENUMERATED {

gsm, geranIu, eutra, eutraNoCA,

spare4, spare3, spare2, spare1 }

******************************************* TEXT END ******************************************