3GPP TSG-RAN WG2 #95bis	R2-167182

# Kaohsiung, 10 - 14 October 2016

Agenda Item:	8.4.2.1

Source: 	Huawei

Title:  	TP on Scenarios and RRC states

Document for:	Discussion and Decision

# Introduction

This document updates the text proposal of R2-166913 to take account of meeting discussion.

# Text proposal

# 4.3	Scenarios

The coverage scenarios considered in this study focus on the followings.

-	Evolved ProSe Remote UE and evolved ProSe UE-to-Network Relay UE are EUTRAN in-coverage;

-	Evolved ProSe UE-to-Network Relay UE is in EUTRAN coverage and evolved ProSe Remote UE can be in enhanced coverage (enhanced coverage implies that the UE is connecting to the network via Rel-13 MTC in CE mode);

-	Evolved ProSe UE-to-Network Relay UE is in EUTRAN coverage and evolved ProSe Remote UE is out of coverage of EUTRAN.

The considered scenarios are reflected in Figure 4.3-x.  The shown connections for the Uu interface indicate coverage and reachability, i.e. the Evolved ProSe UE-to-Network Relay UE shown is not necessarily in RRC_CONNECTED, but is in-coverage and can communicate with the network on Uu interface.

Figure 4.3-x: Coverage scenarios

In Figure 4.3-x, “in coverage” for the Evolved ProSe Remote UE may include extended coverage.

Editor’s Note: A definition of when the Evolved ProSe Remote UE and Evolved ProSe UE-to-Network Relay UE are “paired” or “connected” is needed.

Editor’s Note: Additional scenarios may be considered, e.g. when the Evolved ProSe Remote UE and Evolved ProSe UE-to-Network Relay UE are in coverage of different cells or eNBs.

The following procedures are supported for these scenarios:

- In Scenario 2, the Evolved ProSe Remote UE can initiate establishing a “connection” to the Evolved ProSe UE-to-Network Relay UE.

- In Scenario 2, the network can initiate establishing a “connection” between the Evolved ProSe Remote UE and the Evolved ProSe UE-to-Network Relay UE.  It is FFS if the network needs some “prior knowledge” of the relationship between the Evolved ProSe Remote UE and the Evolved ProSe UE-to-Network Relay UE.

Editor’s Note: The nature of such “prior knowledge” needs to be considered.

- In scenario 3, either the Evolved ProSe Remote UE or the network can initiate establishing a connection between the Evolved ProSe Remote UE and the Evolved ProSe UE-to-Network Relay UE.  It is FFS if the network can use some “prior knowledge” of the relationship between the Evolved ProSe Remote UE and the Evolved ProSe UE-to-Network Relay UE.

- It can be considered if there should be further restrictions on the network initiated cases.

The Evolved ProSe Remote UE is not required to be in RRC_CONNECTED while paired with an Evolved ProSe UE-to-Network Relay UE.

The Evolved ProSe UE-to-Network Relay UE may be in RRC_IDLE while paired with an Evolved ProSe Remote UE, and in this case the Evolved ProSe Remote UE is not in RRC_CONNECTED.  It is FFS if the Evolved ProSe Remote UE can still be in some form of connected mode.

Editor’s Note: A definition of “paired” is needed to clarify the meaning of the above items.

When used to describe the Evolved ProSe Remote UE, RRC_CONNECTED means that the UE has a context in the eNB.  The Evolved ProSe Remote UE behaviour in RRC states is FFS.