3GPP TSG-RAN WG2 Meeting #96						R2-167850

Reno, USA, 14th – 18th November 2016

Agenda item: 9.2.1.4

Source: National Instruments

Title:	Considerations on the Random Access Procedure in Massive MIMO NR

Document for: Discussion & Decision

# Introduction

The 5G Study Item regarding the “Study on New Radio Access Technology” was approved at the 3GPP TSG RAN#71 meeting. It has the objective to be able to operate from sub 1 GHz to 100 GHz in a large variety of deployment scenarios in a single technical framework [1]. Massive MIMO has been proposed as a promising candidate to meet the spectral efficiency targets for New Radio (NR) [2]. Within RAN2, there has been discussions about improvements to the LTE random access procedures and changes required to accommodate higher frequency NR that requires TX/RX beamforming. In this contribution, we discuss enhancements that can improve the efficiency of Massive MIMO NR.

# Discussion

## Reciprocity-based Massive MIMO NR

A canonical Massive MIMO system [2] has the following defining characteristics

- Large number of antenna ports at the eNB and conventional MIMO UEs

- TDD Operation

- DL Precoding based on UL Channel estimation and Channel Reciprocity

Since reciprocity is leveraged, no explicit feedback is required from the UE to eNB to determine the DL MU-MIMO precoding. As shown in Figure 1, during each coherence time, UL channel sounding is performed before UL data and DL data transmission based on that channel instance. The massive antenna array size enables the eNB to resolve a point in space to focus the energy. This is an important differentiator from conventional beamforming where the beams are steered in a specific direction. NR protocols should be designed to take into account these differences of Massive MIMO NR.

Figure 1: Reciprocity-based DL in Massive MIMO

## Random Access Procedure for Massive MIMO NR

Figure 2: LTE contention-based and contention-free random access procedures

Figure 2 shows the contention-based and contention-free random access procedures defined in LTE. For the contention-based procedure, in step 1, the UE transmits a RACH preamble to the eNB. In the Massive MIMO NR, the gNB can use this transmission to do UL channel sounding and use reciprocity to compute the DL precoder for beamforming to the UE. In Step 2, the random access response is transmitted by the gNB to UE using this computed precoder. In Step 3, the MSG3 is transmitted by the UE which contains information such as UE identity, RRC connection request etc. This UL transmission may be used by the gNB to update the DL precoder. In Step 4, contention resolution message can be sent out by the gNB using the DL precoder it computed for the relevant UE from the prior steps of the random access procedure.

For the contention-free random access procedure, the gNB first sends the RACH preamble assignment via dedicated signaling. While the procedure does not show an initial UL message, note that the contention-free procedure is used during scenarios such as handover. In case of a handover, the UE is in communication with the source gNB already. Hence, it can be assumed that the necessary DL precoder is available, if the assignment message is carried by a Massive MIMO NR connection.

Observation 1: The existing LTE random access procedures are compatible with the reciprocity-based random access procedure required in Massive MIMO NR

Proposal 1: Use contention-based and contention-free random access procedures in LTE as the baseline for Massive MIMO NR.

## Considerations with MIMO UE

If the UE has MIMO capability, transmit diversity can be used during the random access procedure. The UE will appear as a single antenna UE to the gNB and the DL precoder will be computed by the gNB based on this equivalent single antenna channel.

Since the Massive MIMO NR depends on reciprocity and DL beamforming, the system efficiency improves if we can utilize all the available degrees of freedom in the channel. In other words, enabling MIMO channel sounding in the UL and, thereby, the estimation of the optimal DL precoder by the gNB is highly desirable. Ideally, this information can be conveyed during the random access procedure so that the system can take advantage of this as early as possible [3], [4].

Observation 2: It is desirable to communicate the UE MIMO capability information during the random access procedure.

The following options can be considered to convey the UE MIMO capability information during the random access procedure. The discussion below assumes a contention-based random access procedure.

Option 1: In MSG3

The MIMO capability information is sent during MSG3. In this case, the random access procedure itself cannot take advantage of the enhancement. Subsequent message exchanges between UE and gNB can utilize it.

Option 2: RACH preamble subsets

The available RACH preambles are separated into subsets where each subset corresponds to a specific UE MIMO capability. The UE selects the RACH preamble from a subset based on its MIMO capability. Then, the UE and gNB can do UL MIMO channel sounding starting from MSG3 exchange onwards, i.e. after the first two steps in the random access procedure.

Option 3: UL MIMO channel sounding during RACH preamble

The available RACH preambles are separated into subsets where each subset corresponds to a specific UE MIMO capability. Assume, for example a UE can support 2 spatial streams. The UE then selects 2 RACH preambles from the subset corresponding to 2 spatial stream capability. The preambles in this subset may be paired such that the pairs are always selected together. The UE then sends the selected pair of RACH preambles during the first step of the contention-based random access procedure. The gNB will be able to determine the MIMO capability information, estimate the UL MIMO channel and compute the DL precoder. In this option, all steps in the random access procedure utilize the MIMO capability of the UE. There are trade-offs involved such as the number of available RACH preambles and link margin per RACH preamble.

Proposal 2: Study procedural enhancements that enable the gNB to obtain the MIMO capability of the UE during the random access procedure in Massive MIMO NR.

# Conclusion

In this contribution, we provide some considerations on Massive MIMO NR and the random access procedure and propose the following :-

Observation 1: The existing LTE random access procedures are compatible with the reciprocity-based random access procedure required in Massive MIMO NR

Proposal 1: Use contention-based and contention-free random access procedures in LTE as the baseline for Massive MIMO NR.

Observation 2: It is desirable to communicate the UE MIMO capability information during the random access procedure.

Proposal 2: Study procedural enhancements that enable the gNB to obtain the MIMO capability of the UE during the random access procedure in Massive MIMO NR.

# References

[1] RP-160671, “New SID Proposal: Study on New Radio Access Technology”, NTT DoCoMo

[2] R1-164117, “Overview of Massive MIMO for NR”, National Instruments, Nanjing, May 2016.

[3] R1-1609722, “NR MIMO Initial Access Procedure”, National Instruments, Lisbon, Oct 2016.

[4] R1-1612609, “Physical Layer Considerations for Random Access Procedure in Massive MIMO NR”, National Instruments, Reno, Nov 2016.