3GPP TSG-RAN WG2 #96	Tdoc R2-168588

# Reno, Nevada, USA, 14th – 18th November 2016

Agenda Item:	8.5.2

Source:	Ericsson

Title:	Absence of AC indication

Document for:	Discussion, Decision

# Introduction

At RAN2#95 it was agreed:

Agreements

1	 Mapping between LWA bearer and IEEE 802.11 AC will be determined in the WT. RRC used to provide the mapping to the UE. RAN3 to complete the Xw aspects

It was later discussed which AC the UE applies in case the eNB does not indicate any AC for a bearer. We discuss this in this paper.

# Discussion

There is an FFS on which WLAN AC the UE applies in case the eNB does not configure any for a certain bearer. We think the most straightforward way is to indicate in the field description which WLAN AC the UE applies in case the eNB does not configure any.

We assume the AC which will be most commonly used is AC_BE. We propose:

Indicate in the field description for lwa-WLAN-AC that the UE applies value ac_be in case no value is configured.

A text proposal based on the running CR is found below:

lwa-WLAN-AC For LWA bearers, indicates the corresponding WLAN access category for uplink. AC-BK (value ac-bk) corresponds to Background access category, AC-BE (value ac-be) corresponds to Best Effort access category, AC-VI (value ac-vi) corresponds to Video access category and AC-VO (value ac-vo) corresponds to Voice access category as defined by IEEE 802.11-2012 [67]. This field is included only when ul-LWA-DRB-ViaWLAN is set to TRUE or ul-LWA-DataSplitThreshold is configured. The UE applies value ac_be for a bearer in case no value has been configured for that bearer.

# Conclusion

Based on the discussion in section 2 we propose the following:

Proposal 1	Indicate in the field description for lwa-WLAN-AC that the UE applies value ac_be in case no value is configured.