3GPP TSG-RAN WG2 #96	Tdoc R2-168625

# Nevada, US, 14th – 18th Novemer 2016

Agenda Item:	8.22.2

Source:	Ericsson

Title:	HARQ process operation with Legacy TTIs, Reduced Processing, and sTTIs

Document for:	Information

# Introduction

At RAN#72 a work item on two features, short TTIs and reduced processing time n+3, was approved [1]. This gives new possibilities for reducing latency.

This contribution discusses the implications for HARQ process handling when short TTIs and reduced processing is introduced.

## Agreements in SI and RAN1 WI

The relevant conclusions from the Latency Reduction SI [2] and RAN1 (#86 and #86bis) [3] are:

- SI recommendations:

  - A UE is expected to handle the following cases in the same carrier in a subframe:

    - -	Receiving legacy TTI non-unicast PDSCH and short TTI unicast PDSCH(s)

  - For PUSCH transmission in sTTI (sPUSCH for short TTI), a UE can be dynamically (with a subframe to subframe granularity) scheduled with PUSCH and/or sPUSCH.

- Working assumption: a mechanism for dynamic fallback to legacy processing timings (n+4) is supported.

- No consensus to support a minimum processing time of n+2.

- Agreements:

- For FS1,2&3, a minimum timing n+3 is supported for UL grant to UL data and for DL data to DL HARQ for UEs capable of operating with reduced processing time with only the following conditions:

  - A maximum TA is reduced to x ms, where x <= 0.33ms (exact value FFS);

  - At least when scheduled by PDCCH

- PHICH-less asynchronous HARQ for UL is used for 1 ms TTI with shortened processing time

  - For FS1 and FS2, bit fields are defined in the applicable DCI messages to indicate HARQ processes ID and RV

- The DL sTTI length of a UE is configured by RRC signaling.

- For the combination of sTTI for DL and UL, RAN1 chooses one to be supported among the following alternatives.

  - Alt 1. {2,2}, {7,7}

  - Alt 2. {2,2}, {2,4}, {7,7}

  - Alt 3. {2,2}, {2,7}, {7,7}

  - Alt 4. {2,2}, {2,4}, {2,7}, {7,7}

  - Note: {a,b} denotes {DL sTTI length, UL sTTI length}.

  - Note: DL sTTI length is used for sPDCCH and sPDSCH.

  - Note: UL sTTI length is used for sPUSCH and sPUCCH corresponding to sPDCCH and sPDSCH, respectively.

# Discussion

To switch between the features, Legacy operation n+4, Short TTIs and Reduced Processing Time n+3, one must define what HARQ processes (HPs) that are valid for each TTI (be it a sTTI or a 1 ms TTI).

The handling of HARQ feedback for earlier transmissions and cases where more than one grant is valid in a TTI or sTTI needs to be considered.

To enable switching between different modes of operation without loss of data, one must consider how HARQ processes can be retransmitted after the switch, as well as how HARQ feedback should be handled after a switch.

## HARQ and HARQ process ID

For HARQ operation the eNB and UE must know which HP ID is to be used in each TTI.

In legacy downlink the UE receives a HP ID in the DCI, that is, it is asynchronous HARQ. Legacy uplink uses synchronous HARQ where each TTI is associated to a certain HP ID and there is no need to signal the HP ID to the UE.

It was agreed, at RAN1#86, that reduced processing n+3 shall use PHICH less asynchronous HARQ for UL.

In RAN1 there are up to 16 HPs being discussed for short TTIs, and 6 HPs for reduced processing n+3, and there are 8 HPs in legacy n+4.

One option to enable switching (fall-back) between reduced processing time (n+3), with asynchronous HARQ also for UL, and legacy n+4 synchronous uplink HARQ, is to associate a HP ID to each TTI in legacy, for example by connecting it to the system frame number, SFN, and subframe number ((SFN*10+sf) modulo (number of HPs)).

Consider defining a mapping between HARQ process ID and SFN plus subframe number for n+4 operation.

With asynchronous HARQ, the eNB must signal the HP ID in each assignment and grant, deciding which HARQ process that is used.

For short TTIs it has been decided to use asynchronous HARQ. The range 4 <= k <= 8 for the minimum UL grant to UL data timing k has been agreed in [2]. Further discussion on the exact value of k for different sTTI lengths are on-going but it is very likely that four bits are needed to address the HP ID.

## Switch cases

RAN2 should support switching between different sTTI UL-DL length combinations, for example due to changes in propagation, interference, HARQ payload, or when eNB must redistribute resources as traffic changes.

There are several different UL-DL sTTI combinations being considered in RAN1, for example (number of symbols in DL-UL):

- 2-2. This is the main short TTI operation case.

- 2-7. This is a mixed operation case.

- 7-7. This is the slot-TTI operation case.

- 2-14 / 7-14. These are cases where legacy PUCCH and PUSCH (n+3) is used together with DL short TTI.

The numbers indicate the number of symbols in the TTI, where 14 is the legacy 1 ms TTI (including reduced processing time n+3 and n+4 thus for each 14 there are two different cases). To these cases one can add the switch to 1ms TTI with n+4 and n+3 timing, and finally the fall-back switch:

- 2-2 to n+3 / n+4

- 2-7 to n+3 / n+4

- 7-7 to n+3 / n+4

- 2-14 / 7-14 to n+3 / n+4

- n+4 to n+3.

Consider what cases to switch between of sTTIs, and n+4, and n+3, as each have implications on the HP handling.

## Loss less switching between features

In case of switching from sTTIs to legacy n+4 synchronous HARQ, there can be data loss as only 8 HPs will be used. To minimize the loss, we could define a scheme where, for example, the eight newest HPs are mapped to the 8 processes of legacy n+4 but it is a complex solution with many error cases if a UE misses a grant. Simpler is to only take the 8 lowest HP IDs. Both these might incur HARQ data loss resulting in RLC retransmissions and longer delays.

To avoid data loss at switching, one solution is if, before and after the switch, it is possible to use asynchronous HARQ and the address space of HP IDs are the same. In case of a switch after which fewer HPs are needed, the eNB can still schedule all of them in order and thus avoiding any data loss.

This works fine for all switches besides when switching from sTTIs to legacy n+4 (as it has only three bits to address the HP ID in DL and no asynchronous HARQ for UL) and possibly switching from sTTIs to reduced processing n+3 (if it only will have three bits for HP ID). We there for suggest to consider a new DCI format to address four bits HP ID in n+4 and n+3 operation.

UL asynchronous HARQ n+4 would be beneficial to lower delay when switching from sTTIs operation to n+4

A new DCI for legacy n+4 and n+3 operation with four bits of HARQ process ID is beneficial for lower delay when switching from sTTIs to n+3/n+4

## HARQ feedback

In case of continuous transmission to one UE and switching from n+4 to n+3 operation, the UE will need to send HARQ feedback for double as many HPs as normal three TTIs after the switch. It must be defined how the UE shall handle the HARQ feedback in this case. For example, one could bundle the HARQ feedback, or use more bits to transmit it – but one must consider the error cases when an assignment/grant was not correctly received by the UE.

When switching from sTTIs to 1 ms TTIs, the HARQ feedback can be transmitted in the 1 ms TTI that begins at the same time or after where HARQ feedback would have been transmitted if there had not been a switch.

When switching from 1 ms TTIs to sTTIs, the HARQ feedback can be transmitted in an sTTI that begins at the same time or after where HARQ feedback would have been transmitted if there had not been a switch.

If number of bits for HARQ feedback is much higher than before, the eNB might chose to schedule a PUSCH/sPUSCH transmission to avoid coverage problems of PUCCH/sPUCCH.

How to handle HARQ feedback in case of switching needs to be defined to avoid unnecessary HARQ and RLC retransmissions.

When HARQ feedback is to be sent after a switch between sTTIs and 1 ms TTIs needs to be defined.

## Switching point for carrier aggregation

In case RAN1 decides all carriers must switch between sTTI/n+3/n+4 at the same time, it has to be defined when that is.

How and when switching of sTTI UL-DL combination or between sTTI and n+4 or n+3 for carrier aggregation may need to be defined.

# Conclusion

In the previous section we made the following observations:

- Consider what cases to switch between of sTTIs, and n+4, and n+3, as each have implications on the HP handling.

UL asynchronous HARQ n+4 would be beneficial to lower delay when switching from sTTIs operation to n+4 .

A new DCI for legacy n+4 and n+3 operation with four bits of HARQ process ID is beneficial for lower delay when switching from sTTIs to n+3/n+4

When HARQ feedback is to be sent after a switch between sTTIs and 1 ms TTIs needs to be defined.

When HARQ feedback is to be sent after a switch between sTTIs and 1 ms TTIs needs to be defined.

How and when switching of sTTI UL-DL combination or between sTTI and n+4 or n+3 for carrier aggregation may need to be defined.

In addition, we propose:

Consider defining a mapping between HARQ process ID and SFN plus subframe number for n+4 operation.

# References

RP-161299, New Work Item on shortened TTI and processing time for LTE, Ericsson, RAN#72, June 2016.

TR 36.881 v1.0.0,	Study on latency reduction techniques for LTE.

R1-1611055 LS on Shortened TTI and processing time for LTE, RAN1#86bis, October 2016.