3GPP TSG-RAN WG2 #96	Tdoc R2-168628

# Reno, US, 14th – 18th Novemer 2016

Agenda Item:	8.22

Source:	Ericsson

Title:	Impacts on L2/3 of sTTI

Document for:	Discussion, Decision

# Introduction

With short TTI (sTTI) as currently defined in RAN1 according to the WI. LTE latency in DL and UL is reduced by allowing transmissions on a shorter timescale. In this contribution, we discuss high level impact on L2, mainly MAC and RRC with the introduction of a short TTI and shorter processing compared with current Rel-14 LTE.  Not that the content herein is a uncomprehensive list.

# Discussion

The general assumption given below are based on input given in relevant conclusions from the RAN1 work progressed up to now.

A more detailed analyse is given in several accompanying contributions.

## MAC

### General concepts

#### TTI

Currently the definition of “TTI” in MAC refers to an interval in which a transport block is conveyed. Absolute time on the other hand, is using subframes and would be the absolute 1ms reference for current LTE. This is largely used for all timer definitions. For scheduling on the other hand, operations are done per TTI. This is true as a result of TTI being a “transport channel” property whereas the subframe is a “physical channel property. One could therefore consider a TTI for transmitting data on a physical channel using only a few symbols, or alternatively 14 symbols (as in legacy). It would thus make sense to maintain the definition of TTI also in cases when sTTI is introduced. This should also be the goal when specifying instances of PDCCH, ePDCCH or sPDCCH.

RAN2 should maintain transparency and definitions in the MAC specification when implementing support for sTTI and short processing as far as possible.

### Impact on MAC Timers

LTE MAC employs a number of subframe-based timers. As a result of shorter TTI duration, the impact on whether timers should be kept on e.g. 1ms-subframe operation, should be scaled along with the sTTI length or otherwise be adjusted accordingly.

RAN2 should study the impact on timers in MAC as a result of shorter TTI lengths.

- An initial analysis of possible impact(s) on MAC from timers and processing can be found in [1].

### Impact on DRX

The basic aim of DRX is to provide battery saving opportunities to the terminal by controlling the time instances when the UE has to be accessible by the network. With the introduction of short TTI, it is important to assess its effect on DRX timers. Note though, that as a consequence of achieving lower latency, it can be expected that in some cases the battery efficiency may be lower compared to legacy subframe/TTI structure, or it can be enhanced if there is a possibility to introduce micro sleep between short TTI’s.

RAN2 should study the impact on DRX and whether changes are needed to enhance or sustain current UE battery efficiency.

### Scheduling

A UE supporting sTTI and short processing time will similarly to LTE monitor downlink control channels such as the PDCCH and in addition sPDCCH. The grant received or configured from the network will similarly entail a resource for transmission on PUSCH/sPUSCH and/or PDSCH/sPDSCH. Furthermore, a UE can be dynamically scheduled with PUSCH and/or sPUSCH, based on a subframe to subframe granularity. Thus, an evaluation on UL/DL scheduling functionality is needed to assess the impact of sTTI and shorter processing time.

RAN2 should study the impact of sTTI and reduced processing time on scheduling and possible dynamic switching of TTI duration in scheduling functionality.

### HARQ and retransmissions

Closely related to Scheduling; with the introduction of Short TTIs and Reduced Processing Time (RPT), a definition pertaining to what HARQ processes are valid for each TTI (be it a sTTI or a 1 ms TTI) is needed. This also includes how to handle HARQ feedback for earlier transmissions and how grant validity is handled.

As to retransmissions, similarly to what have been introduced for LAA, PHICH-less asynchronous HARQ for UL needs to be supported along with shortened processing time.

RAN2 should study the impact on HARQ and handling of retransmissions due to asynchronous HARQ operation.

### SPS

The SPS period is RRC-configured with intervals on subframe basis. This is sufficient for legacy DL/UL SPS operation and should therefore be reused. However, sTTI and reduced processing time brings a potential reduction in latency by increasing the granularity of SPS grants to sTTI level from the current subframe level granularity. An analysis of the potential enhancements in SPS pertaining to SPS periodicity as well as capacity enhancements using sTTI and reduced processing time is described in [2].

RAN2 should study the possible enhancements in SPS functionality using shortTTI which could lead to potential benefits om terms of system capacity and latency.

## RRC

### UE capability

With the introduction of sTTI there is a need for a new UE capability in UE-EUTRA-Capability signalling.

### General MAC timer configuration

An analysis of the existing MAC subframe-based timers has been described in [1], and it is proposed that there should be no need to change or scale these timers due to the introduction of sTTI. This would mean that impacts to RACH-ConfigCommon or MAC-MainConfig for configuration of these timers can be avoided.

Furthermore, additions and changes to configurations with respect to SPS could be foreseen, although details to the technical benefits and solutions needs to be decided.

### DRX timer configuration

An analysis of the existing DRX specific timers has also been described in [1]. Based on the analysis, signalling updates will be necessary in MAC-MainConfig for:

- shortDRX-Cycle

- longDRX-CycleStartOffset

- drx-RetransmissionTimer

### sTTI length configuration

The following RAN1 agreement was listed in [3]:

Agreement:

- The DL sTTI length of a UE is configured by RRC signaling.

  - FFS on whether different DL sTTI lengths for a given UE can be configured for different serving cells or not.

So for DL sTTI length configuration there is impact on signalling, most likely in MAC-MainConfig.

### Physical layer configuration

The following RAN1 agreement was listed in [3]:

Agreement

- Select a sTTI scheduling scheme among the following candidates for each sTTI length

  - Single level DCI

    - RRC configuration of sPDCCH search space and/or sPDCCH frequency region

    - UE-specific information in sDCI related to sPDSCH/sPUSCH

  - Two level DCI

    - RRC configuration may or may not at least partially indicate sPDCCH frequency region/search space for some of the variants described below

    - variant 1

      - Slow DCI: non UE-specific information in PDCCH

      - Fast DCI: UE-specific information in sDCI

    - variant 2

      - Slow DCI: UE-specific information in PDCCH

      - Fast DCI: UE-specific information in sDCI

    - variant 3

      - Slow DCI: UE-specific information in PDCCH and/or sPDCCH

      - Fast DCI: UE-specific information in sDCI

  - Note: the sTTI scheduling scheme may be the same or different for different sTTI length

- FFS how to reduce the payload of sDCI/DCI messages for sTTI operation

- FFS support of multi-sTTI scheduling

- Additional L1 signaling related to sTTI operation can be considered

So based on this first agreement, there will be a need to introduce signalling for the configuration of the sPDCCH search space/frequency region.

RAN2 should introduce support for RRC configurable changes and additions based on progress in the technical solutions for introducing sTTI and short processing

# Conclusion

In the previous section we made the following observations and propose:

- Proposal 1	RAN2 should maintain transparency and definitions in the MAC specification when implementing support for sTTI and short processing as far as possible.

- Observation 1	RAN2 should study the impact on timers in MAC as a result of shorter TTI lengths.

- Observation 2	RAN2 should study the impact on DRX and whether changes are needed to enhance or sustain current UE battery efficiency.

- Observation 3	RAN2 should study the impact of sTTI and reduced processing time on scheduling and possible dynamic switching of TTI duration in scheduling functionality.

- Observation 4	RAN2 should study the impact on HARQ and handling of retransmissions due to asynchronous HARQ operation.

- Observation 5	RAN2 should study the possible enhancements in SPS functionality using shortTTI which could lead to potential benefits om terms of system capacity and latency.

RAN2 should introduce support for RRC configurable changes and additions based on progress in the technical solutions for introducing sTTI and short processing

# References

R2-16xxx, Impact of sTTI on MAC Timers, Ericsson, 3GPP TSG-RAN WG2 #96, 14th – 18th November 2016

R2-16xxx,  SPS and sTTI, Ericsson, 3GPP TSG-RAN WG2 #96, 14th – 18th November 2016

R2-167420, LS on Shortened TTI and processing time for LTE, RAN1, 3GPP TSG-RAN WG2 #96, 14th – 18th November 2016