3GPP TSG-RAN WG2 #96	Tdoc R2-168655

# Reno, Nevada, USA, 14th – 18th November 2016

Agenda Item:	9.2.1.1

Source:	Ericsson

Title:	On the need of PDCP/RLC reordering and in-sequence delivery

Document for:	Discussion, Decision

# Introduction

In RAN2#95bis, reordering and in-sequence delivery functionalities were discussed. It was agreed that RLC can be delivered out-of-order to PDCP, and “PDCP reordering is always enabled if in sequence delivery to layers above PDCP is needed (i.e. even in non-DC case)”.

We think that this agreement needs further clarification as “in-sequence” delivery and reordering are two relatively independent features. Thus, the purpose of this contribution is to discuss whether re-ordering is needed in PDCP and RLC, and to further develop the agreement with regards in-sequence delivery of RLC and PDCP layers.

# PDCP

In LTE, PDCP reordering is a functionality for:

- Keeping track of missing PDUs

- Moving the receiving window forward (and avoid window stalling) when a PDU has not been received after a period of time.

In-sequence delivery is another functionality which aims at ensuring that packets are delivered to higher layers in consecutive order following their Sequence Number (SN).

## Reordering and no-reordering operation

Reordering is realized in LTE by introducing a timer (T-reordering) which keeps control of how long to wait for a missing packet. When this timer expires, the data is delivered to higher layers and the state variables are updated.

T-reordering is started when the receiver has a SN gap. T-reordering is stopped when the missing PDU is received and, then, the receiving window is moved forward. If T-reordering timer expires but the missing SN was not received, the receiver window is moved forward until the last PDU submitted to higher layers, or until the next missing PDU.

No reordering means that the time (T-reordering) is not used. This implies that the receiving window is not moved until the missing PDU is received. If the receiving window fills before the missing packet is received (if ever received), the receiving window stalls and no more packets can be received/accepted. Stalling is resolved by forcing a reestablishment or receiving the data.

Reordering is, therefore, not needed when the lower layer provides in-sequence delivery of packets. This is the case when PDCP is connected to a single RLC (single connectivity). In this case, the RLC provides in-sequence delivery of PDUs and the PDCP does not need to implement the reordering function. However, in dual connectivity (e.g. PDCP connected to more than one RLC), in-sequence delivery of PDCP PDUs is not guaranteed. Therefore, reordering is needed.

Re-ordering in PDCP is needed if lower layers cannot guarantee in-sequence delivery.

## In/Out-of-/sequence delivery to upper layers

In LTE, in normal operation, PDCP always delivers packets in-sequence. When a packet is missing, PDCP does not deliver to higher layers any received packets which have a SN larger than the SN of the missing PDU. In principle, any packets received after the missing PDU could be delivered to higher layers (i.e. out-of-sequence delivery) if higher layers can handle out-of-sequence reception.

One of the reasons why PDCP has traditionally deliver packets in-sequence has been due to TCP. TCP has been quite sensitive to the out-of-sequence reception of packets. Newer versions of the TCP protocol, however, may handle out-of-sequence reception of packets in a better way. Another example is RoHC which expects in-sequence delivery to operate adequately (Section 4.4. in RFC 4995).

It is also true that new services may drive new requirements. This may be the case, for example, of URLLC. This type of services may need to deliver packets as soon as possible without waiting others to arrive.

RAN cannot take the assumption that protocols above PDCP may handle out-of-sequence reception of PDUs. Thus, RAN protocols need to be able to provide a diverse range of services to higher layers to fulfil diverse requirements. Having the possibility to configure either in-sequence or out-of-sequence delivery of PDCP packets is, therefore, necessary.

# RLC

In LTE, RLC reordering is a functionality for:

In AM

- Keeping track of missing PDUs

- Supporting the functionality to report STATUS PDUs

In UM

- Keeping track of missing PDUs

- May update the lower end of the reordering window

It is worth noticing that in RLC, reordering in AM does not directly affect the parameters which control the window the RLC window, while reordering in UM affects the lower end of the reordering window.

In-sequence delivery is another functionality which aims at ensuring that packets are delivered to higher layers in consecutive order following their Sequence Number (SN).

## Reordering

Reordering is always enabled in RLC for UM and AM in LTE. There are some reasons for this. One reason is that the layer below RLC (i.e. MAC) does not ensure that its packets are delivered, and that they are delivered in-sequence.

In RLC AM, reordering supports the “Acknowledge mode” functionality i.e. reordering variables assist the ARQ functionality (status PDUs are generated based on the reordering variables).

In RLC UM, it assists the protocol to keep track and wait for missing PDUs for a period of time. When a missing PDU is received or T-reordering expires, the lower end of the receiving window is moved forward. The lower end of the receiving window is also moved forward if the upper end of the receiving window moves forward. This means that reordering is not strictly needed in RLC UM. Even if reordering is finally agreed not to be supported for RLC UM, the RLC UM entity will need to keep track of the PDUs which are delivered, those which are not, and avoid duplication of data.

## In/Out-of-/sequence delivery to upper layers

RAN2#95 agreed that the receiving RLC entity can deliver RLC SDUs as they are reassembled or as they arrived to the RLC. Thus, in-sequence delivery might not be ensured. The benefit of doing so is unclear as the buffer requirements will anyway be in the RLC or in the PDCP.

This agreement has a two impacts. One of them is that PDCP needs to support reordering even for the single connectivity (see Observation 1). A second impact is that the re-ordering functionality is likely to become more complex. When in-sequence delivery is enabled, the protocol only needs to keep track of the last submitted PDU. However, when in-sequence delivery is not enabled, the protocol needs to keep track of both, missing PDUs and delivered PDUs (to avoid delivering duplicate PDUs).

We acknowledge that services like URLLC may benefit from a fast delivery of PDUs. Thus, disabling in-sequence delivery may be beneficial. It is, however, arguable whether the additional complexity justifies the gains. For services like URLLC, a very short T-reordering could be set which would ensure packets are delivered fast at the expense of missing late PDUs.

# Conclusion

Re-ordering functionality serves different purposes in PDCP, RLC AM, and RLC UM. Thus, re-ordering functionality is not strictly duplicated in RLC and PDCP.

Based on the discussion above, we propose the following for NR:

PDCP should implement the re-ordering functionality (T-reordering)

RLC AM should implement re-ordering functionality (T-reordering)

For RLC UM, discuss whether reordering should be made configurable or should not be supported.

Out-of-sequence may be enabled (or in-sequence delivery disabled) by the network independently for RLC and PDCP via RRC signalling.