3GPP TSG-RAN WG2 Meeting #96	Tdoc R2-168657

Reno, Nevada, USA, 14th – 18th November 2016

Agenda Item:	9.2.1.3 – NR – User Plane – QoS

Source:	Ericsson

Title:	QoS framework for NR

Document for:	Discussion, Decision

# Introduction

RAN2-95 and RAN-95bis reached several agreements (see section 6). In particular, it was agreed that the mapping of flows to DRBs is left to RAN implementation. That means, the RAN determines which flows to map onto which bearers. The RAN may map flows with the same or different QoS characteristics onto a common DRB and it may also multiplex Non-GBR and GBR flows. Of course, the chosen mapping should enable the scheduler to fulfil the QoS characteristics for all flows.

As a result of the previous discussions several open issues remained. In this document we discuss primarily “whether there is a single level of mapping from UL TFT (5 tuple) to DRB, or whether there is a 2 level mapping from UL TFT to QoS flow and then from QoS flow to DRB”.

Only after making that choice, RAN2 should discuss other open issues such as re-mapping of flows to DRBs or handling of initial packet or mapping of PDU sessions to a single DRB.

# Background

## Legacy QoS mapping/filtering

Figure 1 depicts the mapping of data (IP) packets to EPS bearers and to S1- and Data Radio Bearers as it is done in E-UTRA/EPC. The DL and UL packet filters on NAS level (TFTs) determine (based on source and destination IP Addresses and Port Numbers) in which bearer to carry each packet.

Figure 1: QoS Mapping in E-UTRA

## QoS mapping/filtering in the NextGen CN

### Flows instead of EPS bearers

Figure 2 shows the changes expected for the next generation CN according to discussions in SA2. Instead of mapping IP packets to EPS bearers, the Next Generation core network is supposed to group packets into flows. We expect that this may be done by packet filters similar to the TFTs defined in EPS, i.e., the next generation CN and the UE could ensure that all packets to and from, for example, the same IP/Port number tuple belong to a “flow”. On their way through the transport network each packet may be marked with some sort of “Flow ID”. In Figure 2 we denote these filters as “NAS filters”.

Like in E-UTRA/EPC, the core network determines and applies the downlink filters locally and it may configure the UE by means of NAS signalling with a set of uplink “NAS filters”.

Besides these explicitly configured uplink packet filters, SA2 discussed also a reflective QoS function. As a basic principle, the UE detects which DL packets appear in which DL flow and creates packet filters that identify corresponding uplink packets and map those to the same flow in UL direction. Hence no explicit NAS signalling to the UE is needed for configuring the “NAS” filters, which has large potential to decrease the control signalling for services where the filter criteria are subject to frequent changes. Explicitly adding and removing filters on port numbers and IP addresses could be avoided by such a mechanism.

Figure 2: Mapping of (IP-)Packets to Flows

### PDU Sessions instead of PDN Connections

Similarly to the PDN Connections in EPS the NextGen CN will support multiple PDU sessions per UE. Each PDU session is mapped to a separate transport network bearer in order to separate them even if the contained packets have an overlapping IP address range. Also the UE must be able to determine which IP packet belongs to which PDU session in order to route packets correctly. This may also need to be taken into account in the reflective QoS filtering.

# Discussion

## Mapping Flows to DRBs

As explained in section 2.2, SA2 intends to perform the mapping from service data flows to “flows” in the CN and in the UE’s NAS layer. Hence, as shown in Figure 2, the RAN and the UE’s AS layer remain agnostic to IP/TCP/UDP port numbers and service data flows. SA2 reflected this intention as follows:

10.1.1. In the downlink the (R)AN binds QoS Flows onto access-specific resources based on the NG3 marking and the corresponding QoS characteristics provided via NG2 signalling, also taking into account the NG3 tunnel associated with the downlink packet. Packet filters are not used for binding of QoS Flows onto access-specific resources in (R)AN.

According to the SA2 design the Access Stratum in eNB and UE remain agnostic to IP/TCP/UDP port numbers and service data flows.

RAN2 already agreed that the “RAN determines the mapping relationship between QoS flow (as determine by the UE in UL or marked by the CN in DL) and DRB for UL and DL”. The question in the last RAN2 meeting was how to achieve this. When following the RAN2 agreement that “RAN determines the mapping between QoS flow and DRB” and the principle to keep the Access Stratum agnostic to services, it has to be a two-step mapping as depicted in Figure 3.

Figure 3: 2-level mapping from UL NAS filters to QoS flow and then from QoS flow to DRB

In Figure 3 the “AS filters” determine the DRB by just looking at the “flow ID” of the incoming packet, i.e., the AS layer does not need to be aware of services, traffic-flow-templates and address/port tuples. The NAS filters, on the other hand, determine the mapping from services to “flows ID” but do not need to be aware of DRBs.

One should also remember that such 2-step mapping existed also in the EPS QoS concept: NAS handled the mapping from service data flows to EPS bearers and AS was responsible for mapping from EPS bearers to DRBs. However, in EPS the latter mapping was a pure 1:1 mapping whereas it may be a M:1 mapping in NR.

The RAN2 agreement that “RAN determines the mapping between QoS flow and DRB” and the principle to “keep the Access Stratum agnostic to services”, mandates a two-step mapping (NAS: service  flow; AS: flowDRB).

The two-step filtering fits well to the pre-configured QoS mapping: The eNB determines for each flow ID the DRB and provides such a configuration to the UE via RRC. This AS configuration is independent of the corresponding NAS mapping (IP packets to “flows”) except that the AS and NAS should use a common set of flow IDs. Hence, the RAN configures the “AS filters” whereas the CN configures the “NAS filters”.

The 2-level mapping from “UL NAS filter to QoS flow” and from “QoS flow to DRB” is well suited for pre-configured QoS.

For completeness, Figure 4 visualizes the alternative of a direct 1-level mapping from UL NAS filter (5-tuple) to DRB.

Figure 4: 1-level mapping from UL TFT (5 tuple) to DRB

We kept the network side unchanged (following the SA2 guidance) and just tried to modify the UE side. As can be seen, it is however not obvious how to realize such a direct mapping. The eNB is supposed to remain unaware of NAS filters and services. Hence, it cannot configure the upper part of the filter. And the CN should remain agnostic to radio bearers. Therefore, it cannot decide on the lower part of the filters.

1-level mapping from UL NAS filters (5 tuple) to DRB violates the principle to keep the RAN “service agnostic” and the CN “radio agnostic”.

Based on this analysis, we propose the following:

2-level mapping from “UL NAS filters to Flow” and from “Flow to DRB” is used in NR. Only the “Flow to DRB” mapping is under responsibility of the access stratum, i.e., UE and eNB.

Besides the pre-configured QoS mapping, also reflective QoS should be supported both on NAS level as well as on AS level. To determine the reflective “AS-filters” at the UE, the eNB must include the flow IDs into the DL packets (e.g. into the PDCP header). Furthermore, the NAS layer uses the flow-ID associated with each IP packet to create uplink “NAS filters” that map the corresponding uplink IP packets to flows. It should be noted that for a certain flow NAS may operate with reflective NAS filters while the RAN provides a configured AS filter or vice versa. So far, SA2 has not planned to indicate to the RAN whether or not it intends to apply reflective QoS on NAS level.

The flow ID assigned by the CN should not only be used for determining the DRB on the Uu interface but also for QoS handling in the transport network. For example, the “Flow ID” could be used as or mapped to a Diff-Serv code point in the IP header. Moreover, the Flow ID in UL may be used by the CN for the purpose of SDF binding verification, i.e. to verify that a Flow Id has not been misused by an SDF. This implies that the eNB should mark uplink packets with the correct “Flow ID” before sending them on the NG3 interface towards the CN. To enable this, UE should include the flow IDs in the UL PDCP headers. And also for this to work, the eNB should include the flow ID in all DL packets so that the UE can determine the necessary “AS filter” and determine the “Flow ID” to include in the UL PDCP header.

UE and eNB include a “Flow ID” into uplink and downlink PDCP PDU headers respectively.

The UE determines the reflective “AS filter” based on the downlink packets received within a DRB and applies those filters for mapping uplink Flows to DRBs.

Determining reflective “AS filters” should not be a one-shot action. The UE should continuously monitor the flow ID in DL PDCP packets and update packet filters accordingly. For example, if the UE observes initially a downlink packet with Flow ID X on DRB 1, it creates an AS filter that maps uplink packets with Flow ID X to DRB 1, too. But if the UE later observes a downlink packet with Flow ID X on DRB 2, it should change the filter for Flow X so that also the UL packets are mapped to DRB 2.

The UE continuously monitors the Flow ID in downlink PDCP packets and updates the reflective uplink “AS filters” accordingly.

As mentioned in the beginning of this paper, the reflective QoS concept intends to reduce the signalling overhead but the need to include the flow ID in each PDCP header increases the user plane protocol overhead. The relative overhead is marginal for large IP packets but could be considered significant for services such as VoIP.

One could consider omitting the flow ID in all but the first DL packet or a certain flow. Or one could compress the flow ID on the radio interface. But since such schemes increases the complexity (book-keeping) both in UE and eNB they don’t come for free either.

There are however still cases when the Flow ID does not need to be conveyed: Whenever the eNB configures a dedicated DRB for a Flow (1:1 mapping) and if it configures the Flow to be mapped to the DRB (explicitly configured UL “AS filter”), neither the UE nor the eNB need to include the flow ID. Since this may be a typical case for IMS VoIP as well as for latency critical services where the relative overhead due to the Flow-ID is significant, the eNB should have means to configure for each DRB whether the Flow-ID is conveyed in the PDCP header.

The eNB configures by RRC for each DRB whether or not the UE shall include the Flow-ID in uplink PDCP headers.

## Precedence order of reflective and configured mapping

RAN2 agreed that there will be both, reflective and preconfigured mapping of flows to DRBs. It should also be possible to define a dedicated mapping of flows to DRBs even though the NAS level applies reflective filters only. To ensure the desired behaviour, the order in which the UE evaluates the mapping needs to be settled. Hence, we suggest the following:

If the eNB configures the UE with an uplink “AS filter” that determines the mapping of an uplink flow to a DRB this mapping overrides any reflective “AS filter” for this flow.

RAN2-95bis agreed that “if the first packet of the flow is UL packet, if no mapping rule is configured in the UE, the packet is sent through default DRB to the network”. We suggest specifying this in a more general way, e.g. by not limiting it to a first packet of an uplink flow:

If an incoming UL packet matches neither a configured nor a reflective uplink “AS filter”, the UE shall map that packet to the default DRB.

We believe that these two bullets also address the following open issue that was captured at RAN2-95bis: “FFS the first packet is handled in the case that pre-authorised QoS is configured”. With this rule, the UE will follow the configured “AS filter” whenever there is one. If there is no configured AS filter, it applies the reflective AS filter as soon as those have been determined based on an observed DL packet. Before that maps initial uplink packets to the default bearer.

## Flows belonging to different PDU Sessions

Assuming that there will be very few PDU sessions for a UE and that the establishment and release of PDU sessions will be very static over time, it seems preferable to avoid additional overhead (for signalling the PDU session ID in each PDCP packet) and to allow only traffic of one PDU session to be mapped to a DRB. This approach will also ensure separation of packets that belong to different PDU sessions since there will be separate queues in UE and eNB even if they belong to the same QoS class. In other words, it minimizes the impact that packets of different PDU sessions have on each other.

Flows associated with different PDU sessions are mapped to different DRBs.

# Conclusion

Based on the discussion in section 3 we propose the following:

Proposal 1	2-level mapping from “UL NAS filters to Flow” and from “Flow to DRB” is used in NR. Only the “Flow to DRB” mapping is under responsibility of the access stratum, i.e., UE and eNB.

Proposal 2	UE and eNB include a “Flow ID” into uplink and downlink PDCP PDU headers respectively.

Proposal 3	The UE determines the reflective “AS filter” based on the downlink packets received within a DRB and applies those filters for mapping uplink Flows to DRBs.

Proposal 4	The UE continuously monitors the Flow ID in downlink PDCP packets and updates the reflective uplink “AS filters” accordingly.

Proposal 5	The eNB configures by RRC for each DRB whether or not the UE shall include the Flow-ID in uplink PDCP headers.

Proposal 6	If the eNB configures the UE with an uplink “AS filter” that determines the mapping of an uplink flow to a DRB this mapping overrides any reflective “AS filter” for this flow.

Proposal 7	If an incoming UL packet matches neither a configured nor a reflective uplink “AS filter”, the UE shall map that packet to the default DRB.

Proposal 8	Flows associated with different PDU sessions are mapped to different DRBs.

# References

R2-165336, “QoS framework for NR”, Ericsson, Gothenburg, Sweden, 22nd – 26th August 2016

# Annex: QoS-related agreements in previous meetings

RAN2-95 discussed the basic principles of the NR QoS framework and reached the following agreements:

Agreements

1	For DL for a non-GBR flow, the eNB sees an indication over NG-u and based on the indication the eNB maps the packet to a DRB of an appropriate QoS. RAN2 understanding of SA2 agreements is that eNB has a QoS profile associated with the indication.

FFS whether there is a requirement for every different QoS indication to be mapped to a different radio bearer.

2	Functionality is required to differentiate flows from different PDN-connections over the radio interface (e.g. by using separate DRBs or by an explicit indication in a header)

3	For DL, the eNB establishes DRBs for the UE taking the QoS profiles in to account.

FFS how the DRB is established in the first packet is an UL packet.

At RAN2-95bis some further agreements were achieved and the first of the FFSs above was resolved:

Agreements

1:	RAN determines the mapping relationship between QoS flow (as determine by the UE in UL or marked by the CN in DL) and DRB for UL and DL.

1a	RAN can map multiple QoS flows to a DRB.

2	Specification will not forbid a GBR flow and non-GBR flow to be mapped to the same DRB, but we will not introduce mechanisms to optimise this case.

3	Specification will not forbid more than one GBR flow to be mapped to the same DRB, but we will not introduce mechanisms to optimise this case.

FFS: Whether traffic from different PDU sessions can be mapped to one DRB or not.

Agreements

1	Default DRB is established by eNB at PDU session establishment (or an existing DRB may be used if mapping of more than one session to a DRB is allowed)

2. 	If the first packet of the flow is UL packet, if no mapping rule is configured in the UE, the packet is sent through default DRB to the network.

FFS How and when the network can remap the flow to more appropriate DRB.

FFS the first packet is handled in the case that pre-authorised QoS is configured

FFS whether the pre-authorised QoS applies to RAN or only to the UE.

FFS whether there is a single level of mapping from UL TFT (5 tuple) to DRB, or whether there is a 2 level mapping from UL TFT to QoS flow and then from QoS flow to DRB.