3GPP TSG-RAN WG2 #97	Tdoc R2-1700838

Athens, Greece, 13th – 17th February 2017	Update of R2-1700430

Agenda Item:	10.2.1.4

Source:	Ericsson

Title:	Uplink dynamic scheduling in NR

Document for:	Discussion, Decision

# Introduction

Uplink scheduling is a key functionality to meet a broad range of use cases including enhanced mobile broadband, massive MTC, critical MTC, and additional requirements defined during the RAN requirements study. Details to how this functionality can be designed, compared to the baseline framework of LTE is discussed in this contribution.

This contribution also includes a corresponding text proposal describing scheduling in NR.

# Discussion

## Role of SR and BSR

In LTE, scheduling requests (SRs) are used for requesting UL-SCH resources for new transmissions when the UE has no valid grant. If SRs are not configured for the UE, the UE initiates a Random Access procedure in order to get scheduled in UL. In LTE SRs consists of only one bit of information and indicates only that the UE needs an UL grant. This means that upon the reception of SR, gNB knows neither which logical channel (associated with certain QCI) has data available for transmission, or the amount of data available for transmission at UE. Furthermore, in view of the latest RAN2 agreements, it should be noted that the numerology/TTI duration should be conveyed in the grant. This implies that the gNB must also be made aware of what numerology/TTI duration is desired by the UE for the upcoming transmission. In short, in NR an accurate grant cannot be provided to the UE only based on the one-bit information of the LTE type of SR.

LTE scheduling request saves physical layer resources but does not provide sufficient information for efficient grant allocation in NR.

Buffer Status Reports (BSRs) on the other hand carry more detailed information compared to SR. A BSR indicates buffer size for each LCG. However, the BSR requires a grant for transmission so it may take longer time until the gNB receives it since it may need to be preceded by an SR. The interaction between SR, BSR and grant is exemplified in Figure 1.

Figure 1: Scheduling procedure for dynamic scheduling in LTE

LTE BSRs provide more detailed information than an SR, but it brings additional delay due to the need for a grant.

We believe the framework with SR/BSR from LTE in general works well but there may be room for improvements. We propose:

The SR/BSR scheme from LTE can be reused in NR as a baseline.

Naturally, RAN2 should identify limitations of SR and BSR and study potential improvements where necessary. In the below sections we expand more on the options at hand.

## Aspects for improvements in NR

NR should support wide spread of use cases which have different requirements. In some use cases, e.g. critical MTC and URLLC, NR has tighter latency requirements than has been considered for LTE so far. Also, services such as eMBB can enjoy the enhancements to SR and BSR. Some required modifications are briefly described in this section as examples of what can be improved.

In NR, the required modifications of SR/BSR aim to report UE buffer status (priority and the buffer size) as well as wanted numerology/TTI duration within the given time constraints. It is assumed that a mapping of LCH to LCG to numerology/TTI duration will make it possible to infer which numerology/TTI duration to use given the LCG. Hence no explicit signaling of numerology/TTI duration is needed in the SR/BSR if LCG (or LCH) is present in the SR/BSR. Considering the limitations identified in 2.1, it is possible to either enhance SR with more information bits to indicate more information or enhance BSR.

### Enhancement to SR

A possible improvement is to extend the SR to not only indicate whether data is available not. With more bits used in SR it would be possible to provide more detailed information such as the type of LCG which has data available, and/or the amount of available data associated with the LCG.

By knowing the type of LCG, gNB can provide grants for the traffic that needs to be scheduled. This enables a more correct priority handling. By indicating the amount of available data associated with the LCG that needs grant at the UE, gNB can provide a more suitable grant size on the preferred e.g. numerology/TTI duration to the UE.

Since the numerology/TTI duration can be derived from the LCG, situations where the UE has data for transmission on e.g. a short TTI, but receives a grant on a long TTI can be avoided.

How many bits that SR should be extended with, is a question of how to achieve a good trade-off between the increased L1 control channel issues (overhead, design complexity etc) and the possible achieved gain in terms of UP latency reduction. Further studies in RAN1 and RAN2 are necessary.

More efficient priority handling is achievable by extending additional bits for SR.

### Enhancement to BSR

#### Grant-free transmission for BSR

To avoid the delay caused by BSR grant allocation, grant-free transmission of BSR without sending an SR should be supported. This may be a viable opportunity at low and medium load and in cells serving relatively few (active) UEs.

Similar grant-free mechanisms are also expected to be introduced for delay critical use cases such as URLLC. For fast BSR reporting purposes, a dedicated resource allocation per UE is preferred. If grant-free transmissions are supported, it would be efficient to send BSR per logical channel group (also referred to as short BSR in LTE). In this way, only the BSR intended for high priority of traffic can be allowed to use the grant-free channel. For efficiency reasons, the grant-free resources assigned per UE should be large enough to fit just the BSR. The grant-free resources should also be possible to be utilized by data transfer, if there is no BSR pending for transmission.

Grant allocation delay for BSR can be reduced with grant-free transmission of BSRs.

#### Improved BSR triggering

In LTE, some of the existing rules for BSR triggering are too strict. For instance, the UE is allowed to transmit a BSR when there is new data available in the buffer with higher priority than the existing data, while the UE is not allowed to send a BSR if the new data has the same or lower priority than the existing data. This may lead to information mismatch between the UE and gNB, resulting in a long unnecessary scheduling delay until the UE can empty its transmission buffer. In this case, a simple solution is to remove the above restriction, i.e., let UE to send BSR when there is new data regardless of its priority. The NW can configure this feature considering the balance between increased BSR reporting overhead and the need for accurate buffer information estimation.

Scheduling delay may be reduced by allowing UE to send BSR upon the arrival of new data regardless of the priority of its associated logical channel.

#### Additional information in BSR

Just as in the case of SR, the gNB needs to be made aware of what numerology/TTI duration that is preferred or wanted the data. Since we assume that a mapping of LCH to LCG to numerology/TTI duration will make it possible to infer which numerology/TTI duration to use given the LCG indicated in the BSR, no additional information is needed in the BSR.

No additional information is needed in the BSR to infer wanted numerology/TTI duration.

### Pros and Cons

SR enhancements give fast reporting without grant allocation at Layer 2. However, it would incur higher control channel overhead, and higher design complexity. It is also more difficult to ensure the transmission reliability given that more information bits are carried.

BSR enhancements potentially achieves the same performance as SR enhancements in terms of reduction of UP latency. While it requires NW to assign dedicated resources to each UE, it might have a risk of resource over-provision in case there are a massive amount of connected UEs.

In some cases, if SR enhancements are adopted, BSR enhancements are not needed and vice versa.

Therefore, it is meaningful for RAN1/RAN2 to further compare different enhancements.

Study and compare possible enhancements to the buffer reporting mechanisms in SR and BSR areas:

- Enhancements to SR

  - Study solutions where SR carry additional bits to provide more information such as logical channel (group) requesting resources, (and)or the amount of the data associated to that logical channel (group).

- Enhancements to BSR

  - Study solutions using grant-free resources for BSRs, and/or sending BSR upon the arrival of new data regardless of the logical channel.

## Text proposal to TR 38.804 on dynamic scheduling

Below texts should be captured by TR 38.804 for the completion of the SI phase.

.

Adopt the text proposal on “uplink dynamic scheduling” to TR 38.804

7 Scheduling In order to utilise the SCH resources efficiently, a scheduling function is used in MAC. In this subclause, an overview of the scheduler is given in terms of scheduler operation, signalling of scheduler decisions, and measurements to support scheduler operation. 7.1 Basic scheduler operation MAC in NR gNB includes dynamic resource schedulers that allocate physical layer resources for the DL-SCH, UL-SCH transport channels. Different schedulers operate for the DL-SCH, UL-SCH. The scheduler should take account of the traffic volume and the QoS requirements of each UE and associated radio bearers, when sharing resources between UEs. Only "per UE" grants are used to grant the right to transmit on the UL-SCH. Since a logical channel can be mapped to one or more numerologies/TTI durations, the grant may be limited to certain logical channels mapped with certain numerologies, so, only those logical channels are allowed to transmit upon reception of this grant. Editor’s note: How grant is connected to certain logical channels and numerologies is FFS. Schedulers may assign resources taking account the radio conditions at the UE identified through measurements made at the e gNB and/or reported by the UE. 7.1.1 Downlink scheduling 7.1.2 Uplink Scheduling In the uplink, NR gNB can dynamically allocate resources (PRBs and MCS) to UEs at each TTI via the C-RNTI on PDCCH(s). Within each scheduling epoch, the scheduling entity may assign a grant associated with a set of numerologies/TTI durations for each schedulable UE. 7.3 Measurements to Support Scheduler Operation Measurement reports are required to enable the scheduler to operate in both uplink and downlink. These include transport volume and measurements of a UEs radio environment. Uplink buffer status reports (BSR) and scheduling request (SR) are needed to provide support for QoS-aware packet scheduling. The scheduling request (SR) as a layer one signalling message is used for requesting UL resources for new transmissions when the UE has no valid grant. An SR can be transmitted via either a PUCCH like channel in case the UE has dedicated resources assigned for it or a Random Access procedure in case the UE has no dedicated resources assigned for it or the UE is out of synchronization from the network. Uplink buffer status reports refer to the data that is buffered in for a group of logical channel (LCG) in the UE. Uplink buffer status reports are transmitted using MAC signalling. Prior to a BSR transmission, the UE is required to have a valid grant. The scheduling entity needs to be aware of the information including - An indication that UE has data to transmit - Buffer size for each logical channel (group) - Priority indication for each logical channel (group) - Indication of a set of the associated numerologies/TTI durations for each logical channel (group) For each UE, the above information may be reported by a SR or a BSR. Editor’s note: Detailed information content for SR and BSR is FFS

# Conclusion

In section 2 we made the following observations:

Observation 1	LTE scheduling request saves physical layer resources but does not provide sufficient information for efficient grant allocation in NR.

Observation 2	LTE BSRs provide more detailed information than an SR, but it brings additional delay due to the need for a grant.

Observation 3	More efficient priority handling is achievable by extending additional bits for SR.

Observation 4	Grant allocation delay for BSR can be reduced with grant-free transmission of BSRs.

Observation 5	Scheduling delay may be reduced by allowing UE to send BSR upon the arrival of new data regardless of the priority of its associated logical channel.

Observation 6	No additional information is needed in the BSR to infer wanted numerology/TTI duration.

Based on the discussion in Section 2 we propose the following:

Proposal 1	The SR/BSR scheme from LTE can be reused in NR as a baseline.

Proposal 2	Study and compare possible enhancements to the buffer reporting mechanisms in SR and BSR areas:

a.	Enhancements to SR

	Study solutions where SR carry additional bits to provide more information such as logical channel (group) requesting resources, (and)or the amount of the data associated to that logical channel (group).

b.	Enhancements to BSR

	Study solutions using grant-free resources for BSRs, and/or sending BSR upon the arrival of new data regardless of the logical channel.

Proposal 3	Adopt the text proposal on “uplink dynamic scheduling” TR 38.804

# References

TR 36.881, Study on latency reduction techniques for LTE, v0.6.0

R2-165340, Targeted Protocol Latency, Ericsson, RAN2#95, 22–26 August 2016