3GPP TSG-RAN WG2 NR Ad Hoc	Tdoc R2- 1700861

Spokane, USA, 17th – 19th January 2017

Agenda Item:	3.3.1.1.1

Source:	Ericsson

Title:	Filtering of RRM measurements in CONNECTED

Document for:	Discussion, Decision

# Introduction

=> RAN2 understanding of the LS is that RAN1 will define some form of additional RS to be used by the UE for RRM measurements in CONNECTED in addition to the RS(s) available in idle.

In RAN2#NRAdHoc meeting, in Spokane, an LS was sent from RAN1 to RAN2 stating that for CONNECTED mode RRM measurement for L3 mobility either CSI-RS and/or an additionally RS separately designed (previously called mobility RS) would be used if needed [1]. After a discussion of what “if needed” could possibly mean, RAN2 has interpreted the agreement as follows:

Another contribution has addressed the implications of these assumption concerning the triggering conditions for cell-based and beam-based mobility events. In this contribution we cover some initial analyses associated to the filtering of RRM measurements associated to these additional RSs in the particular case some beam grouping value needs to be defined, e.g. in the case of a cell.

In RAN2#96, connected-mode cell quality derivation from individual beams was discussed and the following agreements were made [2]:

Agreements for connected active

1: 	RRM measurement for cell level mobility should be performed based on a common framework regardless of network beam configurations (e.g., number of beams) and the UE beam configuration.

FFS: Which beams the UE selects from the detected beams in order to derive a cell level quality. Options to be studied:

a/ best beam,

b/ N best beams,

c/ all detected beams

d/ beams above a threshold.

Other options are not precluded

In this contribution, we describe how cell level quality derivation should be performed by an RRC_CONNECTED UE in order to provide RRM measurements for RRC driven ‘cell’ level mobility. Simulations results are also provided.

# Discussion

Mobility only based on individual beams in some scenarios may result in an extensive number of beam evaluations and measurement report triggering. It may also cause ping-pong effects, when the per-beam variations result in UEs being handed over between base stations back and forth. In the example in Figure 1, taking handover decisions based on measurements on individual beams, i.e. always handing over to the beam with the strongest RSRP, would lead to six handovers between beams (11->22->12->13->31->32->33) and three between base stations (1->2->1->3), where the handover between base stations 1 and 2 is unnecessary.

Figure 1 Multiple beams per gNB.

In the example, it would have been beneficial if the UE could be configured to process groups of beams and use per beam-group processed measurements for report triggering to better support RRC based mobility.

## Simulation assumptions

In this contribution, in order to select the most appropriated filtering alternatives for RRM measurements in connected mode we study the alternatives listed in the RAN2#96 agreement i.e.:

- ‘Best (strongest) beam’ with the grouping done before L1 filtering, after L1 filtering/before L3 filtering, and after L3 filtering

- ‘Average of X beams’ with the grouping done before L1 filtering, after L1 filtering/before L3 filtering, and after L3 filtering, wherein X = 3.

- ‘Best X beams’ with the grouping done before L1 filtering, after L1 filtering/before L3 filtering, and after L3 filtering, wherein X = 3.

- ‘All detected beams’

The rest of the simulation parameters are listed in Annex A.

## Ping-pong and handover failure probabilities

The ping-pong probability is evaluated for the different options. The ping-pong probability is evaluated by counting the number of ping-pong occurrences (i.e., as in X  Y  X where X, Y refer to different cells) and successful and unsuccessful handover events. The ping-pong probability is derived by:

PPP=NPPNHO_success + NHO_fail

where NPP is the number of ping-pong occurrences, NHO_success is the number of successful handovers, NHO_fail is the number of failed handovers. The simulation results are shown in Figure 3.

Figure 2 The probability of ping-pong occurrence for the following filtering options: ‘Best 3 beams, ‘All detected beams’, ‘Average of 3 beams’, ‘Best beam’.

The handover failure probability is calculated by the ratio of the number of handover failures to all handover decisions, wherein the handover failure is determined by comparing the RSRP of the target cell to a cell detection threshold during the duration of TTT. If the measured RSRP is lower than the threshold, it is considered that the target cell cannot be detected hence a handover failure is registered.

The evaluated handover probability is given by:

PHO_fail=NHO_failNHO_success + NHO_fail

where NPP is the number of ping-pong occurrences, NHO_success is the number of successful handovers, NHO_fail is the number of failed handovers. The simulation results are shown for the “Best beam” and “Average of 3 beams” filtering options in Figure 4, wherein both are based on grouping after L3 filtering.

Figure 3 Handover failure probabilities for the ‘Best beam’ and ‘Average of 3 beams’ filtering options, wherein grouping is performed after L3 filtering.

As expected, the alternatives relying on ‘Best beam’ and ‘Average of 3 beams’ filtering appear to be less subject to ping-pongs as opposed to the other options although the differences between them are not significant, as shown in Figure 2. On the other hand, when comparing the handover failure probability, ‘Best beam’ filtering option performs better than ‘Average of 3 beams’ in all of the simulated cases (30 kmph and 120 kmph users) when the network operates at 2 GHz and 30 GHz frequencies, as shown in Figure 3.

Although these are preliminary results, under limited scenarios, they were somewhat expected. For the handover failure performance, only the best beam matters since that would be the one serving the UE after the handover instead of a combination of them (in reality, the UE would be served by a refined version of that beam) On the other hand, averages considering other beams that are not the best increase the chance the UE moves to a cell that will fail.

Therefore, since the difference in the number of ping-pongs for the ‘Best beam case’ compared to the ‘average of 3 beams’ is not significant, RAN2 should further consider at least filtering based on ‘best beam’ to be further evaluated.

- Based on these observations the following has been proposed:

- At least filtering based on ‘Best beam’ should be supported to derive cell-level quality

# Conclusion

We made the following proposal:

Proposal 1    At least filtering based on ‘Best beam’ should be supported to derive cell-level quality

# Reference

R2-1700648 LS on NR Mobility, RAN1, 3GPP TSG-RAN WG2 NR AdHoc, Spokane, USA, 17th – 19th January 2017.

Chairman’s Report of RAN2-96; ftp://ftp.3gpp.org/tsg_ran/WG2_RL2/TSGR2_96/Report/

# Annex A. 	Simulation Parameters

Table 1 Parameters

| Parameter name | Value |
| --- | --- |
| Operating frequency | 2GHz / 30 GHz |
| Deployment | 3-site urban-macro with wrap-around (9 sectors) |
| Duplex | FDD |
| Tx antenna configuration | 1x16 (row array), 16 beams per sector |
| System Bandwidth | 40 MHz |
| TTI length | 0.125 ms |
| Subcarrier spacing | 60KHz |
| UE speed | 30 kmph / 120 kmph |
| UE position update period | 2 ms |
| UE ports | 2 |
| Propagation mode | 3GPP TR 36.873 at 2 GHz, 3GPP TR 38.900 at 30 GHz |
| Channel estimation | Ideal |
| Cell detection threshold | -110 dBm at 2 GHz, -130 dBm at 30 GHz |
| Hysteresis margin | 3 dB |
| Filter coefficients | L1 filter λ = 0.8, L3 filter k = 4 |
| Handover measurement period | 20 ms |
| Handover update period | 20 ms |