3GPP TSG-RAN WG2 #97	Tdoc R2-1700866

Athens, Greece, 13th – 17th February 2017

Agenda Item:	10.3.1.1.3

Source:	Ericsson

Title:	SeNB to MeNB reconfiguration for NR dual-connectivity

Document for:	Discussion, Decision

# Introduction

In RAN2#96 following agreements were made:

1	The mobility enhancement similar to that discussed for LTE (“Maintaining Source eNB connection during handover”) should be considered also for NR.

2	For DC (NR-NR), study how to reconfigure the UE from an MeNB to an SeNB to target the 0 ms UP interruption. FFS whether also applicable to LTE-NR

In addition, at RAN2-NR#1 it was agreed that:

1	Security key refresh is not performed at every mobility procedure (i.e. handover), at least for the case of mobility where the PDCP anchor point is not changed. (To be confirmed by SA3)

1a	RAN2 will let SA3 consider whether the agreement 1 has any implication on the inputs for key derivation (e.g. PCI)

NR needs to fulfil several very challenging requirements, such as on the one hand very high data rates for MBB user cases and on the other hand, ultra-reliability and very low latency for C-MTC use cases. Dual -connectivity is seen as one of the key enablers to meet these demanding requirements of the 5G use cases. However, further improvements are also needed, with respect to LTE Dual Connectivity (DC), to ensure seamless connectivity and robust mobility in addition to higher data rates.

# Discussion

In order to be able to provide very high data rates to end users, NR will be designed to operate also in high carrier frequencies where wider spectrum bands are available. Exploiting the high frequencies has an impact on the propagation conditions. The path loss increases with higher frequency and suffers from worsen diffraction and higher outdoor/indoor penetration losses. This means that signals will have less ability to propagate around corners and penetrate walls. In addition, atmospheric/rain attenuation and higher body losses could also contribute to making the coverage of the NR spotty.

In order to compensate for the higher path loss at high frequencies and improve the signal quality, beamforming with massive antennas is needed. However, despite potential link budget gains using massive antennas, reliability of a system purely relying on beamforming and operating in higher frequencies becomes even more challenging, as the coverage might be even more spotty and sensitive to both time and space variations. Thus, the UE may experience sudden drops of SINR or even lose the connection for short time periods. If there is a coverage provided on another NR frequency, the issues mentioned above can be partially addressed by enabling NR dual connectivity. Similarly, LTE-NR interworking can help to alleviate the challenges of high-frequency NR deployments by means of a low-frequency LTE overlay.

If there is a coverage provided by a secondary node, NR dual-connectivity can partially resolve the issues caused by the spotty coverage of NR high-frequency nodes.

LTE-NR interworking can also help to alleviate the challenges of high-frequency NR deployments by means of a low-frequency LTE overlay.

The UP interruption time is defined as [1]:

Mobility interruption time means the shortest time duration supported by the system during which a user terminal cannot exchange user plane packets with any base station during transitions.

The target for mobility interruption time should be 0ms.

This KPI is for both intra-frequency and inter-frequency mobility for intra-NR mobility.

To achieve 0ms UP interruption for intra-frequency mobility, tight synchronization will be required between the nodes to avoid interference and could in those instances be handled by beam management procedures for intra-gNB mobility between different TRPs.

0ms UP interruption for intra-frequency mobility can be achieved with beam management procedures between different TRPs in the same gNB

For inter-frequency mobility, DC could assist in reducing the UP interruption time as discussed in [3]. All intra MeNB mobility can be handled with 0ms interruption by allocating split bearer and adding, modifying and removing SCG.

The agreement that security key refresh is not needed when performing mobility without moving the PDCP anchor point is implicit.

0ms UP interruption for inter-frequency mobility can be achieved with dual connectivity for intra MeNB mobility

For inter MeNB mobility, a role switch of the SeNB to MeNB during mobility was suggested to be studied.

## Reconfigure MeNB to an SeNB to target the 0 ms UP interruption

In LTE, the only way to reconfigure from an MeNB to an SeNB is to release the SeNB and perform a handover to that eNB. As discussed in [2], this results in an UP interruption while the UE connects to the new eNB.

In LTE DC, the procedure for reconfiguring SeNB to MeNB results in a UP interruption while the UE connects to the new eNB.

For NR, it has been proposed that a role switch from SeNB to MeNB should be possible [3]. However, there are several implications which need to be addressed in order to achieve this.

When a UE is connected with DC, the MeNB uses the security key KeNB while the SeNB uses the S-KeNB which is derived from the KeNB. If the MeNB is changed, the KeNB will be updated and so will the S-KeNB. To ensure the security, the keys are updated every time the UE changes eNB based on the Next Hop (NH) parameter provided by the CN. In order to synchronize the use of the security key, the UE has to perform a random access to the new eNB. Thus, there will inevitably be an interruption when the keys are updated.

Updating the security KEY in MeNB and SeNB simultaneously will lead to some interruption during RA.

To avoid the UP interruption, it should be studied whether the key for the SeNB can be updated during the role switch without releasing the Source MeNB. Since the MeNB and SeNB are inter-frequency, the UE doesn’t have to turn off its transmission and reception to one eNB when performing the RA to the other eNB. Once the Target eNB has synchronized the new key, the Source MeNB can be released or reconfigured as a SeNB with an updated security KEY.

In LTE Rel-14, there is currently work in the WI Further mobility enhancements in LTE to reduce the interruption time during handovers where solutions based on RACH-less handovers and make-before-break procedures are considered. However, these solutions only consider the intra-frequency scenario [4].

- RAN2 should study how the Rel-14 mobility enhancement solutions with RACH-less handover and make-before-break procedures could be used for inter-frequency handover to SeNB in NR.

## SeNB to MeNB reconfiguration for LTE-NR DC

In addition, it is essential to consider the early deployments where there is both LTE and NR coverage. In this case, if a procedure for SeNB to MeNB reconfiguration is agreed for NR dual-connectivity, then it could be also beneficial for certain deployments to enable the same procedure for LTE-NR tight-interworking.

- If a procedure for SeNB to MeNB reconfiguration is agreed for NR dual-connectivity, then it could be also beneficial to enable the same procedure for LTE-NR tight-interworking.

# Conclusion

In section 2 we made the following observations:

Observation 1	If there is a coverage provided by a secondary node, NR dual-connectivity can partially resolve the issues caused by the spotty coverage of NR high-frequency nodes.

Observation 2	LTE-NR interworking can also help to alleviate the challenges of high-frequency NR deployments by means of a low-frequency LTE overlay.

Observation 3	0ms UP interruption for intra-frequency mobility can be achieved with beam management procedures between different TRPs in the same gNB

Observation 4	0ms UP interruption for inter-frequency mobility can be achieved with dual connectivity for intra MeNB mobility

Observation 5	In LTE DC, the procedure for reconfiguring SeNB to MeNB results in a UP interruption while the UE connects to the new eNB.

Observation 6	Updating the security KEY in MeNB and SeNB simultaneously will lead to some interruption during RA.

Based on the discussion in section 2 we propose the following:

Proposal 1	RAN2 should study how the Rel-14 mobility enhancement solutions with RACH-less handover and make-before-break procedures could be used for inter-frequency handover to SeNB in NR.

Proposal 2	If a procedure for SeNB to MeNB reconfiguration is agreed for NR dual-connectivity, then it could be also beneficial to enable the same procedure for LTE-NR tight-interworking.

# Reference

TR 38.913, Study on Scenarios and Requirements for Next Generation Access Technologies; (Release 14).

R2-168299, How to realize zero ms UP interruption in NR, Ericsson, 3GPP TSG-RAN WG2 #96, Reno, Nevada, USA, 14th – 18th November 2016.

R2-168564, 0ms mobility interruption in NR, Huawei, 3GPP TSG-RAN WG2 #96, Reno, Nevada, USA, 14th – 18th November 2016.

R2-168986, Report from LTE Break-Out session (eVoLTE, Light conn, Mobility enh, eMBMS), TSG-RAN WG2 #96, Reno, Nevada, USA, 14th – 18th November 2016.