3GPP TSG-RAN WG2 #97	Tdoc R2-1700914

Athens, Greece, 13th – 17th February 2017

Agenda Item:	10.2.2.5

Source:	Ericsson

Title:	Security in LTE-NR interworking

Document for:	Discussion, Decision

# Introduction

Contribution R2-1701056 analyses the impacts on LTE for supporting potentially new 5G-CN/RAN security mechanism. The contribution concludes that as long as the current key derivation principles within LTE access are reused also for 5G-RAN there should be no issues to inter-work with legacy eNBs HW even if the 5G security introduces support for:

- New AKA and key derivation in the 5G-CN

- New integrity/encryption algorithms in RAN or CN

- Longer security keys (up to 256 bits) in RAN

- Support for UP integrity protection

The reason is that the LTE security is future proof, e.g. it is possible to add new algorithms and even UP integrity protection which does not need to be supported by all nodes. Additionally, the key handling in RAN uses 256 bits keys even though the current algorithms only use 128 bits.

This contribution is addressing some LTE and NR inter-working scenarios incl.

- Handover between LTE and NR connected to 5G-CN

- Dual connectivity between LTE and NR for both EPC and 5G-CN

# Discussion

The current LTE access security model is based on the following principles: The eNB receives a security key from the MME at initial context setup. This key is used to derive other keys in the eNB as well as used to derive S-KeNB to be used for SCG bearer for dual connectivity. When handover is performed the source node will generate a KeNB* key to be used by target node. The target node will also receive new Next Hop Key and Next Hop Chaining Counter from the CN to be used for future handovers.

If this model is also followed for 5G-RAN there should be no issues to support various LTE and NR inter-working scenarios. See some example below.

## Example: UE starts in LTE / EPC and then enters EN-DC with NR (using SCG bearer)

- UE attaches to EPC

- UE performs authentication, key agreement, generate keys etc.

- EPC setups UE context in RAN, includes a KeNB (256 bits)

- RAN security setup. RAN may configure UE to use integrity protection algorithm A and encryption algorithm B.

- RAN decides to setup DC with NR and derives S-KeNB (256 bits) using SCG counter = zero. The SeNB context is setup in NR gNB. The gNB decides to use encryption C algorithm for the SCG bearer. It is FFS is the SCG counter should be transferred to the SeNB so it can be provided to the UE using NR RRC, or if it should sent separately to the UE from the LTE MeNB.

- RRC reconfiguration to setup DC with NR, in the message the SCG counter as well as the SCG encryption algorithm

- UE derives S-KeNB

With the procedure above it is possible as in LTE DC to use different security algorithm on the NR SCG bearers (incl. different key size) than for MCG bearer. The security configuration can be transferred transparently through the LTE base stations.

The same solution can be applied in case LTE is connected to 5G-CN since neither the LTE eNB or NR gNB should care how the 256 bit KeNB was generated.

Currently for LTE DC the S-KeNB is always updated when the KeNB is updated. It is proposed to keep this principle also for DC with NR, since it is assumed that for 5G-RAN the KeNB would only need to be updated when the RAN node changes (PDCP is relocation), in this way unnecessary KeNB changes, causing S-KeNB changes can be avoided. Solutions allowing independent change of S-KeNB would most likely add complexity.

It is proposed to adopt similar principle for KeNB and S-KeNB handling as for DC in LTE. As a baseline the S-KeNB should be changed when the KeNB is changed,

## Example: UE starts in LTE / 5G-CN and then is handed over to NR

- UE attaches to 5G-CN

- UE performs authentication, key agreement, generate keys etc.

- 5G-CN setups UE context in RAN, includes a KeNB (256 bits)

- RAN security setup. RAN may configure UE to use integrity protection algorithm A and encryption algorithm B.

- RAN decides to perfom handover to NR

- Handover request procedure over Xn. RAN sends KeNB* to NR gNB. NR gNB decides on which integrity protection algorithm and encryption algorithm to use and generates the “RRC handover to NR command”.

- The eNB sends a RRC reconf with a mobility info (“RRC handover to NR command”).

- UE enters target cell and send an RRC reconf complete message.

- NR gNB informs the 5G-CN about the handover and receives new keys to use for next handover.

With the procedure above it is possible to use different security algorithm in the NR node compared to the LTE node. The security configuration can be transferred transparently through the LTE base stations.

The KeNB can be generated in a different way in the 5G-CN compared to EPC without impacts on LTE.

It is proposed to adopt similar principle for key derivation within the 5G RAN as in LTE (e.g. KeNB* derivation based on KeNB or NH) assuming SA3 concludes the solution is secure.

# Conclusion

- With the procedure above (2.1) it is possible as in LTE DC to use different security algorithm on the NR SCG bearers (incl. different key size) than for MCG bearer. The security configuration can be transferred transparently through the LTE base stations.

The same solution can be applied in case LTE is connected to 5G-CN since neither the LTE eNB or NR gNB should care how the 256 bit KeNB was generated.

- It is proposed to adopt similar principle for KeNB and S-KeN B handling as for DC in LTE. As a baseline the S-KeNB should be changed when the KeNB is changed,

With the procedure above (2.2) it is possible to use different security algorithm in the NR node compared to the LTE node. The security configuration can be transferred transparently through the LTE base stations.

The KeNB can be generated in a different way in the 5G-CN compared to EPC without impacts on LTE.

It is proposed to adopt similar principle for key derivation at mobility within the 5G RAN as in LTE (e.g. KeNB* derivation based on KeNB or NH) assuming SA3 concludes the solution is secure.