3GPP TSG-RAN WG2 #97	 R2-1700997

Athens, Greece, Feb. 13th- Feb. 17th, 2017

Agenda Item:	10.4

Source: 	Xiaomi

Title:  	Cell Selection and Reselection Consideration for Network Slicing

Document for:	Discussion and Decision

Introduction

During last RAN2 meeting, RAN2 had the following agreements regarding to cell reselection for NR:

1 As in LTE, UE can prioritise a frequency based on service. On the selected frequency the UE attempts to camp on the best cell. 2 Suitability criterion: Cell quality is above a threshold; Cell is not barred; Cell belongs to selected/R (E) PLMN. Other conditions (if any) are FFS. 3 Cell broadcasts (e.g. in minimum SI) the service(s) supported by it. FFS for which services (e.g. MBMS, CSG, V2X, URLLC) we apply this mechanism. FFS how this might apply for the case of network slices.

In this contribution, we will look at the cell selection and reselection aspects for network slicing.

Discussion

Before starting the discussion of cell selection & reselection, we first try to understand the scenarios for network slicing. In 22.864[1], it is required for the networking slicing that:

[PR 5.1.2.1-003] The 3GPP system shall be able to associate specific services, devices, UEs, and subscribers with a particular network slice.

In 23.799 [2], SA2 has also agreed that:

- Network slices may differ for features supported and Network Functions optimisations use cases. - Networks may deploy multiple Network slice instances delivering exactly the same optimisations and features as per but dedicated to different groups of UEs, e.g. as they deliver a different committed service and/or because they may be dedicated to a customer.

That is, network slicing may be used for one or both of the following purposes:

- To isolate different services

- To isolate different devices/UEs/subscribers

Observation 1: network slicing may be used either for isolating different services, or for isolating different devices/UEs/subscribers, or both.

If network slicing is to be provided to certain devices/users, who may be restricted to certain areas (e.g. industrial zones), it should be possible for operator to provide the same network slicing service to scattered zones under one frequency layer, as illustrated in Fig 1.

Fig 1 network slicing for user groups in restricted area

Observation 2: operator may deploy the same network slicing service to some restricted areas under one frequency layer.

Cell Selection

For cell selection, the PLMN and the associated RAT are indicated by NAS. If LTE is chosen, RAN performs cell selection based on one of the three cell selection procedure: initial cell selection, stored information cell selection, and cell selection when leaving RRC_CONNECTED state. The corresponding procedures are given as follows [3]:

a)	Initial Cell Selection

This procedure requires no prior knowledge of which RF channels are E-UTRA or NB-IoT carriers. The UE shall scan all RF channels in the E-UTRA bands according to its capabilities to find a suitable cell. On each carrier frequency, the UE need only search for the strongest cell. Once a suitable cell is found this cell shall be selected.

b)	Stored Information Cell Selection

This procedure requires stored information of carrier frequencies and optionally also information on cell parameters, from previously received measurement control information elements or from previously detected cells. Once the UE has found a suitable cell the UE shall select it. If no suitable cell is found the Initial Cell Selection procedure shall be started.

c)	cell selection when leaving RRC_CONNECTED state

On transition from RRC_CONNECTED to RRC_IDLE, UE shall attempt to camp on a suitable cell according to redirectedCarrierInfo, if included in the RRCConnectionRelease message. If the UE cannot find a suitable cell, the UE is allowed to camp on any suitable cell of the indicated RAT. If the RRCConnectionRelease 	message does not contain the redirectedCarrierInfo UE shall attempt to select a suitable cell on an EUTRA carrier. If no suitable cell is found according to the above, the UE shall perform a cell selection starting with Stored Information Cell Selection procedure in order to find a suitable cell to camp on.

The definition of suitable cell is given by:

A "suitable cell" is a cell on which the UE may camp on to obtain normal service. The UE shall have a valid USIM and such a cell shall fulfil all the following requirements. - The cell is part of either: - the selected PLMN, or: - the registered PLMN, or: - a PLMN of the Equivalent PLMN list - For a CSG cell, the cell is a CSG member cell for the UE; According to the latest information provided by NAS: - The cell is not barred; - The cell is part of at least one TA that is not part of the list of "forbidden tracking areas for roaming", which belongs to a PLMN that fulfils the first bullet above; - The cell selection criteria are fulfilled

As we can see from the procedures, LTE will check CSG but not services like MBMS, sidelink discovery/communication for cell selection. The underline reason is that a UE cannot acquire normal service if the cell is not a CSG member cell, whereas a UE can acquire other normal services even if a cell doesn't provides MBMS/sidelink services that UE interests. The principle is for UE to acquire at least some of the normal services as soon as possible.

For network slicing, it is possible that all the UE normal services are provided through slices. For example, IoT devices may have only one or very limited services. Therefore, network can configure a IoT device with one slice to cope all the services. In a general case, network may configure a UE with a default slice and a couple of specific slices. The specific slices are for some specific traffic (eMBB, URLLC), the default slice is for traffic otherwise. This case is agreed by SA2 [2]:

The network operator may provision the UE with network slice selection policy (NSSP). The NSSP includes one or more NSSP rules each one associating an application with a certain SM-NSSAI. A default rule may also be included which matches all applications and contains a default SM-NSSAI. The UE uses the NSSP to associate UE applications with SM-NSSAIs.

If all the services of a UE are mapped to network slices, the UE should only try to camp on cells that support at least some of the slices so as to acquire normal service.

Proposal 1: Upon cell selection, the selected cell should support at least part of the network slices of a UE whose traffic are all mapped to network slices.

Cell Reselection

In LTE, cell reselection is a two-step mechanism. UE first chooses a frequency/RAT with the highest priority that meets the inter frequency/RAT reselection criteria, then chooses a cell from the chosen frequency. The inter frequency/RAT reselection priorities handling takes into account the availability of the interested services (MBMS, sidelink, CSG), and consider the frequency that provides the interested services as the highest priority. The priorities among the highest priority frequencies are left to UE implementation. Once a frequency is chosen, UE reselects the best cell of that frequency to camp on for services other than CSG. For CSG, UE reselects the best cell among CSG member cells.

Last RAN2 meeting adopted the LTE two-step mechanism for cell reselection, a best cell is chosen on the selected frequency. But the applicable services are FFS, and it is also unclear whether it is applicable to network slicing.

In our understanding, a unified framework should be used as much as possible for cell reselection to accommodate different services. We think the two-step mechanism can be used as the baseline. For network slicing, UE prioritizes the frequencies that support the UE configured network slicing, and chooses a best cell on the selected frequency. Similar to the differentiated operation of CSG and MBMS, we should, for network slicing, consider whether a similar approach applied to CSG is adopted, i.e. checking the availability of the network slicing for the best cell. As we discussed in the former sub division, a UE's all normal services may be provided through network slicing. If UE doesn't check the availability of network slicing, UE may end up with camped on a best cell that none of UE's normal service can be initiated. Therefore, UE needs to check whether the best cell supports at least part of the UE configured network slices.

Proposal 2: Upon cell reselection, UE can prioritize a frequency that supports UE configured network slices, and choose the best cell that supports at least part of the configured network slices on that frequency, if all the UE's normal services are mapped to network slices.

Proposal 3: The check for the support of at least part of the configured network slices if all the UE's normal services are mapped to network slices is included in the suitability criterion for cell selection & reselection.

Conclusions

Based on the discussion, we have the following observations and proposals:

Observation 1: network slicing may be used either for isolating different services, or for isolating different devices/UEs/subscribers, or both.

Observation 2: operator may deploy the same network slicing service to some restricted areas under one frequency layer.

Proposal 1: Upon cell selection, the selected cell should support at least part of the network slices of a UE whose traffic are all mapped to network slices.

Proposal 2: Upon cell reselection, UE can prioritize a frequency that supports UE configured network slices, and choose the best cell that supports at least part of the configured network slices on that frequency, if all the UE's normal services are mapped to network slices.

Proposal 3: The check for the support of at least part of the configured network slices if all the UE's normal services are mapped to network slices is included in the suitability criterion for cell selection & reselection.

References

[1] TR 22.864, Feasibility Study on New Services and Markets Technology.

[2] TR23.799 Study on Architecture for Next Generation System.

[3] TS36.304, User Equipment (UE) procedures in idle mode.