3GPP TSG-RAN WG2 Meeting #97		R2-1701330

Athens, Greece, 13–17 February 2017

Agenda Item:	10.3.1.1.1

Source: 	MediaTek Inc.

Title:  	Measurement Events and Reporting for NR Mobility

Document for:	Discussion and decision

# Introduction

For DL-based mobility, the handover procedure is initiated by measurement events and reporting, which involve evaluation and comparison of cells (i.e. serving and neighbour cells). When beam forming and cell quality derivation is performed in one or both cells, the comparison of two cells becomes more complicated. In this contribution, we discuss measurement reporting and events for NR mobility. We first consider measurement events modified from those for LTE, and then introduce new conditions considering the set of qualified beams.

# Discussion

## Measurement events modified from LTE

In LTE, measurement events A1 to A6 are defined for intra-RAT handover. In NR, LTE measurement design can be considered as a baseline. That is, a single value derived from multiple measurement results can be adopted, based on which we can define measurement event corresponding to those in LTE. Moreover, cell quality can be derived (calculated) in different ways. In [3], weighted summation is proposed, which includes averaging as a special cases. Although weighted summation is a generalized method, it has higher complexity and signalling overhead. We believe that simple averaging works properly, and some simulation results are provided in [4].

Proposal 1:	For Intra-NR mobility in Connected Active, define measurement events A1 to A6, corresponding to those in LTE, based on a single value derived for each cell.

Proposal 2:	A UE derives the quality of a cell by averaging the quality of individual beams in a preconfigured subset of detected beams.

### Measurement events considering qualified beams

Moreover, a UE may want to down-prioritize a cell whose best beam is strong but other beams are weak. To achieve this, the set of beams with signal strength above a given threshold (i.e. qualified beams) may be considered in measurement event triggering conditions.

Proposal 3:	The set of beams above a given threshold may be taken into account when designing the measurement event triggering conditions.

To consider qualified beams in measurement event design, an intuitive way is to derive cell quality based on RSRP/RSRQ average over the set of qualified beams. However, this may not be a good idea. For example, assume cell A has three qualified beams and the third qualified beam has RSRP = threshold + δ, while cell B has two qualified beams and the third best beam has RSRP = threshold – δ (which is not “qualified” but still detectable). Obviously, the comparison between the two cells would be unfair if we count 3 beams in cell A and 2 beams in cell B.

Observation 1:	Deriving cell quality by averaging the set of beams above a given threshold may lead to unfair comparison between cells.

Instead of performing RSRP/RSRQ averaging over the set of qualified beams, we may consider the number of qualified beams as an additional triggering condition of measurement events. For example, the condition may state that the target cell for handover must have a preconfigured number of qualified beams. The number of qualified beams may be included in measurement report. Moreover, we propose to introduces auxiliary events (A1a to A6a), which introduce additional conditions about the number of qualified beams to main events (A1 to A6).

Proposal 4:	The number of qualified beams can be included in measurement report.

Proposal 5:	Introduce auxiliary measurement events A1a to A6a, addressing the numbers of qualified beams in serving and neighbour cells.

When an auxiliary measurement event is configured, its parameters may be different from that for main event. For example, when A3a event is configured, a lower RSRP offset may be used to allow more aggressive handover decision when the neighbour cell does have a number of qualified beams.

Based on the above discussions, our proposed measurement events are shown in the table below.

Table 1. Measurement events based on absolute thresholds

| Measurement events in LTE | Measurement events in NR |
| --- | --- |
| A1: Serving cell becomes better than threshold | A1: Average of N best beams of serving cell becomes better than threshold. A1a: A1 & Number of qualified beams in serving cell is no less than a preconfigured value. |
| A2: Serving cell becomes worse than threshold | A2: Average of N best beams of serving cell becomes worse than threshold A2a: A2 & Number of qualified beams in serving cell is less than a preconfigured value |
| A3: Neighbour cell becomes offset better than PCell | A3: Average of N best beams of neighbour cell becomes offset better than that in PCell. A3a: A3 & the number of qualified beams in neighbour cell is more than that in PCell. |
| A4: Neighbour cell becomes better than threshold | A4: Average of N best beams of neighbour cell becomes better than threshold. A4a: A4 & Number of qualified beams in neighbour cell is no less than a preconfigured value. |
| A5: Serving cell becomes worse than threshold1 and neighbour becomes better than threshold2 | A5: Average of N best beams of serving cell becomes worse than threshold1 and weighted sum of N best beams of neighbour cell becomes better than threshold2. A5a: A5 & Number of qualified beams in serving cell is less than a preconfigured value1 and number of qualified beams in neighbour cell is no less than a preconfigured value2. |
| A6: Neighbour cell becomes offset better than SCell | A6: Average of N best beams of neighbour cell becomes offset better than that of SCell. A6a: A6 & Number of qualified beams in neighbour cell is more than that in SCell. |

Note that in NR measurement events, the term of “N best beams” includes both cases of “the best beam” as well as “all detected beams”. The set of beams selected from the detected beams in order to derive a cell level quality should be configured by the gNB.

Table 1 shows the concept of measurement event design for NR. In formal measurement event definitions, there are indeed more parameters such as cell-specific and frequency-specific offsets and hysteresis. Also, signalling for parameters and measurement event configurations need to be designed. We will work on these details once the proposed measurement events are agreed.

Proposal 6:	If the proposed measurement events are agreed, provide formal definitions and signalling designs for related parameters and measurement event configurations.

# Conclusion

We have the following observation:

Observation 1:	Deriving cell quality by averaging the set of beams above a given threshold may lead to unfair comparison between cells.

It is proposed to discuss and decide on the following proposals:

Proposal 1:	For Intra-NR mobility in Connected Active, define measurement events A1 to A6, corresponding to those in LTE, based on a single value derived for each cell.

Proposal 2:	A UE derives the quality of a cell by averaging the quality of individual beams in a preconfigured subset of detected beams.

Proposal 3:	The set of beams above a given threshold may be taken into account when designing the measurement event triggering conditions.

Proposal 4:	The number of qualified beams can be included in measurement report.

Proposal 5:	Introduce auxiliary measurement events A1a to A6a, addressing the numbers of qualified beams in serving and neighbour cells.

Proposal 6:	If the proposed measurement events are agreed, provide formal definitions and signalling designs for related parameters and measurement event configurations.

# Reference

- RP-160671, New SID Proposal: Study on New Radio Access Technology

- R2-160092, Downlink Measurements for NR Mobility, MediaTek Inc.

- R2-168042, Cell Quality Derivation based on Measurements from Individual Beams, Samsung

- R2-1700274, Performance Evaluation for NR Mobility, MediaTek Inc.

- TR 38.913,  Study on Scenarios and Requirements for Next Generation Access Technologies

- 3GPP TS 36.331, Radio Resource Control (RRC) Protocol specification, Release 13