3GPP TSG-RAN WG2 #97	R2-1701357

# Athens, Greece, 13th – 17th February 2017

Agenda Item:	8.13.8

Source:	Samsung

Title:	V2X Multi-carrier operation: UECapabilityInformation enhancement

Document for:	Discussion, Decision

Introduction

In the V2X conference call 1 (11th Jan. 2017), the agreeable proposals were derived as following:

Agreable Proposals

- Support mode-4 inter-carrier configuration for V2x sidelink communication

- Use both SIB21 and RRC dedicated signalling to carry the inter-carrier configuration for mode-4

- For inter-carrier configuration of mode-4, the selection of the carrier and corresponding TX/RX resource pool is up to UE implementation.   FFS if some additional mechanisms/configuration is needed.

- No additional enhancement is needed for mode-3 to support inter-carrier configuration.  FFS whether more than one resource pool needs to be configured for mode-3 to allow configuration of pools with different synchronization types.

- Support inter-carrier configuration of RX resource pool for V2x sidelink communication

- Proposal 6 and 7 need to be discussed together and will be treated in the next meeting based on contributions.

- Enable the UE to read from other PLMNs the RX resource pool configuration.

- The serving eNB can indicate to the UE the RX resource configuration for inter-PLMN operation directly.    FFS what information is indicated (e.g. provide the frequency which the UE should read the resource configuration from or the full configuration).   FFS if the eNB can indicate the TX resource configuration/pool, provide just the frequency for tx pool acquisition, or nothing for inter-PLMN operation.

- Enhance legacy ProSe capability signalling to multi-carrier sidelink V2V operations

- Enhance SidelinkUEInformation signalling to allow UE to report multiple interested carriers for V2X sidelink transmission/reception.  FFS what and how this information is used or chosen by the UE (e.g. safety/non-safety).

- FFS what is the maximum number of carriers to be configured for V2x PC5 communication.

Based on the assumption that these proposals would be agreed at the coming meeting RAN2#97, we discuss on the enhancement of UECapabilityInformation msg for supporting multi-carrier operation in this contribution.

Discussion

Current UECapabilityInformation msg is based on the single frequency operation on sidelink. There are three indicators related to sidelink operation.

- commsimultanesouTX = {indicates simultaneous TX for Uu and sidelink },

- commSupportedbands = {frequency band supported by the UE which is single band. },

- commSupportedBandperBC = {bit map which indicates the frequency band in commSupportedbands available for simultaneous TX or RX with Uu}

However, V2X inter-frequency operation assumes, at least, simultaneous multiple carrier RX operation (and multiple TX operation might be determined further in this meeting). Therefore, it is natural to extend the UEcapabilityInformation msg to have 1. indication on simultaneous TX or RX bands for sidelink only and 2. the band combination on simultaneous sidelink operation with Uu operation (based on 1 information).

If this information is given to eNB, then eNB can reflect this to the resource allocation and scheduling by considering the availability of simultaneous usage of sidelink frequency and Uu frequency.

In summary, commSupportedbands and commSupportedBandperBC in current UECapabilityInformation msg should be extended to include simultaneous sidelink operation.

Proposal 1: UECapabilityInformation message should be extended to include the sidelink bands available for simultaneous RX or simultaneous TX, and band combination information which indicates sidelink bands available for simultaneous operation with Uu operation.

Conclusion

Based on the above discussion, we have the following proposal.

Proposal 1: UECapabilityInformation message should be extended to include the sidelink bands available for simultaneous RX or simultaneous TX, and band combination information which indicates sidelink bands available for simultaneous operation with Uu operation.