3GPP TSG-RAN WG2 #97 Meeting                                                       R2-1701475

Athens, Greece, 13–17 February 2017

Agenda item:	10.2.1.4

Source:	Samsung

Title:	Bandwidth configurations for NR single carrier wideband

Document for:	   Discussion

Introduction

In RAN1#86bis, the following were agreed regarding wider bandwidth for NR [1];

Agreements:

- At least for Phase 1, study mechanisms to support operation over e.g. around 1GHz contiguous spectrum from both NW and UE perspectives including the maximum single carrier bandwidth of at least 80 MHz

  - Carrier Aggregation/Dual Connectivity (Multi-carrier approach)

    - Details are FFS

    - FFS: non-contiguous spectrum case

  - Single carrier operation

    - Details are FFS

    - Maximum channel bandwidth continues to be studied in RAN1/4

      - Maximum bandwidth supported by some UE capabilities/categories may be less than channel bandwidth of serving single carrier

        - Note that some UE capabilities/categories may support channel bandwidth of serving single carrier

- Send an LS to ask RAN4 to study the feasibilities of mechanisms above from both NW and UE perspectives

Agreements:

- At least for single carrier operation, NR should allow a UE to operate in a way where it receives at least downlink control information in a first RF bandwidth and where the UE is not expected to receive in a second RF bandwidth that is larger than the first RF bandwidth within less than X µs (FFS: value of X)

  - FFS the first RF bandwidth is within the second RF bandwidth

    - FFS the first RF bandwidth is at the center of the second RF bandwidth

  - FFS the maximal ratio of the first RF bandwidth over the second RF bandwidth

  - FFS detailed mechanism

- FFS RF bandwidth adaptation for RRM measurement

In RAN1#87, the following LS to RAN4 was sent regarding wider bandwidth for NR [2];

Agreements:

- Send LS to RAN4 to study the following points for single/multi-carrier operation

  - How fast is the UE RF bandwidth adaptation

  - How much power saving is possible for UE RF bandwidth adaptation

  - Other benefits

  - Whether any of the above depends on the conditions, such as

    - Whether or not first and second RF bandwidth are centered at the same frequency

    - Whether or not first RF bandwidth are partially or fully contained in the second RF bandwidth

    - The ratio of first and second RF bandwidth

    - Whether or not first and second RF bandwidth are in the same band

    - Dependency of modulation scheme

    - Whether or not neighbor cell synchronization signals are within first RF bandwidth

    - Whether or not first and/or second RF bandwidth are centered at the same frequency as neighbor cell synchronization signals

    - Whether or not additional reference signals are needed, for example for AGC settling

    - Whether it depends on transmission direction

- LS includes corresponding agreements/WA for DL control in past meetings

  - Note: RAN1 is under discussion on RF bandwidth adaptation for DL data and UL control/data

In RAN1 NR Adhoc, the following were agreed

Agreements:

- For frequency range category #1 (below 6 GHz) where [15 kHz, 30 kHz, 60 kHz] are candidate subcarrier spacing values:

  - Candidate minimum NR carrier bandwidth are [5 MHz, 10 MHz, 20 MHz]

  - Candidate transmission bandwidth of each synchronization signal are about [1.08 MHz, 2.16 MHz, 4.32 MHz, 8.64 MHz]

- For frequency range category #2 (above 6 GHz) where [120 kHz, 240 kHz] are candidate subcarrier spacing values:

  - Candidate minimum NR carrier bandwidth are [20 MHz, 40 MHz, 80 MHz]

  - Candidate transmission bandwidth of each synchronization signal are about [8.64 MHz, 17.28 MHz, 34.56 MHz, 69.12 MHz]

- The above frequency range categories may be further divided into different categories with different parameters

- FFS on bandwidth of additional synchronization signal(s) if defined

- NR minimum carrier bandwidth for carrier which does not support initial access is FFS

FFS: UE bandwidth

This contribution shares, for informative purpose, physical layer aspects for configuring various BWs for the single carrier operations in NR.

Discussion

The following terminology is used in this contribution.

BW region – Size and location of a bandwidth under consideration

Synch BW - The location and transmission bandwidth of NR synchronization signal which is about [1.08 MHz, 2.16 MHz, 4.32 MHz, 8.64 MHz] for below 6GHz and about [8.64 MHz, 17.28 MHz, 34.56 MHz, 69.12 MHz] for above 6GHz.

Initial Access BW – BW region where UE performs initial measurements and random access, paging, system information related receptions and associated procedures. This includes the synch BW.

RRM BW – This is the BW region an RRC_CONNECTED mode UE monitors for performing RRM measurements (for e.g. handover purposes). The RRM BW can be in the same BW region or in different BW regions across cells.

1st RF and 2nd RF BW – These are UE specific BW regions that can be configured after initial access for connected mode UE’s. While the 1st RF BW is for NR-PDCCH, NR-PDSCH transmissions, the 2nd RF BW can be used for both NR-PDCCH (if deemed necessary beyond the contents of the 1st RF BW) and NR-PDSCH transmissions. Other usage cases, such as for CSI-RS, are not precluded.

BW’s monitored by RRC_CONNECTED UE: RRM BW, 1st and 2nd RF BW

In this contribution, we focus mainly on the DL aspects of BW adaptation.

Bandwidths

While NR considers communications, it is not necessary for a UE to always monitor a wide BW [2] and the 1 GHz single carrier operation can be accompanied by the 1st RF and 2nd RF BW operations/configurations to realize UE power savings (a main KPI for NR). Aspects to be considered when configuring the 1st RF BW and 2nd RF BW to a UE are subsequently discussed.

Initial Access BW

Initial Access BW region is one where synch signals are located (i.e., synch BW) andall the UE procedures, before dedicated resources are configured, are supported. This may include NR-PDCCH and possibly NR-PDSCH decoding for random access response, connection establishment, system information delivery and paging. Otherwise, the UE will have to retune its RF frequently and the associated signaling overhead is not negligible either. However, additional NR-PDSCH resources outside of the initial access BW may be considered for enhancing the system capacity. For instance, NR-PDCCH in the initial access BW indicates the NR-PDSCH resources outside the initial access BW. The RF re-tune time should be taken into account when choosing the NR-PDSCH resource in this case.

A wide bandwidth allows to support a large number of users and thereby increases the capacity to support a massive number of users e.g. paging capacity of the system. But if the UE monitors such a wide band, the power consumption massively rises due to the hardware complexities (high sampling rate, filters etc.). The issue is also relevant for the case when UEs are required to monitor a PDSCH message that does not contain it’s (a UE’s) own identity as in the case of paging. Considering these aspects RAN2 should consider whether there is any requirement for NR-PDSCH BW to be different from and larger than the NR-PDCCH BW for initial access purposes.

As per the RAN1 agreements shown above, the synch BW is used for RSRP and RSRQ measurements by RRC IDLE mode UEs. Furthermore, the synch signals will be used for connected mode RRM measurements as well in addition to other reference signals (if needed).

Observation 2: The minimum BW for a UE is equal to or larger than the synch BW.

Observation 3: A RRC Idle mode UE only monitors the initial access BW region.

Observation 4: The RF re-tuning time and the associated complexity must be considered when designing the NR-PDCCH and NR-PDSCH for initial access purposes.

RRM BW

The RRM BW is used by the RRC Connected mode UEs for performing RRM measurements (for e.g. handover purposes). As agreed in NR-Adhoc [2], the RRM measurements can be performed with NR synch signals and other signals such as CSI-RS or wideband RS if needed. Therefore, the RRM BW can be a) same as the synch BW or b) larger than the synch BW. When synch signals are used, the RRM BW is same as the synch BW. For case (b), when wideband RS is used, the bandwidth of this RS may be configured to be the full BW so that measurement accuracy can be higher. In such cases, the wideband RS can be used for connected mode RRM measurement within UE allotted BW. Hence, the RRM BW size is greater than or equal to the synch BW

Observation: RRM BW is equal to or larger than the synch BW.

It is generally desirable to avoid having a UE to re-tune in order to perform inter-cell measurements. This will also require some network coordination where the gNB indicates to UEs the RRM BW for adjacent cells. It is therefore beneficial if RRM measurements are performed over the BW that should preferably be common among cells [5].

Proposal: UE shall be able to assume cell-common RRM BW configurations across cells, for inter-cell measurement.1st RF BW

As mentioned before, the 1st RF BW is configured only for a connected mode UE. One issue is whether or not the 1st RF BW can be smaller than the initial access BW. Depending on the use cases for the 1st RF BW, its BW values can be defined.

The following are some of the purposes identified for the 1st RF BW:

- Monitor the UE common information (for RRC Connected UE’s),

- Monitor the common per-beam information in above 6GHz systems

- Support RRM measurements (as will be discussed shortly, this is needed if the RRM BW is inside the 1st RF BW)

Therefore, the 1st RF BW can overlap with the RRM BW (or initial access BW) region

RRM considerations for 1st RF BW

For RRM measurements when a UE is using the 1st RF BW, the following options exist: (i) RRM BW is inside the 1st RF BW, (ii) RRM BW is separate from the 1st RF BW, and (iii) new RS inside the 1st RF BW for measurement purposes.

For option (i), the 1st RF BW of UEs overlaps with the RRM BW that is provided by the network. For example, when the RRM BW spans the entire BW, then there are RRM signals present inside the UE’s 1st RF BW that enable RRM measurements. For these cases i.e., when the RRM BW spans the entire NW BW, no new configurations are needed from the NW side for configuring the 1st RF BW for RRM measurements. However, when the RRM BW is the same as the synch BW (as mentioned earlier, when SSS is used for RRM measurements), then the network has to configure the 1st RF BW for all UEs to overlap with the RRM BW region which can limit the ability to flexibly configure the 1st RF BW to UEs.

For option (ii), the 1st RF BW is separated from the RRM BW. This case occurs when the RRM BW is same as the synch BW. In such a setting, the NW can configure measurement gaps while accounting for a re-tuning delay necessary for a UE to switch its RF from 1st RF BW region to the RRM BW region. This can increase the burden on the NW side and also increases the power consumption at the UE due to this frequent re-tuning.

For option (iii), UE re-tuning is avoided but the NW needs to provide new RS for measurements (measurement RS) inside each 1st RF BW region. This option increases the signaling overhead.

Considering the above options and associated tradeoffs, option (iii) seems least favorable considering the signaling overhead. Considering the fact that option (ii)-type configuration is already supported in LTE (e.g. for eMTC), the gNB can decide to either support option (i) or option (ii).

Observation: RRM BW inside the 1st RF BW or separate from the 1st RF BW with support of UE retuning are viable options for a network to select.

Neighbor cell RRM signals may not be transmitted in the same BW as the ones for the serving cell. Hence, it is not necessary that the neighbor cell RRM BW falls within the 1st RF BW for a UE and in such case, for inter cell measurements, the UE may need to re-tune during measurement gaps [5]. Depending on the input from RAN4 for the re-tuning timelines (both inter-carrier and intra-carrier), appropriate configurations for the 1st and 2nd RF BW can be designed.

Observation 8: The 1st RF BW for a UE need not be mandated to include RRM BWs for all neighbor cells.

Observation 9: Depending on whether or not the neighbor cell RRM BW is within the 1st RF BW, measurement gaps need to consider RF re-tuning time.

2nd RF BW

Just like the case of 1st RF BW, the measurement gaps need to account for the time durations needed for re-tuning (if needed, for example in inter-carrier cases) when the UE operates in the 2nd RF BW.

The measurement gaps for RRM purposes must be set considering all these aspects into consideration specifically whether the RRM BW is within the 1st RF or 2nd RF BW or not even in the case of intra-frequency measurement.

Observation 10: Different from LTE, the measurement gap settings in NR should be based on the RRM BW configurations and the 1st, 2nd RF BW configurations even in the case of intra-frequency measurements.

Proposal 1: RAN2 takes into account single carrier wideband operation when designing radio interface protocols.

Summary

In this contribution, we discussed various aspects about supporting wider BW for NR and the following observations/proposals are made –

Proposal 1: RAN2 takes into account single carrier wideband operation when designing radio interface protocols.

Observation 2: The minimum BW for a UE is equal to or larger than the synch BW.

Observation 3: A RRC Idle mode UE only monitors the initial access BW region.

Observation 4: The RF re-tuning time and the associated complexity must be considered when designing the NR-PDCCH and NR-PDSCH for initial access purposes.

Observation 5: RRM BW is equal to or larger than the initial access BW.

Observation 6: Cell common RRM BW is preferable, at least for the same frequency scenario.

Observation 7: RRM BW inside the 1st RF BW or separate from the 1st RF BW with support of UE retuning is viable options for a network to select.

Observation 8: The 1st RF BW for a UE need not be mandated to include RRM BWs for all neighbor cells.

Observation 9: Depending on whether or not the neighbor cell RRM BW is within the 1st RF BW, measurement gaps need to consider RF re-tuning time.

Observation 10: Different from LTE, the measurement gap settings in NR should be based on the RRM BW configurations and the 1st, 2nd RF BW configurations even in the case of intra-frequency measurements.

References

[1] RAN1 Chairman’s Notes, RAN1#86bis, RAN1 Chairman, October 2016.

[2] RAN1 Chairman’s Notes, RAN1#87, RAN1 Chairman, November 2016.

[3] R1-1612474, NR mobility in idle mode, Samsung

[4] R1-1612470, Mobility RS design for supporting multi-beam operations, Samsung

[5] R1-1612471, NR mobility in connected mode, Samsung

[6] R1-1612436 Wide Bandwidth Operational Aspects, Samsung