3GPP TSG-RAN WG2 #97	Tdoc R2-1701563

Athens, Greece, 13th – 17th February 2017

Agenda Item:	10.2.2.4

Source:	Ericsson

Title:	Further analysis of NR minimum SI at network sharing

Document for:	Discussion, Decision

# Introduction

RAN2 has made the following agreements for NR minimum SI at the NR Ad hoc meeting in January:

1. Minimum SI includes at least SFN, list of PLMN, Cell ID, cell camping parameters, RACH parameters.

2. Scheduling information in minimum SI includes an indicator whether the concerned SI-block is periodically broadcasted or provided on demand.

3. Parameters required for requesting other SI-block(s) (if any needed, e.g. RACH preambles for request) if network allows on demand mechanism shall be included in minimum SI.

4. Cell-reselection neighbouring cell information is considered as other SI and can be provided on demand based on UE request.

Some estimates of the size of the NR Minimum SI have been included in different contributions. An LS was also sent to RAN1 in R2-1700654, where it was indicated that RAN2 expects several hundred bits to be required for the NR minimum SI, with at least around 250 bits for L2/L3 operation based on the current agreements.

This document contains further discussions on the content in the NR minimum SI and an estimate of the size that can be foreseen when supporting network sharing. The principles agreed for system information in NR need to cater for broadcast of minimum SI with the maximum size.

# Discussion

## NR minimum SI content related to network sharing

It is important that the UE can determine if a cell belongs to its PLMN and whether it is allowed in the cell prior to accessing the cell, including transmission of any SI request. Otherwise there would be cases where a lot of UEs, which are not even allowed in the cell, transmit requests for on-demand SI in the cell just to find out that they are not allowed there. As already agreed in RAN2 the NR minimum SI includes, among others, the PLMN list. The UE can thus determine what PLMNs that are sharing the cell by reading the broadcasted minimum SI. Access barring information then also needs to be broadcasted.

The PLMN list is part of the minimum SI and thus periodically broadcasted

Issues have been identified with the network sharing mechanisms in legacy LTE concerning the limitation that only a single Tracking Area and Cell ID can be defined per cell. By forcing a cell that is shared by several PLMNs to use the same Tracking Area and Cell ID the operators need to coordinate their Tracking Area Codes and Cell IDs, which requires quite some effort. In addition, with the shared cell(s) being part of a different tracking area than the surrounding cells of an operator, the UEs perform a Tracking Area update when entering and/or leaving the shared area/cell(s), which thus may lead to a lot of additional signalling.

It should therefore be possible to set  the Tracking Area Code and Cell ID per PLMN in the NR minimum SI.

It should be possible to set the Tracking Area Code and Cell IDs per PLMN in the PLMN list in the NR minimum SI

In LTE up to 6 different PLMNs are supported for network sharing. With the introduction of NR it can be foreseen that there will be more virtual operators and that 6 PLMNs may not be sufficient. There may also be a need for differentiation of services, which could as an option be addressed through different PLMN IDs.

NR supports sharing a cell with more than 6 PLMNs.

## Size estimate for NR minimum SI with network sharing

As not all parameters have been identified yet it is still premature to give any precise size of the NR minimum SI. Parts of the NR minimum SI will be included in the MIB and thus transmitted on the NR-PBCH. The remaining minimum SI will be transmitted on another channel, e.g. a secondary PBCH (NR-SPBCH). Table 1 contains a rough size estimate for the NR minimum SI that is not included in the MIB. The estimate, which is for network sharing with 6 PLMNs, is based on the agreed content for the NR minimum SI and the corresponding content and size in LTE.

|  | Parameter | Description | Size estimate (bits) |
| --- | --- | --- | --- |
|  | hyperSFN | Hyper System Frame Number (i.e as included in SIB1 in LTE) | 10 |
|  | PLMN list | PLMN list with 6 PLMNs, including cellReservedForOperatorUse | 161 (6*26 + 5) |
|  | TAC (per PLMN) | 6 Tracking Area Codes, one per PLMN | 96 (6*16) |
|  | Cell ID (per PLMN) | 6 Cell IDs, one per PLMN | 168 (6*28) |
|  | cellSelectionInfo | q-RxLevMin and q-RxLevMinOffset (optional) | 10 |
|  | cellBarred | Barring information | 1 |
|  | FreqBandIndicator | Frequency band indicator | 8 |
|  | schedulingInfoList | Scheduling of other SI messages, including SIB type, validity information, periodicity and indicator whether periodically broadcasted or provided on demand. Repeated per SI message, with 6 SI messages each containing 2 SIBs assumed here. | 84 (6*(3+5*2+1)) |
|  | si-WindowLength | SI window length, common to all SI messages | 4 |
|  | Access barring info | Access barring information for different categories. 16 categories, each 8 bits assumed here. | 128 (16*8) |
|  | onDemandReq info | Parameters for requesting on-demand SI, e.g. PRACH preambles. 4 separate PRACH preambles assumed. | 40 (4*10) |
|  | systemInfoValueTag | System Information value tag per SI message | 12 (6*2) |
|  | rach-ConfigCommon prach-Config | (P)RACH parameters | 60 |
|  | PCCH-Config | Paging parameters | 5 |
|  | BCCH-Config | SI modification period | 2 |
| Sum |  |  | 789 |

Table 1 Size estimate of NR minimum SI with 6 PLMNs

Different alternatives exist to compress the content of the system information and thus decrease the size. As proposed in [1] such methods for efficient encoding should be studied further. It can however be assumed that the maximum size of the minimum SI will be even larger than the 789 bits in Table 1. More parameters will probably be added during the WI phase and in future releases. More than 6 PLMNs should be supported in NR (see Proposal 2) and more than one version of some parameters in the minimum SI, which may differ between cells and/or beams, will need to be included.

The solution that is agreed for system information in NR needs to support broadcast of the minimum SI with the maximum size, which thus can be assumed to be more than the estimated 789 bits.

The size of the NR minimum SI, supporting network sharing of 6 PLMNs, can be estimated to 789 bits.

The system should support broadcast of NR minimum SI with the maximum size, i.e. a size of more than 789 bits

# Conclusion

In section 2 we made the following observations:

Observation 1	The PLMN list is part of the minimum SI and thus periodically broadcasted

Observation 2	The size of the NR minimum SI, supporting network sharing of 6 PLMNs, can be estimated to 789 bits.

Based on the discussion in section 2 we propose the following:

Proposal 1	It should be possible to set the Tracking Area Code and Cell IDs per PLMN in the PLMN list in the NR minimum SI

Proposal 2	NR supports sharing a cell with more than 6 PLMNs.

Proposal 3	The system should support broadcast of NR minimum SI with the maximum size, i.e. a size of more than 789 bits

# References

R2-1701564, Encoding of broadcasted NR system information, Ericsson, 3GPP RAN2#97, Athens, Greece, 13th-17th February 2017