3GPP TSG-RAN WG2 Meeting #97 	R2-1701614

# Athens, Greece 13 – 17 February 2017

Agenda Item:	8.10.1

Source:	Ericsson

Title:	MCCH and SI notification configuration for feMBMS carrier

Document for:	Discussion, Decision

# Introduction

The MCCH and SI were discussed in both RAN1 and RAN2 and the following agreements were reached:

RAN2:

Bcch-Config and PCCH-Config IEs should not be provided for FeMBMS SI if RAN1 decides to use L1 signaling to notify the SI modification.

The subsequent SI message should schedule as per configuration SIB15, SIB16, SIB10-12. FFS other SIBs.

RAN1:

MCCH change notification and SI modification notification are sent in PDCCH region of the CAS.

Inform RAN2 that SI modification notification can be conveyed to the UE with Direct Indication signalling.

For a Rel-14 FeMBMS carrier, the current DCI format 1-C used to indicate MCCH change, is extended to also indicate SI change.

This paper discusses periodicities and the MCCH and SI change notification configuration on the FeMBMS carrier.

# Discussion

It follows from the existing RAN1 and RAN2 agreements that a common configuration other than BCCH or PCCH config should be used for MCCH and SI change notification configuration. As MCCH and SI notifications have different periodicity options currently, in addition to the stage-3 details we should discuss which periodicities should be adopted for the common configuration.

On a legacy carrier, the MCCH information change notifications on PDCCH are transmitted periodically with occasions that are common for all configured MCCHs. This configuration is given by parameter notificationConfig in SIB13 which includes a repetition coefficient (notificationRepetitionCoeff), a radio frame offset (notificationOffset) and a subframe index (notificationSF-Index). For release 14 feMBMS, both SI and MCCH change notification configuration can be given by the notificationConfig.

For release 14 feMBMS, both SI and MCCH change notification configuration can be given by the notificationConfig.

The periodicity of the MCCH change notifications is based on the MCCH with the shortest modification period, where the possible MCCH modification periods are 512 or 1024 radio frames. The modification period for the BCCH is expressed (in number of radio frames) as modificationPeriodCoeff * defaultPagingCycle. Both parameters are transmitted as system information, where the modificationPeriodCoeff can take the values 2, 4, 8, 16 or 64 and the defaultPagingCycle the values 32, 64, 128 or 256 radio frames. Thus the shortest modification period is 64 radio frames and the longest is 16384 radio frames. As both mixed MBMS/unicast cell (80%MBMS) and the MBMS-dedicated cell only broadcast MBMS related system information, the shortest modification periods are not that relevant here. It could be assumed that MCCH information changes more often than the BCCH information on dedicated MBMS cell thus reusing MCCH modification for both MCCH and SI modification notifications periodicities could be acceptable from this perspective.

It could be assumed that MCCH information changes more often than the BCCH information on the dedicated MBMS cell thus reusing MCCH modification periodicities for both MCCH and SI modification notifications could be acceptable.

The ETWS has requirement that it needs to be delivered to the UE within 4s according to [3]. It is thus worth checking that ETWS can be delivered within this time. The shortest period for the MCCH notification is 512/4=128 radio frames (1.28seconds) and after receiving the notification UE immediately acquires SIB1-MBMS as per Section 5.3.2.3 and 6.6 in TS36.331. This is under 1.5 s leaving more than 2.5 s to deliver the SIB10 itself.

On an FeMBMS carrier, reuse MCCH change notification configuration and periodicity for the DCI format 1-C.

Have MCCH change notification configuration mandatory present in SIB1-MBMS.

Add MCCH change notification configuration optionally present in SIB1 used in mixed carrier

The subframe used to transmit MCCH change notifications is indicated with the notificationSF-Index parameter in SIB13. Since the option with no unicast region shall be supported on the FeMBMS carrier, the MCCH information change notifications cannot be transmitted on the MBSFN subframes. Instead, the non-mbsfn subframes containing the PDCCH should be used for transmission of MCCH change notifications.There is a RAN1 agreement for MBMS-dedicated cell from RAN1#87:

- MCCH change notification and SI modification notification are sent in PDCCH region of the CAS.

On the mixed MBMS/unicast carrier, the change notification can be sent on subframe #0 or in subframe #5 and on MBMS dedicated carrier, the change notification can be sent on subframe #0. As it is already possible to configure subframe #5 it only needs to add subframe #0.

Add subframe #0 to notificationSF-Index to be able to send MCCH and SI notification on subframe #0.

As these change notifications are now given in non-MBSFN subframes, the text in TS36.331 in Section 5.8.1.3 which states MCCH change notifications are only given in MBSFN subframes needs to be updated.

Adopt the changes as proposed in draft CR R2-1701599.

# Conclusion

Based on the discussion we have the following observations and proposals.

Observation 1	For release 14 feMBMS, both SI and MCCH change notification configuration can be given by the notificationConfig.

Observation 2	It could be assumed that MCCH information changes more often than the BCCH information on the dedicated MBMS cell thus reusing MCCH modification periodicities for both MCCH and SI modification notifications could be acceptable.

Proposal 1	On an FeMBMS carrier, reuse MCCH change notification configuration and periodicity for the DCI format 1-C.

Proposal 2	Have MCCH change notification configuration mandatory present in SIB1-MBMS.

Proposal 3	Add MCCH change notification configuration optionally present in SIB1 used in mixed carrier

Proposal 4	Add subframe #0 to notificationSF-Index to be able to send MCCH and SI notification on subframe #0.

Proposal 5	Adopt the changes as proposed in draft CR R2-1701599.

# References

R1-1613813, CR ST36.212, Editor(Huawei)

R1-1613815 CR TS36.213, Editor(Nokia)

3GPP TS 22.168 V9.0.0, “Earthquake and Tsunami Warning System (ETWS) requirements”

R2-1701599, Draft CR TS36.331 MCCH and SI notification configuration, Ericsson, Athens, Greece 13 – 17 February 2017

R1-1613816, LS on direct indication in DCI format 1C, RAN1 to RAN2, Reno, USA, November 14-18, 2016

http://www.gsma.com/mobilefordevelopment/wp-content/uploads/2013/01/Mobile-Network-Public-Warning-Systems-and-the-Rise-of-Cell-Broadcast.pdf