3GPP TSG-RAN WG2 #97	R2-1701756

Athens, Greece, 13th-17th February 2017

Agena Item:     7.1

Source:	Apple, Qualcomm Incorporated, Broadcom, Intel Corporation, Interdigital communications, Convida Wireless

Title:	Concurrent LAA/Wi-Fi Using RX Sharing

Document for:		Discussion and Decision

# 1	Introduction

The 3GPP Release 13 LAA feature [1] introduced new requirements for the UE to support secondary carriers RX operations in 5GHz unlicensed band. At the same time, one of the objectives of the LAA Work and Study Items was to provide a solution for the UE to efficiently support LAA operations in parallel to Wi-Fi operations.

In this contribution, we aim to describe the LAA/WiFi RX ressource sharing problems in the UE, analyze its impact on user’s perfromance and propose a solution for the UE and eNB to mitigate the problem.

# 2    Discussion

In the following, we summarize the past 3GPP discussions on LAA/Wi-Fi in-device coexistence, describe a new RX sharing issue, and analyze its impact on LAA performances:

## 2.1 LAA/Wi-Fi In-Device Interferences & IDC Framework

In RAN2#89-bis [2], and #90 [3] a discussion took place on the Wi-Fi & LAA interference problem. Two main issues have been identified:

- Adjacent Carriers Operations.

- Overlapping Carriers Operation.

It has been argued that the Overlapping Carriers Operation issue could be solved by the LBT procedure, while the Adjacent Carrier’s Operation is a marginal intereference problem, hence it could be solved by the IDC framework.

Observation 1: Previous RAN2 discussions on LAA/Wi-Fi in-device co-existence focused on the intereference problem causing desense when both UE’s Wi-Fi and LAA operate in adjacent 5GHz channels.

3GPP IDC framework [4] was standardized in Release 11 to address Wi-Fi/BT and Licensed LTE in-device intereferences problem for certain combinations of frequencies in 2.4GHz. This framework introduced RRC signalling and also multiple intereferences mitigations procedures in the UE and eNB.

However, the deployment of such framework is very limited mainly because of the progress in UE’s autonomous interference mitigation technics, such as advanced RF filters and adaptive transmission power.

Observation 2: IDC framework exists since Release-11 for 2.4 GHz interference avoidance and the observation from the field is that the deployment of such feature is very limited both in the network and the UE sides.

## 2.2 In-Device RX-Sharing

An LAA capable UE faces a strong form-factor limitation in terms of space when duplicating components to support unlicenced 5GHz bands operations.

A natural approach to overcome such limitations is described in [5] and [6], where the LAA device shares antennas and other HW components between LAA secondary cells and other technologies such as 5GHz Wi-Fi.

The impact of this design depends on the implementation, but it is generaly much more severe than the desense impact of legacy IDC interference problems, it could be:

- During Wi-Fi RX in any Wi-Fi channel, the UE cannot receive any control/data in any LAA Scells.

- During Wi-Fi TX in any Wi-Fi channel, the UE cannot receive any control/ data in any LAA Scells

Observation 3: An LAA capable UE could face form-factor limitations that force LAA RX sharing with other technologies, that may cause interruptions of LAA control/data recepetion at the UE level.

Today, the UE could use Wi-Fi to scan, connect to Access Points or to perform peer to peer operations. All of these operations could occur while LAA is configured or activated.

We can consdider LAA SCells as a capacity enhancement connection for LTE that is transparent to the end-user.

On the other hand, Wi-Fi activities in the UE can be split into two main categories:

- Low prioiritiy background activities scheduled by the UE

- High priority activities triggered by the user.

It is common sense that low prioirity Wi-Fi activies could be delayed; however, long WiFi activities triggered by the user could preempt LAA unlicensed shared ressources. If this happens, LTE data path will continue over licensed PCells and SCells, while Wi-Fi high prioritiy operations will be conducted in parallel over unlicensed band.

Observation 4:  Some Wi-Fi operations could be delayed while LAA is activated; however, non-periodic high priority WiFi activities will preempt LAA HW ressources.

## 2.3 Performance Impacts of LAA RX-Sharing

The obvious impacts of the uncoordinated LAA SCell RX interruption at the UE side are packet loss and retransmissions of packets over unlicensed spectrum; moreover, such interruptions could impact the licensed PCell traffic as well.

Consider the following use cases:

- Ongoing TCP session over LTE PCell.

- LAA SCell is enabled and some of the TCP packets are sent over LAA SCell in addition to LTE PCell.

- The user triggers high priority Wi-Fi operations over 5GHz.

- TCP congestion window of the LTE source, will interpret TCP packets loss of LAA SCell as a congestion, hence reduces the throughput, or worse, triggers slow start procedure for ongoing TCP session.

Observation 5: eNB-UE Uncoordinated LAA RX sharing operations could impact the LTE Licensed carrier in addition to unlicensed traffic.

Of course, the most efficient solution to overcome such a problem is to allow the UE to inform the eNB dynamically of imminent long Wi-Fi operation that could result in significant LAA RX interruption, so that the eNB could take appropriate actions such as suspending LAA transmission or deactivating LAA SCells.

Proposal 1: LAA should support a native mechanism for the UE to indicate important RX unavailability to the eNB when LAA SCell is configured or activated.

A similar issue was discussed in Rel-14 eLWA where the Wi-Fi path for LWA can be pre-empted by higher priority (e.g. user preference) WiFi activity and RAN2 has agreed that the UE should signal WiFi unavailability (via a “suspend” and “resume” indication). The proposal here is in-line with that agreement in principle.

As stated in the first paragraph, IDC framework was standardized to mitigate the in-device interference issues with mitigation solutions that do not solve the problem of Wi-Fi high priority activities in LAA RX-sharing devices.   Introducing new signalling in IDC framework to provide RX sharing indications will mandate LAA capable UEs and LAA eNB vendors to support, test and validate the full IDC framework in addition to LAA. (RRC signalling in addition to interference mitigation solutions for 2.4GHz and 5GHz that are not needed).

In 3GPP, we generally avoid the introduction of unecessary dependencies between different features. In-line with this principe, the most appropriate approach is to define a specific message within the LAA framework, allowing the UE to provide LAA eNB with RX unavailability and availability in case of RX sharing.

LAA is a carrier aggregation feature, hence a MAC-CE message is an excellent candidate to carry such indication from the UE to eNB.

Proposal 2:  An LAA MAC-CE could be introduced for the UE to indicate RX unavailability and availability back again to the eNB when LAA SCell is configured or activated.

# 3    Conclusion

This paper highlights some important aspects related to the UE RX sharing in LAA and their impact on the user experience. In order to overcome such issues, the following proposals are made:

Proposal 1: LAA should support a native mechanism for the UE to indicate important RX unavailability to the eNB when LAA SCell is configured or activated.

Proposal 2: An LAA MAC-CE could be introduced for the UE to indicate RX unavailability to the eNB when LAA SCell is configured or activated.

# 4    References

[1]		3GPP TR 36.888 v13.0: Feasibility Study on Licensed-Assisted Access to Unlicensed Spectrum.

[2]		R2-152030: Report of 3GPP TSG RAN WG2 meeting #89bis

[3]		R2-153060: Report of 3GPP TSG RAN WG2 meeting #90.

[4]		3GPP TR 36.813 v11.2: Study on signalling and procedure for interference avoidance for in-device coexistence

[5]		R2-168596: Concurrent RAT operation with RX sharing, Qualcomm Incorporated, Convida Wireless.

[6]		R2-164092: Concurrent RAT operation with NR, Qualcomm Incorporated, Convida Wireless