3GPP TSG-RAN WG2 Meeting #97	R2-1701805

Athens, Greece, 13-17 February 2017

Source: 	Huawei, HiSilicon, Qualcomm Incorporated, Sony, Telefonica, Deutsche Telekom

Title: 	Proposed Way Forward on Uplink-Based Mobility for UEs in RRC_CONNECTED active state

Agenda Item:	10.3.1.1.2

Document for:	Discussion and decision

# Introduction

This document proposes a way forward on uplink-based mobility for NR UEs in RRC_CONNECTED active state.

# Discussion

The following principles and assumptions are proposed.

Proposal 1: Concerning RRC driven UL-based active state mobility:

- For connected active state mobility, handover can be triggered by reports of DL measurements performed in the UE as the baseline, or by UL measurements in the network.

- DL-based mobility scheme should not prevent from introducing or improving UL-based mobility

Proposal 2: Uplink-based mobility in active state operates on the following principles:

- The source node configures the UE to transmit an UL signal.

- UL signal configuration e.g. signature/code allows the network to uniquely identify the UE within a geographical area

- The network measures the UE UL signal at various TRPs and determines which TRP(s) best serve the UE

- The network may configure the UE to perform, or not to perform, any DL measurement

  - DL RS could be switched on/off based on whether there are UEs that need them (pending RAN1 approval)

  - DL-based measurements can be configured by the network simultaneous with UL-based mobility, e.g. when the UE approaches candidate target nodes which do not measure/report the configured UL signal.

Proposal 3: The following items remain FFS for active state (some requiring RAN1 input):

- How the network configures potential target gNBs for uplink measurement

- Design (format/channel) of the uplink RS, choice of signatures, etc.

  - Related to how many signatures can be distinguished, and how often signature changes are needed

- Level of synchronisation required between TRPs that measure a UE UL signal

- If there would be differences in the mechanism for high frequency, e.g. relaxing some of the above requirements on UE to deal with the effects of beam sweeping

Proposal 4: Capture the following observations:

Observation 1: The main UE impact for UL-based mobility in RRC_CONNECTED is the transmission of an UL signal which can be detected by candidate target nodes.

Observation 2: For low frequencies, under appropriate conditions, the UL signal which can be used for UL scheduling (SRS) in RRC_CONNECTED could be detected and measured by candidate target nodes.

Observation 3: For higher frequencies, for UL-based mobility in RRC_CONNECTED, it is necessary to define an UL transmission procedure for an UL signal to be received by candidate target nodes.

Observation 4: On the network side, using UL measurements to trigger handovers in RRC_CONNECTED for mobility between cells controlled by the same gNB is possible without any specification impact.

Observation 5: RAN2 LTE procedures and messages for RRC_CONNECTED UEs seem to allow the support of UL-based mobility, at least in certain scenarios.